| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0405-v1 |
| --- | --- |

| Title: | Non-CE8: Minimum Allowed QP for Transform Skip Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tung Nguyen, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes to clip the Quantization Parameter (QP) in case of Transform Skip Mode (TSM) so that the minimum allowed QP for TSM is equal to four. That is necessary since the lossless condition is already met by a QP equal to four. When the QP is lower than four for TSM, the quantization stage introduces rounding errors, which is, from the authors point-of-view, an unintended behaviour. The results of the proposed bugfix are as follows. Note that the runtimes are not reliable.

Low QPs 2 – 7	(Luma BD-rates for CTC, Class F, TGM, encoding times for CTC, Class F, TGM)

All-Intra:			-0.18%	-1.51%	-1.60%	101%	100%	100%

Random-Access:	-0.00%	-0.05%	-1.72%	100%	104%	100%

# Introduction

For the Transform Skip Mode (TSM), the lossless condition holds when the Quantization Parameter (QP) is equal to four. When the QP becomes lower than four, the quantization stage introduces rounding errors. A significant example occurs for the Desktop sequence in the Random-Access configuration. The PSNR has its peak with 68.19 dB for QP 7, which results in QP 4 for the intra frame, and the PSNR drops down to 65.30 dB for QP 2. Detailed results for the case are as follows.

QP		bitrate		Y-PSNR
2		5419.94		65.30
7		4346.44		68.19
12		3439.70		60.67
17		2763.09		57.14

# Proposed Fix

This document proposes to limit the minimum allowed QP for TSM to be equal to four. With the proposed fix, the default QP and the QP for TSM differs when the QP ≤ 3. This proposal also includes source code with the proposed fix. For the example above, the results are as follows when applying the fix.

QP		bitrate		Y-PSNR
2		5211.55 	75.29
7 		4346.44 	68.19
12		3439.70 	60.67
17		2763.09 	57.14

# Experimental results

The results are reported using the Low QPs test conditions only since the effect only occurs for QP ≤ 4. Note that the runtimes are not reliable due to cluster usage.

## CE7/CE8 Low QPs (2, 7, 12, 17)

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.04% | 99% | 120% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 123% |
| Class B | -0.13% | 0.05% | 0.05% | 103% | 120% |
| Class C | -0.11% | 0.00% | 0.00% | 100% | 117% |
| Class E | -0.72% | 0.03% | 0.03% | 100% | 119% |
| Overall | -0.18% | 0.01% | 0.01% | 101% | 120% |
| Class D | -0.05% | 0.00% | 0.01% | 99% | 105% |
| Class F | -1.51% | -0.04% | -0.02% | 100% | 121% |
| Class TGM | -1.60% | -0.30% | -0.30% | 100% | 111% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.01% | 0.01% | 104% | 121% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.00% | 0.00% | 102% | 123% |
| Class F | -0.05% | -0.22% | -0.23% | 104% | 111% |
| Class TGM | -1.72% | -0.28% | -0.29% | 100% | 117% |

# Draft changes

### Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block,

- a variable bitDepth specifying the bit depth of the current colour component.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(8-954)

- Otherwise, if transform_skip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, the following applies:

qP = Max( 4, Qp′Y)		(8-954)

- Otherwise, if cIdx is equal to 1 and tu_joint_cbcr_residual[ xTbY ][ yTbY ] is equal to 1, the following applies:

qP = Qp′CbCr		(8-955)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(8-956)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(8-957)

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).