| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0407-v1 |
| --- | --- |

| Title: | Non-CE3: On adaptive rounding offset of MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, experimental results are reported which an adaptive rounding offset in the matrix based intra prediction (MIP) is removed. It is introduced that MIP process, needs to compare a value to compute a rounding offset. To simplify it, the offset is removed, it is tested using VTM-5.0. The simulation results are reported that 0.00%, 0.00% and x.xx% BD-rate for AI, RA and LB case, the encoding times are 100%, 101% and xxx% for AI, RA and LB, the decoding times are 100%, 100% and xxx% for AI, RA and LB.

Introduction

A technique to improve intra prediction called MIP was adopted in the 14th JVET meeting in Geneva [1]. In this MIP process, there are three steps that are averaging, matrix computation and up-sampling. The up-sampling process is described in [2], section 8.4.5.2.4 “MIP prediction upsampling process”.

In the up-sampling process, sample values after computation of matrix multiplication are given as an input, and those samples are linearly interpolated to current prediction size. At last stage of this process, there is a computation what is described in [2] as below.

sum = ( upHor − dX ) * predSamples[ xHor ][ yHor ] + dX * predSamples[ xHor + upHor ][ yHor ]	(8-84)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 − ( sum < 0  ?  1  :  0 ) ) / upHor	(8-85)

Where, the equation 8-85 checks a variable “sum” that a rounding offset is calculated. If the value is less than 0, the value is subtracted 1.

In our experiment, it had bee n conformed that it’s rare case what the value “sum” becomes negative number. Based on the results, it’s expected that an impact is small even if the adaptive rounding offset was removed. And, it can reduce complexity.

For the removal of adoptive rounding offset, the specification draft is changed as shown in sub clause 1.1.

In this contribution, the BD-rate impact is reported when VTM-5.0 was modified as the sub clause 1.1.

Text changes

      - When upHor is greater than 1, horizontal upsampling for all sparse positions ( xHor, yHor ) = ( m * upHor − 1, n * upVer − 1 ) with m = 0..predW − 1, n = 1..predH is applied with dX = 1..upHor − 1 as follows:

sum = ( upHor − dX ) * predSamples[ xHor ][ yHor ] + dX * predSamples[ xHor + upHor ][ yHor ]	(8-84)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 − ( sum < 0  ?  1  :  0 ) ) / upHor	(8-85)

      - Vertical upsampling for all sparse positions ( xVer, yVer ) = ( m, n * upVer − 1 ) with m = 0..nTbW − 1, n = 0..predH − 1 is applied with dY = 1..upVer − 1 as follows:

sum = ( upVer − dY ) * predSamples[ xVer ][ yVer ] + dY * predSamples[ xVer ][ yVer + upVer ]	(8-86)

predSamples[ xVer ][ yVer + dY ] = ( sum + upVer /2 − ( sum < 0  ?  1  :  0 ) ) / upVer	(8-87)

- Otherwise, the following ordered steps apply:

      - When upVer is greater than 1, vertical upsampling for all sparse positions ( xVer, yVer ) = ( m * upHor − 1, n * upVer − 1 ) with m = 1..predW, n = 0..predH − 1 is applied with dY = 1..upVer − 1 as follows:

sum = ( upVer − dY ) * predSamples[ xVer ][ yVer ] + dY * predSamples[ xVer ][ yVer + upVer ]	(8-88)

predSamples[ xVer ][ yVer + dY ] = ( sum + upVer /2 − ( sum < 0  ?  1  :  0 ) ) / upVer	(8-89)

      - Horizontal upsampling for all sparse positions ( xHor, yHor ) = ( m * upHor − 1, n ) with m = 0..predW − 1, n = 0..nTbH − 1 is applied with dX = 1..upHor − 1 as follows:.

sum = ( upHor − dX ) * predSamples[ xHor ][ yHor ] + dX * predSamples[ xHor + upHor ][ yHor ]	(8-90)

predSamples[ xHor + dX ][ yHor ] = ( sum + upHor /2 − ( sum < 0  ?  1  :  0 ) ) / upHor	(8-91)

Experiment

As introduced above, three tests carried out. JVET common test condition [3] is followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.03% | 0.02% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class F | -0.01% | 0.00% | 0.00% | 101% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | -0.05% | 0.01% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.05% | 0.04% | 101% | 100% |
| Class F | -0.04% | -0.12% | -0.07% | 100% | 100% |

Recommendation

In this contribution, an adaptive rounding offset in MIP was introduced. The test results were reported when the offset was removed. It’s confirmed that the BD-rate impact is negligible. Based on the experimental results, it is recommended to adopt into the next version of specification of VVC.

References

[1]	J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019.

[2]	B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, Geneva, Switzerland, March 2019

[3]	F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).