| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0409-v2 |
| --- | --- |

| Title: | Non-CE7: Unification of CCB count method between transform residual and transform skip residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to exclude coded_sub_block_flag from CCB count in transskip residual mode so that syntax used for CCB count is unified between trans and transskip residual modes, i.e. the coded_sub_block_flag is not included CCB count and CCB count starts from coeffloop. Simulation results reportedly show ClassF: 0.00% AI, 0.03% RA, -0.02% LB BD-Rate, and ClassTGM: 0.02% AI, 0.01% RA, 0.05%LB BD-Rate, over the VTM5.0.

Introduction

In VTM5, the constraint on the maximum number of context-coded bins (CCB) is applied for the transform residual and transform skip residual modes. According to VVC Draft, coefficient flags such as sig_coeff_flag , abs_level_gtx_flag[0], par_level_flag, and abs_level_gtx_flag[1] are included in CCB count in transform residual mode. On the other hand, the coded_sub_block_flag and coefficient flags such as sig_coeff_flag, coeff_sign_flag, abs_level_gtx_flag[0], par_level_flag, and abs_level_gtx_flag[j] (1≤j≤4) are included in CCB count in transform skip residual mode. There is a difference whether coded_sub_block_flag is included in CCB count. Table1 shows two syntaxes of the two residual coding modes in VTM5.0.

Table 1: two residual syntaxes in VTM5.0

| Trans residual coding | Transskip residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| last_sig_coeff_x_prefix |  |
| last_sig_coeff_y_prefix |  |
| last_sig_coeff_x_suffix |  |
| last_sig_coeff_y_suffix |  |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( coeff loop ) { | for( coeff loop ) { |
| sig_coeff_flag Count CCB | sig_coeff_flag Count CCB |
| abs_level_gtx_flag[0] | coeff_sign_flag |
| par_level_flag | abs_level_gtx_flag[0] |
| abs_level_gtx_flag[1] | par_level_flag |
| } | } |
| for( coeff loop ) { | for( j=1; j<5; j++ ) { |
| abs_remainder | for( coeff loop ) { |
| } | abs_level_gtx_flag[j] |
| for( coeff loop ) { | } |
| coeff_sign_flag | } |
| } | for( coeff loop ) { |
| } | abs_remainder |
| } | } |

Proposed method

It is proposed to exclude the coded_sub_block_flag from CCB count in transform skip residual mode as shown in Table 2. The maximum number of CCB in the transform skip mode is reduced by the number of subblock in a block. As a result, CCB count starts from the first coefficient loop in both of two residual modes.

Table 2: two residual syntaxes in this proposal

| Transform residual coding | Transformskip residual coding |
| --- | --- |
| residual_coding( ) { | residual_ts_coding( ) { |
| last_sig_coeff_x_prefix |  |
| last_sig_coeff_y_prefix |  |
| last_sig_coeff_x_suffix |  |
| last_sig_coeff_y_suffix |  |
| for( SubBlock loop ) { | for( SubBlock loop ) { |
| coded_sub_block_flag | coded_sub_block_flag |
| for( coeff loop ) { | for( coeff loop ) { |
| sig_coeff_flag Count CCB | sig_coeff_flag Count CCB |
| abs_level_gtx_flag[0] | coeff_sign_flag |
| par_level_flag | abs_level_gtx_flag[0] |
| abs_level_gtx_flag[1] | par_level_flag |
| } | } |
| for( coeff loop ) { | for( j=1; j<5; j++ ) { |
| abs_remainder | for( coeff loop ) { |
| } | abs_level_gtx_flag[j] |
| for( coeff loop ) { | } |
| coeff_sign_flag | } |
| } | for( coeff loop ) { |
| } | abs_remainder |
| } | } |

Simulation results

Table3 show the simulation results of this proposal. These simulations are conducted using VTM-5.0 under common test conditions of [1]. Table4 shows the simulation results of low QP case. The large loss of TGM in RA is due to the sequence, DeskTop. In this sequence, the PSNR is decreasing with the increase of bitrate, and the exsisting calculation method of BD-rate cannot correctly evaluate that tendency, which is a cause of this issue.

Table 3:  Simulation results of this proposal

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.04% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class B | 0.00% | 0.04% | -0.03% | 99% | 100% |
| Class C | 0.00% | -0.09% | 0.03% | 100% | 101% |
| Class E | 0.00% | 0.03% | 0.03% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.16% | -0.04% | 100% | 99% |
| Class F | 0.00% | 0.01% | -0.07% | 100% | 100% |
| TGM | 0.02% | 0.00% | 0.02% | 99% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.03% | 101% | 100% |
| Class A2 | 0.02% | -0.10% | 0.04% | 100% | 100% |
| Class B | 0.00% | 0.06% | -0.06% | 100% | 100% |
| Class C | -0.02% | -0.29% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | -0.01% | 100% | 100% |
| Class D | 0.04% | -0.03% | 0.05% | 101% | 99% |
| Class F | 0.03% | -0.04% | 0.10% | 100% | 100% |
| TGM | 0.01% | -0.01% | -0.03% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.43% | -0.05% | 100% | 99% |
| Class C | 0.02% | 0.44% | 0.20% | 100% | 100% |
| Class E | 0.06% | 0.20% | -0.32% | 100% | 99% |
| Overall | 0.02% | 0.37% | -0.03% | 100% | 99% |
| Class D | -0.01% | -0.01% | 0.71% | 100% | 100% |
| Class F | -0.02% | -0.21% | -0.12% | 100% | 100% |
| TGM | 0.05% | -0.11% | -0.01% | 100% | 100% |

Table 4: Simulation results of this proposal in low QP case

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0(100Frm4K_LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.01% | 101% | 99% |
| Class B | 0.01% | -0.01% | 0.02% | 100% | 99% |
| Class C | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.03% | 0.01% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.04% | -0.02% | 100% | 98% |
| Class F | 0.02% | -0.03% | -0.01% | 100% | 99% |
| TGM | 0.03% | 0.05% | 0.05% | 99% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0(100Frm4K_LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.03% | 100% | 99% |
| Class C | -0.01% | -0.02% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.04% | -0.01% | 100% | 103% |
| Class F | 0.09% | 0.01% | 0.03% | 99% | 99% |
| TGM | 1.49% | -0.23% | -0.21% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0(100Frm4K_LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.02% | 0.01% | -0.07% | 101% | 100% |
| Class E | 0.00% | 0.01% | -0.09% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.05% | 100% | 100% |
| Class D | 0.00% | -0.05% | 0.00% | 100% | 99% |
| Class F | 0.04% | 0.03% | 0.18% | 99% | 99% |
| TGM | 0.25% | 0.07% | 0.07% | 100% | 101% |

Conclusion

This proposal aims to unify syntaxes for CCB count in transform skip residual mode to transform residual mode to that of transform residual mode. There are no coding losses in all classes by this unification.We suggest to adopt this proposal into the test model.

Reference

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).