| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0412-v3 |
| --- | --- |

| Title: | CE3-related: Sampling point extension for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda, Yoichi Yagasaki Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 masaru.ikeda@sony.com Yoichi.Yagasaki@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes sampling point extension for CCLM. While keeping the worst-case as VTM5.0, the sampling point extension provides coding gains. The BD-Rate number for the modification for Y/U/V reportedly shows -0.11% / -0.49% / -0.61% at AI and -0.08% / -0.38% / -0.52% at RA over VTM-5.0.

Introduction

In the VTM5.0 new CCLM algorithm was adopted. It reduces complexity by limiting at most 4 neighbouring chroma samples and their corresponding down-sampled luma samples as figure 1. This limitation fits well for small size block like 4x4, since sampling ratio is relatively high/dense (4 samples out of 8) and the worst-case occurs at 4x4 block. However, this limitation may not be optimal for larger blocks, since the worst-case does not occur at larger blocks and sampling ratio is low/sparse (4 samples out of 32/64/128).

Figure 1: Illustration of the sampling points for the CCLM in VTM5.0

Proposal

Overview

Since larger blocks have more processing time than smaller ones, simple extension is proposed to increase neighbouring chroma samples and their corresponding down-sampled luma samples for larger blocks as figure 2.

Figure 2: Illustration of the proposed sampling points for larger blocks

Complexity Study	1

Number of operations of CCLM are listed based on number of points per block in the following table 1.

Table 1: Number of operations for CCLM based on number of points

| Points per block | # of Swap (A) | # of Add & Shift (B) | Total (A)+(B) |
| --- | --- | --- | --- |
| 4 | 4 | 12 | 16 |
| 6 | 11 | 12 | 23 |
| 8 | 16 | 12 | 28 |
| 10 | 23 | 12 | 35 |
| 12 | 30 | 12 | 42 |
| 16 | 40 | 12 | 52 |

The detail description for number of operations at (4 points case) and (6 points case) follows.

//4 Points Case

(A) 4 Swap

The arrays minGrpIdx and maxGrpIdx are derived as follows:

minGrpIdx[ 0 ] = 0	, minGrpIdx[ 1 ] = 2, maxGrpIdx[ 0 ] = 1, maxGrpIdx[ 1 ] = 3

When pSelDsY[ minGrpIdx[ 0 ] ] is greater than pSelDsY[ minGrpIdx[ 1 ] ], minGrpIdx[ 0 ] and minGrpIdx[ 1 ] are swapped as follows:

( minGrpIdx[ 0 ], minGrpIdx[ 1 ] ) = Swap( minGrpIdx[ 0 ], minGrpIdx[ 1 ] )	(1st swap)

When pSelDsY[ maxGrpIdx[ 0 ] ] is greater than pSelDsY[ maxGrpIdx[ 1 ] ], maxGrpIdx[ 0 ] and maxGrpIdx[ 1 ] are swapped as follows:

( maxGrpIdx[ 0 ], maxGrpIdx[ 1 ] ) = Swap( maxGrpIdx[ 0 ], maxGrpIdx[ 1 ] )	(2nd swap)

When pSelDsY[ minGrpIdx[ 0 ] ] is greater than pSelDsY[ maxGrpIdx[ 1 ] ], arrays minGrpIdx and maxGrpIdx are swapped as follows:

( minGrpIdx, maxGrpIdx ) = Swap( minGrpIdx, maxGrpIdx )			(3rd swap)

When pSelDsY[ minGrpIdx[ 1 ] ] is greater than pSelDsY[ maxGrpIdx[ 0 ] ], minGrpIdx[ 1 ] and maxGrpIdx[ 0 ] are swapped as follows:

( minGrpIdx[ 1 ], maxGrpIdx[ 0 ] ) = Swap( minGrpIdx[ 1 ], maxGrpIdx[ 0 ] )	(4th swap)

(B) 12 Add & Shift are performed to derive maxY, maxC, minY and minC

maxY = ( pSelDsY[ maxGrpIdx[ 0 ] ] + pSelDsY[ maxGrpIdx[ 1 ] ] + 1 )  >>  1 		3 operations

maxC = ( pSelC[ maxGrpIdx[ 0 ] ] + pSelC[ maxGrpIdx[ 1 ] ] + 1 )  >>  1		3 operations

minY = ( pSelDsY[ minGrpIdx[ 0 ] ] + pSelDsY[ minGrpIdx[ 1 ] ] + 1 )  >>  1		3 operations

minC = ( pSelC[ minGrpIdx[ 0 ] ] + pSelC[ minGrpIdx[ 1 ] ] + 1 )  >>  1			3 operations

//6 Points Case

(A) 11 Swap

if (selectLumaPix[Grp[0]] > selectLumaPix[Grp[1]]) std::swap(Grp[0], Grp[1]);

if (selectLumaPix[Grp[2]] > selectLumaPix[Grp[3]]) std::swap(Grp[2], Grp[3]);

if (selectLumaPix[Grp[1]] > selectLumaPix[Grp[2]]) std::swap(Grp[1], Grp[2]);

if (selectLumaPix[Grp[0]] > selectLumaPix[Grp[3]]) std::swap(Grp[0], Grp[3]);

if (selectLumaPix[Grp[0]] > selectLumaPix[Grp[1]]) std::swap(Grp[0], Grp[1]);

if (selectLumaPix[Grp[4]] > selectLumaPix[Grp[5]]) std::swap(Grp[4], Grp[5]);

if (selectLumaPix[Grp[1]] > selectLumaPix[Grp[4]]) std::swap(Grp[1], Grp[4]);

if (selectLumaPix[Grp[0]] > selectLumaPix[Grp[5]]) std::swap(Grp[0], Grp[5]);

if (selectLumaPix[Grp[2]] > selectLumaPix[Grp[3]]) std::swap(Grp[2], Grp[3]);

if (selectLumaPix[Grp[3]] > selectLumaPix[Grp[4]]) std::swap(Grp[3], Grp[4]);

if (selectLumaPix[Grp[2]] > selectLumaPix[Grp[5]]) std::swap(Grp[2], Grp[5]);

(B) 12 Add & Shift are performed regardless number of points per block.

Complexity Study	2

Total number of operations at 64x64 region is studied where 64x64 region are coded with one block shape. The results with square block are listed on Table 2. A comprehensive list including non-square block is on Table A-1 in appendix.

Table 2: Total number of operations at 64x64 region coded by block size

| Luma Block size | # of sampling points per block | # of operations (A) | # of blocks in 64x64 region (B) | Total # of operations (A) * (B) |
| --- | --- | --- | --- | --- |
| 4x4 | 4 | 16 | 256 | 4096 |
| 8x8 | 4 | 16 | 64 | 1024 |
|  | 8 | 28 | 64 | 1792 |
| 16x16 | 4 | 16 | 16 | 256 |
|  | 8 | 28 | 16 | 448 |
|  | 16 | 52 | 16 | 832 |
| 32x32 | 4 | 16 | 4 | 64 |
|  | 8 | 28 | 4 | 112 |
|  | 16 | 52 | 4 | 208 |
| 64x64 | 4 | 16 | 1 | 16 |
|  | 8 | 28 | 1 | 28 |
|  | 16 | 52 | 1 | 52 |

Table 2 shows

(1) The largest number of operations(worst-case) is 4096 at 4x4 block with 4 sampling points.

(2) Other cases, i.e., larger blocks (than 4x4) have smaller number of operations with 4/8/16 sampling points.

As conclusion, sample point extension at larger blocks (than 4x4) does NOT derive the worst-case.

Based on the table 2, we select several combinations of “number of points” and “block size” while keeping the worst-case.

Table 3: selected combination of “number of points” and “block size”

| Combination | Luma 4x4 | Luma 8x8 | Luma 16x16 | Luma 32x32 | Luma 64x64 |
| --- | --- | --- | --- | --- | --- |
| VTM5.0 | 4 points | 4 points | 4 points | 4 points | 4 points |
| A | 4 | 4 | 4 | 8 | 8 |
| B | 4 | 4 | 4 | 8 | 16 |
| C | 4 | 4 | 8 | 8 | 8 |
| D | 4 | 4 | 8 | 8 | 16 |
| E | 4 | 8 | 8 | 8 | 8 |

Additional tool from CE3-3.1

Figure 3: Illustration of the straight line with Luma and Chroma averaged values for β derivation	Left: VTM5.0  Right: CE3-3.1

In VTM5.0 the linear model parameters α and β are derived as Figure 2 Left side:

α=yB-yAxB-xA         β=yA-α*xA

The CE3-3.1 β is derived using both luma and chroma averaged values (xC, yC) among four samples instead of averaged minimum values (xA, yA) as shown in Figure 2 Right side.

In this contribution the linear model parameter α is not changed, but the parameter β is derived using both luma and chroma averaged values (xC, yC) of all sample points as the following equation:

β=yC-α*xC

Experimental results

Proposed extension was implemented on top of VTM-5.0 and tested based on CE3 test conditions [2].

Experimental results for several options

Experimental results for several options are presented in Table 4 & 5.

Table 4: BD-Rate of AI for several combinations of sampling point extension

| Combination tested | Luma 4x4 | Luma 8x8 | Luma 16x16 | Luma 32x32 | Luma 64x64 | Y | Cb | Cr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM5.0 | 4 | 4 | 4 | 4 | 4 | 0.00% | 0.00% | 0.00% |
| A | 4 | 4 | 4 | 8 | 8 | -0.05% | -0.25% | -0.25% |
| B | 4 | 4 | 4 | 8 | 16 | -0.05% | -0.23% | -0.20% |
| C | 4 | 4 | 8 | 8 | 8 | -0.06% | -0.34% | -0.32% |
| D | 4 | 4 | 8 | 8 | 16 | -0.05% | -0.33% | -0.29% |
| E | 4 | 8 | 8 | 8 | 8 | -0.06% | -0.42% | -0.42% |

Table 5: BD-Rate of AI for several combinations with sampling point extension & CE3-3.1 βderivation

| Combination tested | Luma 4x4 | Luma 8x8 | Luma 16x16 | Luma 32x32 | Luma 64x64 | Y | Cb | Cr |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM5.0 | 4 | 4 | 4 | 4 | 4 | 0.00% | 0.00% | 0.00% |
| A +β | 4 | 4 | 4 | 8 | 8 | -0.09% | -0.33% | -0.31% |
| B+β | 4 | 4 | 4 | 8 | 16 |  |  |  |
| C+β | 4 | 4 | 8 | 8 | 8 | -0.11% | -0.50% | -0.48% |
| D+β | 4 | 4 | 8 | 8 | 16 | -0.11% | -0.46% | -0.47% |
| E+β | 4 | 8 | 8 | 8 | 8 | -0.11% | -0.56% | -0.58% |

Experimental results for E+β

Results of E+βare presented in Table 6 for AI and Table 7 for RA.

Due to resource allocation, encoding time comparison is not available.

Table 6: E+β　BD-Rate of AI

Table 7: E+β　BD-Rate of RA

Conclusion

This proposal presents sampling point extension for CCLM. While keeping the worst-case as VTM5.0, the sampling point extension provides coding gains. We think coding gain per coding cost is reasonable. So, we recommend considering that the proposed extension is included into next version of test model.

Reference

[1] J. Huo, Y. Ma (Xidian University), S. Wan (Northwestern Polytechnical University), Y. Yu (OPPO), M. Wang, K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance), J. Li, S. Wang, W. Gao (Peking University), “CE3-1.5: CCLM derived from four neighbouring samples”, JVET-N0271, March 2019

[2] G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, JVET-N01023, March 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix

Table A-1: Total number of operations at 64x64 region coded by block size

| Luma Block size | # of sampling points per block | # of operations (A) | # of blocks in 64x64 region (B) | Total # of operations (A) * (B) |
| --- | --- | --- | --- | --- |
| 4x4 | 4 | 16 | 256 | 4096 |
| 4x8 | 4 | 16 | 128 | 2048 |
|  | 6 | 23 | 128 | 2944 |
| 4x16 | 4 | 16 | 64 | 1024 |
|  | 6 | 23 | 64 | 1472 |
|  | 10 | 35 | 64 | 2240 |
| 4x32 | 4 | 16 | 32 | 512 |
|  | 6 | 28 | 32 | 736 |
|  | 10 | 52 | 32 | 1120 |
| 4x64 | 4 | 16 | 16 | 256 |
|  | 6 | 28 | 16 | 368 |
|  | 10 | 52 | 16 | 560 |
| 8x8 | 4 | 16 | 64 | 1024 |
|  | 8 | 28 | 64 | 1792 |
| 8x16 | 4 | 16 | 32 | 512 |
|  | 8 | 28 | 32 | 896 |
|  | 12 | 42 | 32 | 1344 |
| 8x32 | 4 | 16 | 16 | 256 |
|  | 8 | 28 | 16 | 448 |
|  | 12 | 42 | 16 | 672 |
| 8x64 | 4 | 16 | 8 | 128 |
|  | 8 | 28 | 8 | 224 |
|  | 12 | 42 | 8 | 336 |
| 16x16 | 4 | 16 | 16 | 256 |
|  | 8 | 28 | 16 | 448 |
|  | 16 | 52 | 16 | 832 |
| 16x32 | 4 | 16 | 8 | 128 |
|  | 8 | 28 | 8 | 224 |
|  | 16 | 52 | 8 | 416 |
| 32x32 | 4 | 16 | 4 | 64 |
|  | 8 | 28 | 4 | 112 |
|  | 16 | 52 | 4 | 208 |
| 32x64 | 4 | 16 | 2 | 32 |
|  | 8 | 28 | 2 | 56 |
|  | 16 | 52 | 2 | 104 |
| 64x64 | 4 | 16 | 1 | 16 |
|  | 8 | 28 | 1 | 28 |
|  | 16 | 52 | 1 | 52 |