| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0413-v3 |
| --- | --- |

| Title: | Chroma deblocking filter adjustments for 4:4:4 and 4:2:2 format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Masaru Ikeda Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 yoshitaka.morigami@sony.com masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to adjust chroma deblocking filter to support YUV4:4:4, RGB4:4:4 and 4:2:2 format. There are basically two modifications for the adjustments in chroma long filter. One is to change the position of second line which is referred in the decision to align with luma. The other one is to change the parameter for large block decision to adjust to the chroma scaling. The proposed modifications were tested for JCT-VC RExt YUV4:4:4, YUV4:2:2 and RGB4:4:4 and it reportedly show 0.00% / -0.05% / -0.02%, 0.00% / -0.07% / -0.07% and 0.00% / 0.00% / 0.02% at AI, 0.04% / -0.10% / 0.06%, 0.00% / -0.05% / -0.04% and -0.05% / -0.14% / 0.06% at RA over VTM-5.0, respectively. For 4:4:4 TGM sequences, they reportedly show 0.00% / -0.05% / -0.05%, 0.03% / -0.06% / -0.06% and 0.06% / 0.03% / 0.03% at AI, RA and LDB, respectively.

Introduction

In last Geneva meeting, several modifications, software fixes and editorial changes for CCLM, affine and so on were proposed and adopted to support 4:4:4 and 4:2:2 format [1]. The long deblocking filter was adopted in Marrakesh meeting, and in the chroma deblocking filter, the decision is the same as HEVC style luma strong filter decision. [2] In the decision, the two lines are used for both on/off decision and filter type selection between long and weak filter. For 4:4:4 format, first and second line are used for the decision of 4-line segment while first and last line are used in luma case. And the threshold for large block decision is currently for 4:2:0 format. Those should be adjusted for 4:4:4 and 4:2:2 format.

Proposed modifications

It is proposed to use first and forth line for the chroma decision of both edges in case of 4:4:4 format so that the position of the lines is aligned with luma. For the threshold, the parameter is proposed to be scaled according to chroma sampling number. Thus, in case of 4:4:4 format, the parameter is scaled from 8 to 16 for both edges. In case of 4:2:2 format, those modifications are proposed to be applied to vertical edge (EDGE_VER). For the horizontal edge (EDGE_HOR), the same decision and parameter are used as for 4:2:0 format. The Figure 1 and 2 show the relationship between 4:2:0, 4:4:4 and 4:2:2 with blue highlighted which shows used line for the decision in each edge, respectively.

Figure 1: chroma sample at vertical edge in 4:2:0, 4:4:4 and 4:2:2 format

Figure 2: chroma sample at horizontal edge in 4:2:0, 4:4:4 and 4:2:2 format

Experimental results

Proposed method was implemented on top of VTM-5.0 and tested for JCT-VC RExt CTC test set [3] and 4:4:4 TGM sequences [4] under JVET CTC conditions [5]. For 4:4:4 TGM sequences, IBC, BDPCM and HashME are enabled. The results for RExt CTC test set and TGM sequences reportedly are obtained and presented as shown in Table 1 and Table 2, respectively, for AI, RA and LDB configurations. The encoding time is not reliable since the simulation was conducted on the different machines.

Table 1: BD-Rate results for YUV4:4:4 in JCT-VC RExt CTC test set

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | 0.00% | -0.06% | -0.15% |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.00% | -0.07% | -0.11% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 0.00% | -0.13% | 0.14% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | 0.00% | -0.05% | -0.04% |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.01% | 0.05% | 0.02% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 0.00% | -0.01% | 0.04% |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.00% | -0.06% | -0.07% |
| Overall | 0.00% | -0.05% | -0.02% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | -0.01% | -0.05% | 0.10% |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.07% | -0.01% | 0.15% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 0.02% | -0.17% | -0.08% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.01% | -0.19% | 0.02% |
| VenueVu_1920x1080_30_10bit_444.yuv | 0.00% | 0.21% | -0.07% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 0.17% | -0.45% | 0.29% |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.02% | -0.05% | 0.02% |
| Overall | 0.04% | -0.10% | 0.06% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_444_crop.yuv | 0.03% | 0.06% | 0.73% |
| Kimono1_1920x1080_24_10bit_444.yuv | 0.09% | -0.08% | 0.30% |
| EBULupoCandlelight_1920x1080_50_10bit_444.yuv | -0.13% | 0.52% | 0.25% |
| EBURainFruits_1920x1080_50_10bit_444.yuv | -0.06% | 0.42% | 0.03% |
| VenueVu_1920x1080_30_10bit_444.yuv | -0.02% | 0.44% | 0.29% |
| BirdsInCage_1920x1080_60_10bit_444.yuv | 0.30% | -1.44% | 0.96% |
| CrowdRun_1920x1080_50_10bit_444.yuv | 0.07% | 0.08% | 0.28% |
| Overall | 0.04% | 0.00% | 0.41% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: BD-Rate results for YUV4:2:2 in JCT-VC RExt CTC test set

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.00% | -0.03% | -0.01% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.00% | -0.09% | -0.11% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.00% | -0.04% | -0.04% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.00% | -0.03% | -0.02% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.00% | -0.03% | -0.05% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.00% | -0.16% | -0.16% |
| Seeking_1920x1080_50_10bit_422.yuv | 0.00% | -0.09% | -0.08% |
| Overall | 0.00% | -0.07% | -0.07% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | -0.02% | 0.02% | -0.01% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.07% | -0.09% | -0.07% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.00% | -0.07% | -0.05% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.01% | -0.03% | -0.03% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 0.00% | 0.03% | -0.06% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | -0.05% | -0.18% | -0.07% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.02% | 0.00% | 0.03% |
| Overall | 0.00% | -0.05% | -0.04% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_10bit_422_crop.yuv | 0.07% | 0.20% | 0.42% |
| Kimono1_1920x1080_24_10bit_422.yuv | 0.06% | 0.27% | 0.19% |
| EBUHorse_1920x1080_50_10bit_422.yuv | 0.11% | -0.18% | -0.09% |
| EBUGraphics_1920x1080_50_10bit_422.yuv | 0.02% | 0.04% | 0.06% |
| EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | -0.01% | 0.05% | 0.07% |
| EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 0.03% | -0.47% | 0.23% |
| Seeking_1920x1080_50_10bit_422.yuv | -0.01% | -0.15% | -0.16% |
| Overall | 0.04% | -0.04% | 0.10% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3: BD-Rate results for RGB4:4:4 in JCT-VC RExt CTC test set

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_12bit_444.rgb | 0.00% | -0.01% | 0.04% |
| Kimono1_1920x1080_24_10bit_444.rgb | 0.00% | -0.01% | 0.02% |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | 0.00% | 0.00% | 0.03% |
| EBURainFruits_1920x1080_50_10bit_444.rgb | 0.00% | -0.01% | 0.01% |
| VenueVu_1920x1080_30_8bit_444.rgb | 0.01% | 0.09% | 0.08% |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | 0.00% | 0.00% | 0.01% |
| OldTownCross_1920x1080_50_10bit_444.rgb | 0.00% | 0.00% | 0.00% |
| ParkScene_1920x1080_24_10bit_444.rgb | 0.00% | -0.01% | 0.00% |
| Overall | 0.00% | 0.00% | 0.02% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_12bit_444.rgb | -0.03% | -0.20% | 0.08% |
| Kimono1_1920x1080_24_10bit_444.rgb | -0.02% | -0.10% | 0.45% |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | -0.03% | -0.16% | -0.13% |
| EBURainFruits_1920x1080_50_10bit_444.rgb | 0.05% | 0.11% | 0.11% |
| VenueVu_1920x1080_30_8bit_444.rgb | 0.01% | 0.35% | 0.36% |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -0.05% | -0.73% | -0.30% |
| OldTownCross_1920x1080_50_10bit_444.rgb | -0.30% | -0.26% | -0.30% |
| ParkScene_1920x1080_24_10bit_444.rgb | -0.01% | -0.16% | 0.22% |
| Overall | -0.05% | -0.14% | 0.06% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 99% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic_2560x1600_30_12bit_444.rgb | 0.00% | -0.25% | 0.30% |
| Kimono1_1920x1080_24_10bit_444.rgb | #VALUE! | #VALUE! | #VALUE! |
| EBULupoCandlelight_1920x1080_50_10bit_444.rgb | 0.08% | #VALUE! | -0.76% |
| EBURainFruits_1920x1080_50_10bit_444.rgb | 0.02% | #VALUE! | 0.02% |
| VenueVu_1920x1080_30_8bit_444.rgb | 0.06% | 0.44% | 0.46% |
| DucksAndLegs_1920x1080_30_10bit_444.rgb | -0.04% | -1.18% | -0.69% |
| OldTownCross_1920x1080_50_10bit_444.rgb | #VALUE! | #VALUE! | #VALUE! |
| ParkScene_1920x1080_24_10bit_444.rgb | #VALUE! | #VALUE! | #VALUE! |
| Overall | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |

Table 4: BD-Rate results for 4:4:4 TGM sequences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.03% | -0.03% | 84% | 99% |
| TGM 720p | 0.00% | -0.05% | -0.07% | 81% | 100% |
| Animation | 0.00% | -0.06% | -0.04% | 79% | 99% |
| Mixed content | 0.00% | -0.05% | -0.04% | 79% | 100% |
| Camera-Captured content | 0.00% | -0.05% | -0.06% | 84% | 98% |
| Overall | 0.00% | -0.05% | -0.05% | 81% | 99% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.09% | -0.10% | 116% | 100% |
| TGM 720p | 0.05% | -0.12% | -0.15% | 121% | 99% |
| Animation | 0.04% | -0.06% | -0.05% | 107% | 100% |
| Mixed content | 0.13% | 0.02% | 0.07% | 123% | 99% |
| Camera-Captured content | -0.01% | 0.01% | -0.01% | 124% | 99% |
| Overall | 0.03% | -0.06% | -0.06% | 118% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.10% | 0.03% | 0.01% | 98% | 98% |
| TGM 720p | 0.08% | -0.02% | 0.04% | 99% | 99% |
| Animation | 0.00% | 0.00% | -0.01% | 90% | 100% |
| Mixed content | 0.03% | 0.00% | -0.02% | 98% | 97% |
| Camera-Captured content | 0.05% | 0.22% | 0.19% | 112% | 99% |
| Overall | 0.06% | 0.03% | 0.03% | 99% | 99% |

Conclusion

This proposal presents a modification to adjust the chroma deblocking filter to 4:4:4 and 4:2:2 format. It gives us design consistency with similar performance and negligible complexity. We recommend considering that the modification is included into next version of test model.

WD text

The modifications are highlighted with light blue in JVET-O0413_WDtext-v2.docx.

Reference

[1] A. Filippov, V. Rufitskiy, T. Solovyev, R. Chernyak, J. Chen (Huawei), X. Zhao, X. Li, X. Xu, S. Liu (Tencent), H. Wang, Y.-H. Chao, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), “Support of 4:4:4 and 4:2:2 chroma formats in VVC”, JVET-N0671, March 2019

[2] M. Ikeda, T. Suzuki (Sony), D. Rusanovskyy, M. Karczewicz (Qualcomm), W. Zhu, K. Misra, P. Cowan, A. Segall (Sharp Labs of America), K. Andersson, J. Enhorn, Z. Zhang, R. Sjöberg (Ericsson), “CE11.1.6, CE11.1.7 and CE11.1.8: Joint proposals for long deblocking from Sony, Qualcomm, Sharp, Ericsson”, JVET-M0471, January 2019

[3] C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006-N1028, January 2014

[4] X. Xu, Y.-H. Chao, Y.-C. Sun. J., Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, March 2019

[5] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).