| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0414-r1 |
| --- | --- |

| Title: | Non-CE4 : Symmetric-MVD control considering the reference picture type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | [LG Electronics Inc.] |  |  |

_____________________________

- Abstract

In VTM 5.0, the symmetric MVD (SMVD) scheme allows to derive MVD of the reference list 1 (L1_MVD) from MVD of the reference list 0 (L0_MVD) without signaling of the MVD when the sym_mvd_flag is enabled. The symmetric MVD scheme is motivated that L1_MVD is precisely derived from L0_MVD when both reference pictures are similar and the movement of the object is constant in certain direction. Hence, it is important to define more accurate conditions for applying SMVD. Therefore, it is proposed to consider the reference picture type to determine whether sym_mvd_flag syntax is signaled or not. The experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC.

Introduction

The reference picture type is determined as “short-term reference picture (STRP)” or “long-term reference picture (LTRP)”. Both STRP and LTRP can be used as a reference picture for current block. Figure 1 depicts examples on usage of the mixed STRP and LTRP in SMVD scheme. Using the mirrored L0_MVD as L1_MVD can be inaccurate when reference picture type of the L0 and L1 are different, because the distance ratio (distance between reference picture 0 and current picture and distance between reference picture 1 and current picture) is larger, correlation of the motion vector on each direction can be lower.

Figure 1. Examples on usage of the mixed STRP and LTRP

In current working draft, sym_mvd_flag parsing process is as follows, i.e., availability of the reference pictures are checked and if the conditions are satisfied, sym_mvd_flag is parsed. If sym_mvd_flag is true, MVD of the L1 (MvdL1) is derived as mirrored MvdL0.

| if( sps_smvd_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| --- | --- |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = −MvdL0[ x0 ][ y0 ][ 0 ] |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = −MvdL0[ x0 ][ y0 ][ 1 ] |  |
| } else |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |

Also, in current scheme, RefIdxSymLX with X = 0, 1 is derived as follows. RefIdxSymL0 is the closest reference picture with POC is smaller than POC of the current picture and RefIdxSymL1 is the closest reference picture with POC is larger than the POC of the current picture as following decoding process.

8.3.5 Decoding process for symmetric motion vector difference reference indices Output of this process are RefIdxSymL0 and RefIdxSymL1 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit. The variable RefIdxSymLX with X being 0 and 1 is derived as follows: - The variable currPic specifies the current picture. - RefIdxSymL0 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1. - RefIdxSymL1 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1. - When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies: - For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

Proposed method

In this document, two schemes are proposed considering the reference picture type to enhance the accuracy of the MVD when the SMVD scheme is used. Comparisons among the current SMVD scheme and two proposed schemes are briefly shown in Table 1. In here, MVD means the MVD of the List 0.

Table 1. Current SMVD scheme and proposed schemes

|  |  | SMVD |  | Proposed scheme I |  | Proposed scheme II |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L0 | L1 | L0 MVD | L1 MVD | L0 MVD | L1 MVD | L0 MVD | L1 MVD |
| short | short | MVD | (-1) MVD | MVD | (-1) MVD | MVD | (-1) MVD |
| long | long | MVD | (-1) MVD | MVD | (-1) MVD |  |  |
| short | long | MVD | (-1) MVD |  |  |  |  |
| long | short | MVD | (-1) MVD |  |  |  |  |

Proposed scheme I

In the proposed scheme I, SMVD process is restricted when the current picture has mixed reference picture type. To apply the proposed scheme, specification text is modified as follows.

8.3.5 Decoding process for symmetric motion vector difference reference indices Output of this process are RefIdxSymL0 and RefIdxSymL1 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit. The variable RefIdxSymLX with X being 0 and 1 is derived as follows: - The variable currPic specifies the current picture. - RefIdxSymL0 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1. - RefIdxSymL1 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1. - When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies: - For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1. - When RefIdxSymL0 is not equal to −1 and RefIdxSymL1 is not equal to −1, the following applies: - If RefPicList[ 0 ][ RefIdxSymL0 ] is a long-term reference picture and RefPicList[ 1 ][ RefIdxSymL1 ] is a short-term reference picture, RefIdxSymL0 and RefIdxSymL1 are set equal to -1. - Otherwise, if RefPicList[ 0 ][ RefIdxSymL0 ] is a short-term reference picture and RefPicList[ 1 ][ RefIdxSymL1 ] is a long-term reference picture, RefIdxSymL0 and RefIdxSymL1 are set equal to -1.

Proposed scheme II

In the proposed scheme II, SMVD process is more restricted where the SMVD is not allowed when the current picture refers to LTRP. To apply the proposed scheme, specification text is modified as follows. For proposed scheme II, long-term reference pictures are excluded in the reference picture checking process, therefore, other reference picture (not long-term reference picture) can be considered for SMVD.

8.3.5 Decoding process for symmetric motion vector difference reference indices Output of this process are RefIdxSymL0 and RefIdxSymL1 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit. The variable RefIdxSymLX with X being 0 and 1 is derived as follows: - The variable currPic specifies the current picture. - RefIdxSymL0 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1, - RefPicList[ 0 ][ i ] is a short-term-reference picture. - RefIdxSymL1 is set equal to −1. - For each index i with i = 0..NumRefIdxActive[ 1 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1, - RefPicList[ 1 ][ i ] is a short-term-reference picture. - When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies: - For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL0 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0, - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1, - RefPicList[ 0 ][ i ] is a short-term-reference picture. - For each index i with i = 0..NumRefIdxActive[ 1 ] − 1, the following applies: - When all of the following conditions are true, RefIdxSymL1 is set to i: - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0, - DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1, - RefPicList[ 1 ][ i ] is a short-term-reference picture.

Experimental results

The proposed method has been implemented on top of the VTM5.0 and tested using JVET SDR common test condition [1]. The experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC.

Proposed method II:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 103% |

Conclusion

In this contribution, symmetric-MVD control method considering reference picture type are proposed. The experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC. It is recommended to consider this method in the next VTM.

Reference

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).