| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0415 |
| --- | --- |

| Title: | Non-CE4 : Fix of POC distance condition for MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | [LG Electronics Inc.] |  |  |

_____________________________

- Abstract

In this contribution, two simplified MVD derivation methods for MMVD are introduced by restricting the usage of the POC difference. In method 1, using the POC difference is restricted when a current block refers any long-term reference picture. In method 2, using the POC difference is always restricted. For method 1, the experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC. For method 2, the experimental results reportedly show 0.xx% BD-rate changes with xx% and xx% encoding and decoding run-time compared to VTM5.0 as anchor in RA configurations.

Introduction

In the MMVD scheme, L1 MVD is derived with the scaled or mirrored L0 MVD when POC distance of the L0 is larger or equal to POC distance of the L1. Otherwise, L0 MVD is derived with the scaled or mirrored L1 MVD.

Figure 1 shows the MVD derivation process considering the POC difference and reference picture type in MMVD scheme. In here, currPocDiffLX means the difference between POC of the current picture and POC of the LX reference picture. The currPocDiffL0 and currPocDiffL1 are compared each other and also type of the reference picture is checked as “refPicList0 != LTRP” or “refPicList1 != LTRP”. Considering the conditions, MmvdOffset (which is derived using the mmvd_cand_flag, mmvd_distance_idx and mmvd_direction_idx) is assigned to mMvdLX as same value, mirrored value or scaled value.

Figure 1. MVD derivation process of the MMVD

In current working draft, MVD derivation process for MMVD is as follows.

- 8.5.2.7 Derivation process for merge motion vector difference Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - reference indices refIdxL0 and refIdxL1, - prediction list utilization flags predFlagL0 and predFlagL1. Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1. The variable currPic specifies the current picture. The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows: - If both predFlagL0 and predFlagL1 are equal to 1, the following applies: currPocDiffL0 = DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) (8-336) currPocDiffL1 = DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) (8-337) - If currPocDiffL0 is equal to currPocDiffL1, the following applies: mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-338) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-339) mMvdL1[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-340) mMvdL1[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-341) - Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies: td = Clip3( −128, 127, currPocDiffL0 ) (8-342) tb = Clip3( −128, 127, currPocDiffL1 ) (8-343) tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-344) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-345) mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-346) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-347) - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies: mMvdL1[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 0 ] + (8-348) 128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) ) >> 8 ) mMvdL1[ 1 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 1 ] + (8-349) 128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) ) >> 8 ) - Otherwise, the following applies: mMvdL1[ 0 ] = Sign( currPocDiffL0 ) = = Sign( currPocDiffL1 ) ? mMvdL0[ 0 ] : −mMvdL0[ 0 ] (8-350) mMvdL1[ 1 ] = Sign( currPocDiffL0 ) = = Sign( currPocDiffL1) ? mMvdL0[ 1 ] : −mMvdL0[ 1 ] (8-351) - Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies: td = Clip3( −128, 127, currPocDiffL1 ) (8-352) tb = Clip3( −128, 127, currPocDiffL0 ) (8-353) tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-354) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-355) mMvdL1[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-356) mMvdL1[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-357) - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies: mMvdL0[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL1[ 0 ] + (8-358) 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) ) >> 8 ) mMvdL0[ 1 ] = Clip3( −215, 215 − 1, , (distScaleFactor * mMvdL1[ 1 ] + (8-359) 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) ) >> 8 ) ) - Otherwise, the following applies: mMvdL10[ 0 ] = Sign( currPocDiffL0 ) = = Sign( currPocDiffL1 ) ? mMvdL1[ 0 ] : −mMvdL1[ 0 ] (8-360) mMvdL0[ 1 ] = Sign( currPocDiffL0 ) = = Sign( currPocDiffL1) ? mMvdL1[ 1 ] : −mMvdL1[ 1 ] (8-361) - Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1: mMvdLX[ 0 ] = ( predFlagLX = = 1 ) ? MmvdOffset[ xCb ][ yCb ][ 0 ] : 0 (8-362) mMvdLX[ 1 ] = ( predFlagLX = = 1 ) ? MmvdOffset[ xCb ][ yCb ][ 1 ] : 0 (8-363)

Proposed method

Applying the mirroring scheme considering the POC distance is not necessary when the current picture refers one or more long-term reference picture (LTRP) because mirroring the MVD obtained from the reference picture with very-long distance for the other MVD is not effective.

In this document, two schemes are proposed. In method I, comparing process of the POC distance for LTRP is removed. And in method II, comparing process of the POC distance is removed in all cases. The proposed schemes are briefly shown in Table 1. Here, offset means MmvdOffset.

Table 1. Comparison between the current MMVD and the proposed schemes

|  |  |  | MMVD |  | Proposed MMVD I |  | Proposed MMVD II |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L0 | L1 | POC | L0 Offset | L1 Offset | L0 Offset | L1 Offset | L0 Offset | L1 Offset |
| short | short | Same | Offset | Offset | Offset | Offset | Offset | Offset |
|  |  | Diff (L0>=L1) | Offset | Scaled | Offset | Scaled | Offset | Scaled |
|  |  | Diff (L0 < L1) | Scaled | Offset | Scaled | Offset | Offset | Scaled |
| long | long | Same | Offset | Offset | Offset | Offset | Offset | Offset |
|  |  | Diff (L0>=L1) | Offset | (-1) Offset | Offset | (-1) Offset | Offset | (-1) Offset |
|  |  | Diff (L0<L1) | (-1) Offset | Offset | Offset | (-1) Offset | Offset | (-1) Offset |
| short | long | Same | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | Diff (L0>=L1) | Offset | (-1) Offset | Offset | (-1) Offset | Offset | (-1) Offset |
|  |  | Diff (L0<L1) | (-1) Offset | Offset | Offset | (-1) Offset | Offset | (-1) Offset |
| long | short | Same | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | Diff (L0>=L1) | Offset | (-1) Offset | Offset | (-1) Offset | Offset | (-1) Offset |
|  |  | Diff (L0<L1) | (-1) Offset | Offset | Offset | (-1) Offset | Offset | (-1) Offset |

Proposed scheme I

In the proposed scheme I, MMVD process is restricted when the current block refers one or more long-term reference pictures (LTRP). The comparison condition of the POC difference when the reference picture type is LTRP is removed and anchor MVD used for mirroring process is fixed as L0 MVD.

Figure 2. The proposed scheme I

To apply the proposed scheme, the working draft can be changed as follows.

- 8.5.2.7 Derivation process for merge motion vector difference Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - reference indices refIdxL0 and refIdxL1, - prediction list utilization flags predFlagL0 and predFlagL1. Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1. The variable currPic specifies the current picture. The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows: - If both predFlagL0 and predFlagL1 are equal to 1, the following applies: currPocDiffL0 = DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) (8-336) currPocDiffL1 = DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) (8-337) - If currPocDiffL0 is equal to currPocDiffL1, the following applies: mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-338) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-339) mMvdL1[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-340) mMvdL1[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-341) - If RefPicList[ 0 ][ refIdxL0 ] is a long-term reference picture or RefPicList[ 1 ][ refIdxL1 ] is a long-term reference picture, the following applies mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-342) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-343) mMvdL1[ 0 ] = −mMvdL0[ 0 ] (8-344) mMvdL1[ 1 ] = −mMvdL0[ 1 ] (8-345) - Otherwise, the following applies : td = Clip3( −128, 127, currPocDiffL0 ) (8-346) tb = Clip3( −128, 127, currPocDiffL1 ) (8-347) tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-348) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-349) - if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies: mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-350) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-351) mMvdL1[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 0 ] + (8-352) 128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) ) >> 8 ) mMvdL1[ 1 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 1 ] + (8-353) 128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) ) >> 8 ) - Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies: mMvdL1[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-354) mMvdL1[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-355) mMvdL0[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL1[ 0 ] + (8-356) 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) ) >> 8 ) mMvdL0[ 1 ] = Clip3( −215, 215 − 1, , (distScaleFactor * mMvdL1[ 1 ] + (8-357) 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) ) >> 8 ) ) - Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1: mMvdLX[ 0 ] = ( predFlagLX = = 1 ) ? MmvdOffset[ xCb ][ yCb ][ 0 ] : 0 (8-358) mMvdLX[ 1 ] = ( predFlagLX = = 1 ) ? MmvdOffset[ xCb ][ yCb ][ 1 ] : 0 (8-359)

Proposed scheme II

In the proposed scheme II, MVD derivation process is restricted in all cases. The comparing condition of the POC difference in all cases is removed and anchor MVD used for mirroring or scaling process is fixed as L0 MVD.

Figure 3. The proposed scheme II

To apply the proposed scheme, the working draft can be changed as follows.

- 8.5.2.7 Derivation process for merge motion vector difference Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - reference indices refIdxL0 and refIdxL1, - prediction list utilization flags predFlagL0 and predFlagL1. Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1. The variable currPic specifies the current picture. The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows: - If both predFlagL0 and predFlagL1 are equal to 1, the following applies: currPocDiffL0 = DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) (8-336) currPocDiffL1 = DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) (8-337) - If currPocDiffL0 is equal to currPocDiffL1, the following applies: mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-338) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-339) mMvdL1[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-340) mMvdL1[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-341) - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies: mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-342) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-343) mMvdL1[ 0 ] = −mMvdL0[ 0 ] (8-344) mMvdL1[ 1 ] = −mMvdL0[ 1 ] (8-345) - Otherwise, the following applies: td = Clip3( −128, 127, currPocDiffL0 ) (8-346) tb = Clip3( −128, 127, currPocDiffL1 ) (8-347) tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-348) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-349) mMvdL0[ 0 ] = MmvdOffset[ xCb ][ yCb ][ 0 ] (8-350) mMvdL0[ 1 ] = MmvdOffset[ xCb ][ yCb ][ 1 ] (8-351) mMvdL1[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 0 ] + (8-352) 128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) ) >> 8 ) mMvdL1[ 1 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 1 ] + (8-353) 128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) ) >> 8 ) - Otherwise ( predFlagL0 or predFlagL1 are equal to 1 ), the following applies for X being 0 and 1: mMvdLX[ 0 ] = ( predFlagLX = = 1 ) ? MmvdOffset[ xCb ][ yCb ][ 0 ] : 0 (8-354) mMvdLX[ 1 ] = ( predFlagLX = = 1 ) ? MmvdOffset[ xCb ][ yCb ][ 1 ] : 0 (8-355)

Experimental results

The proposed method has been implemented on top of the VTM5.0 and tested using JVET SDR common test condition [1]. For method I, the experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC. For method II, the experimental results reportedly show 0.xx% BD-rate changes with xx% and xx% encoding and decoding run-time compared to VTM5.0 as anchor in RA configurations.

Proposed scheme II :

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.01% | 0.01% | 0.01% | 100% | 102% |
| Class B | 0.03% | 0.11% | 0.02% | 100% | 100% |
| Class C | 0.00% | -0.03% | -0.02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | -0.04% | 0.08% | 99% | 98% |
| Class F | 0.01% | 0.02% | -0.01% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.21% | -0.04% | 101% | 102% |
| Class C | 0.03% | 0.04% | -0.17% | 100% | 99% |
| Class E | 0.19% | 0.01% | -0.54% | 99% | 99% |
| Overall | 0.06% | 0.10% | -0.21% | 100% | 101% |
| Class D | 0.04% | -0.14% | 0.74% | 100% | 96% |
| Class F | -0.07% | 0.13% | -0.62% | 99% | 100% |

Conclusion

In this contribution, comparing process of the POC distance for MVD derivation in MMVD scheme is restricted 1) when a current block refers one or more long-term reference picture and 2) in all cases. For method I, the experimental results reportedly show no changes in coding performance and run-time compared to VTM5.0 as anchor in CTC. For method II, the experimental results reportedly show 0.xx% BD-rate changes with xx% and xx% encoding and decoding run-time compared to VTM5.0 as anchor in RA configurations. It is recommended to consider this method in the next VTM.

Reference

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).