| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET- O0419-v2 |
| --- | --- |

| Title: | AHG6: Wrap-around spatial merge candidate for inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhun Lee, Jongseok Lee, Juntaek Park, Donggyu Sim, Seung-Jun Oh (KWU) 20 Gwangun-ro, Nowon-gu, Seoul, Republic of Korea (01897) | Tel: Email: | +82-10-8696-5470 {minhun, suk2080, juntaek, dgsim, sjoh} @kw.ac.kr, |
| Author(s) or Contact(s): | Woong Lim, Gun Bang, Hui Young Kim (ETRI) 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-1836 {woong.lim, gbang, hykim5} @etri.re.kr |
| Source: | KWU (Kwangwoon University), ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

In this contribution, a wrap-around spatial merge candidate is proposed. The proposed method adds a wrap-around merge candidate with the above left or above right merge candidate at a picture boundary for coding efficiency. Simulation results show that the wrap-around spatial merge candidate yields 0.xx%, 0.xx%, and 0.00% BD-rate gain in end-to-end WS-PSNR with no complexity burden for PERP with horizontal geometry padding of reference pictures.

Introduction

In VTM-5.0, a merge candidate list is constructed for inter skip/merge modes using the motion vectors of the neighboring blocks to decode the current PU's motion vectors. When the position of the neighboring block is out of the left or right picture boundaries, the motion vector candidate is excluded from the candidate list. Therefore, in the case of the CU adjacent to the picture boundary, the coding efficiency is reduced as compared with the other blocks due to the lack of merge candidates. To alleviate this problem, this contribution employs a wrap-around spatial merge candidate which is a motion vector of the block of wrap-around offset position into the merge list for the blocks adjacent to the picture boundary.

The boundary of the picture, in general, belongs to a background, and the motion of the background is likely to be denoted a global motion. If so, the motions of the left and right boundary regions in a picture are similar. In addition, the picture reformed in ERP or PERP formats has higher motion similarity than in non 360 pictures because the left and right picture boundaries are connected to each other. In VTM-5.0, the horizontal wrap-around motion compensation technology has already been adopted[1].

Proposed Method

In VTM-5.0, the process of constructing the spatial merge candidate list of the current CU proceeds in the order of A0, B0, B1, A1, and B2 in Fig 1.

CU

Figure 1. Position of spatial merge candidate in WD5[2]

If the CU is at the left picture boundary, MVs at A0, A1, and B2 can not be used as merge candidates. On the contrary, MVs at B1 can not be used as merge candidates if the CU is adjacent to the right picture boundary. In the proposed method, the motion vector of the wrap-around position by adding sps_ref_wraparound_offset [3]or subtracting it from the position of the out-of-boundary candidate in the x direction is added in the merge list, as shown Fig. 2. Therefore, motion vector of the wrap-around position with sps_ref_wraparound_offset for B1 or B2 locations can be used at the picture boundary. Note that since the PERP format uses the horizontal geometry padding, the wrap-around offset (ERP width) [1] which is not the picture width should be used to find the corresponding wrap-around merge candidate.

Because motion vectors of wrapped-around A0 and A1 positions have not decoded, they are not used for the proposed wrap-around merge candidates.

Figure 2. Example of wrap-around merge candidate at the picture boundary.

The followings are additional change relative to decoding processing of VVC in working draft.

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
else

if( sps_ref_warparound_enabled_flag )

if( availableFlag(B1 - sps_ref_warparound_offset) )

mergeCandList[i++] = B1

if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-281)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
else

if( sps_ref_warparound_enabled_flag )

if( availableFlag(B2 + sps_ref_warparound_offset) )

mergeCandList[i++] = B2

if( availableFlagCol )
	mergeCandList[ i++ ] = Col

Simulation Results

The proposed method is implemented on VTM-5.0 with 360Lib-9.1 software and tested under common test conditions [4]. Table 1 shows RD performance and running time of the test model with the proposed wrap-around merge candidate over VTM-5.0 with 360Lib-9.1.

Table 1. RD performance of wrap-around merge candidate on the top of VTM-5.0

| Format: PERP |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference: VTM-5.0 |  | Over VTM-5.0 |  |  |  |  |
| Tested: Proposed |  | Y | U | V | EncT | DecT |
| S1 | SkateboardInLot |  |  |  |  |  |
|  | ChairLift | -0.13% | 0.01% | -0.18% | 99% | 100% |
|  | KiteFlite |  |  |  |  |  |
|  | Harbor | -0.01% | -0.04% | -0.02% | 100% | 99% |
|  | Trolley | -0.03% | -0.07% | 0.03% | 101% | 99% |
|  | GasLamp | -0.02% | -0.02% | -0.05% | 100% | 100% |
| S2 | Balboa |  |  |  |  |  |
|  | Broadway |  |  |  |  |  |
|  | Landing2 |  |  |  |  |  |
|  | BranCastle2 |  |  |  |  |  |
| Overall |  |  |  |  |  |  |

Patent rights declaration(s)

ETRI and Kwangwoon university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- Philippe Hanhart, Yuwen He, Yan Ye “CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3)”, JVET-L0231, October 2018, Macau, CN.

- Benjamin Bross, Jianle Chen, Shan Liu “Versatile Video Coding (Draft 5)”, JVET-N1001, Mar 2019, Geneva, CH.

- Byeongdoo Choi, Weiwei Feng, Shan Liu “AHG8: On wrap around motion compensation”, JVET-M0225, Jan 2019, Marrakech, MA.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin, “ JVET common test conditions and evaluation procedures for 360° video”, JVET-L1012, October 2018, Macau, CN.