| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0421 |
| --- | --- |

| Title: | Non-CE4: On CIIP Merge mode with no residual |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno Christophe Gisquet Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents a modification of the CIIP Merge mode. With the current VVC specification, it is not possible to signal a CIIP mode without residual. This contribution proposes to signal a CIIP Merge mode without residual thanks to a cu_cbf flag.

It is reported that the proposed modification obtains the average BDR results of 0.04% 0.00%, for respectively RA and LD configurations compared to VTM5.0 without modification of the CABAC initialisation context values.

# Problem Statement

The CIIP Merge mode is enabled only for Merge mode and not for Skip mode. So a CIIP Merge CU has its cu_cbf flag equal to 1. With the current VVC design, when a cu_cbf is equal to 1 the block residual can’t be equal to zero.

Indeed, when both tu_cbf_cb and tu_cbf_cr are equal to 0, the tu_cbf_luma is inferred to be equal to 1. In addition, it is not possible to transmit all TU coefficients equal to 0 as the sig_coef_flag is inferred to be equal to 1 in that case.

Consequently, it is not possible to signal a CIIP Merge mode with a residual equal to 0. Moreover, the CIIP Skip wasn’t adopted because it gives no BDR improvement for RA and losses for LB configuration compared to the adopted CIIP configuration (cf. JVET-L0100 [1]).

Table 1 gives the results reported in JVET-L0100. The usage of the Skip and Merge CIIP (CE10.1.1.b) gives losses for LB configuration compared to the usage of the Merge mode (CE10.1.1.c) only which was adopted.

Table 1 Results of JVET-L0100 [1]

|  | Config | VTM-2.0.1 |  |  |  |  | Supported modes |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |  |
| CE10.1.1.b | RA | -0.55% | -0.62% | -0.73% | 106% | 104% | skip/merge |
|  | LB | -0.35% | -0.18% | -0.21% | 108% | 103% |  |
| CE10.1.1.c | RA | -0.51% | -0.41% | -0.50% | 108% | 103% | merge |
|  | LB | -0.50% | -0.90% | -0.83% | 109% | 103% |  |

For most encoder implementation, the residual coding is generally evaluated at a very late stage for Inter frames. Consequently the CIIP Merge mode should be less selected in the real VVC encoder than in the current VTM because it will not be evaluated. So it is desirable to enable a no residual signalling for CIIP CU to increase its efficiency in real encoder and to have more flexibility with the VVC design.

# Proposed modification

To signal a zero residual for CIIP Merge, it is proposed to transmit a cu_cbf equal to zero for this Merge mode as depicted in the following syntax tabular form.

| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| --- | --- |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && (general_merge_flag[ x0 ][ y0 ] = = 0 ¦ ¦ ciip_flag[ x0 ][ y0 ] ) ) |  |
| cu_cbf | ae(v) |

# Experimental results

The proposed modification is implemented on top of the VTM5.0 and there is no change of the initial CABAC state values.

Table 2 gives the results of the proposed modification compared to VTM5.0. Minor BDR Y losses of 0.04% and 0.00% are obtained for respectively RA and LD configurations. These results are obtained without modification of CABAC initialisation values.

Compared to the results obtained on JVET-L0100 [1], the result of the proposed method is better for LD configuration and similar for RA configuration.

Table 2 Results of proposed modification compared to VTM5.0.

# Conclusion

In this contribution, the cu_cbf is allowed for CIIP Merge in order to signal zero block residual. This modification offers more flexibility of the VVC current design by allowing the signaling of zero residual without impacting the coding efficiency.

# References

- JVET-L0100, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode”, M.-S. Chiang, C.-W. Hsu
Y.-W. Huang, S.-M. Lei, 12th Meeting: Macao, CN, 3–12 Oct. 2018

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).