| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0423 |
| --- | --- |

| Title: | Non-CE4: On SbTMVP derivation using HMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Patrice Onno Guillaume Laroche Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | <firstname>.<surname>@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes to replace the SbTMVP dependency on the offset determined by accessing a spatial neighbor, by the use of an appropriate motion vector from the stored motion candidate when updating the HMVP candidate list. In addition, as this motion vector can be considered to be almost available, compared to said spatial neighbor, the availability check for the SbTMVP is removed. This is asserted to offer a simplification, for e.g. a software decoder, or an encoder privileging one type of candidate or another in the Subblock Merge list. . It is reported that the proposed modifications obtain the average Y-BDRate results of -0.01%/ and 0.01% for respectively RA and LDB configurations compared to VTM5.0.

# Technical problem and solution

## Availability checks in SbTMVP

The SbTMVP candidate can be summarily described with the following step

- Accessing a spatial neighbor to retrieve its motion

- If the motion matches conditions related to the collocated picture, use the MV as an offset then clip the position

- Retrieve a default motion of the block at the center of the block indicated by said position in the collocated picture

- If unavailable or it is not an inter block, consider the SbTMVP candidate as unavailable

- For each subblock, perform a derivation of collocated motion vectors, using the default motion if unavailable or intra coded.

When evaluating the dependency of the SbTMVP on the above availability checks, it was measured they offer minimal benefits, if at all.

## Using the HMVP history list

Whenever the history list is updated, the selection process applied to the spatial neighbour during SbTMVP derivation is instead applied to the newly added motion. As a consequence, a motion candidate is made almost always available as part of the history.

By going one step beyond, and initializing said candidate with zero motion vectors at every CTB row start (as per the HMVP list management), a satisfying candidate is made always available. It is considered a suitable alternative default motion for the center-derived default motion. As a consequence, the SbTMVP is also considered always available.

# Results

# Conclusion

This contribution proposes several simplifications removing availability checks. In light of the resulting complexity decrease and the coding efficiency impact amounting to noise, it is suggested to adopt the proposal in the VVC Committee Draft.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).