| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0424 |
| --- | --- |

| Title: | Non-CE4: On TMVP reference frame selection using HMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Patrice Onno Guillaume Laroche Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | <firstname>.<lastname>@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

The derivation of the TMVP candidate, and similarly of the bottom-right control point for constructed merge affine candidates, is currently always targeting the first reference frame. However, when the temporal distances between reference pictures is small, it is asserted that this is not as likely to offer the best prediction. It is therefore proposed to derive the reference index to use from the HMVP candidate list. It is reported that the proposed modifications obtain the average Y-BDRate results of 0.00%/-0.09%/-0.13%, for respectively Test1 in RA, LDB and LDP configurations, and 0.01%/-0.12% for Test2 in RA/LDB, compared to VTM5.0.

# Technical problem and solution

## Temporal distances and link to coding efficiency

The random access scenario buffers a large amount of video buffer inputting it to a video encoder. As its name implies, the low delay scenario on the contrary uses as large a DPB, the total delay, as well as the temporal distance between consecutive frames of a reference picture list can be a lot smaller, making it more conservative.

Early versions of the TMVP in HEVC were making use of the reference index of the “top” and “left” neighbors. For the sake of reducing the dependency of a long-to-determine candidate on spatial neighbors that would go through availability and pruning checks, this feature was removed by simply using the first reference picture.

The random access scenario, as it uses way larger temporal distances, indeed represents a lower opportunity for the reference picture for the TMVP to matter. However, it remains that a missed opportunity exists, in particular for the low delay cases.

## Adapting the reference index using the HMVP history list

In the general merge list, the HMVP candidate is determined just after the TMVP candidate, by going through the complete HMVP list if need be. This list is updated as part of the decoding process of the previous CU.

When setting the reference index for the TMVP in this merge list (i.e. also impacting the triangle mode) and the constructed merge affine candidate, it is proposed to only consider up to 2 element of the history-based list, depending on how many are available.

The simplest efficient solution found is to take the maximum reference index of these 2 elements, if available. Indeed, the use of a large reference index is likely to indicate that the associated reference picture is quite effective for the local area.

The resulting modifications to VVC specifications therefore consist in only 14 lines, almost duplicated between the Subblock and general Merge lists.

In addition, as motion from a Triangle-coded CU does not update the history buffer, it was tested whether the merge list for Triangle-coded CUs should use the modified derivation.

# Experiments

## Test1

This test unconditionally modifies the reference index derivation of the TMVP.

## Test2

This test modifies the reference index derivation of the TMVP only when the current CU is not using triangle partitions. Also, as triangle partitions are only used in P-slices, LDP results are identical to Test1, and thus not displayed.

# Conclusion

This contribution proposes to modify the existing TMVP candidate (or control point) by using reference indices from the history-based list, itself used for the HMVP candidate that follows this TMVP candidate. This modification exploits the cases where 2 reference frames may be equally efficient to predict the current picture. Such a small modification reaches 0.1% BDRate gains on the LowDelay-B and LowDelay-P scenarios. It is therefore suggested to adopt the proposed modifies into the VVC Committee Draft.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).