| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O00427-v3 |
| --- | --- |

| Title: | Non-CE5: Remove differential coding for ALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution presents a simplification on coefficient coding in ALF. In VVC-5.0, ALF luma coefficients are coded by differential coding or non-differential coding. If using differential coding, the signalled luma filter coefficients are predicted from the deltas of previous luma coefficients. This contribution proposes to remove differential coding in ALF, which can reduce the RDcost calculation by 260 times for one frame in encoder side and remove the dependency between different filter coefficients. It is reported that as compared to VTM-5.0, it achieves 0.00%, 0.01% and 0.00% coding loss in all intra, random access and low delay B configuration respectively.

Introduction

In WD 5.0 [1], the adaptive loop filter (ALF) parameters are signalled in adaptation parameter set (APS). In one APS, up to 25 sets of luma filter coefficients and clipping value indices, and up to 1 set of chroma filter coefficients and clipping value indices are signalled for the non-linear ALF. In order to reduce the coding bits of ALF luma coefficients, there are two coding modes, differential coding and non-differential coding. In order to select from the two coding modes, RDO process should be conducted in encoder side. And a syntax should be added to indicate the coding mode. If using the differential coding, the signalled luma filter coefficients are predicted from the deltas of previous luma coefficients.

Proposed Method

The differential coding mode is used when the coefficients predicted from deltas of previous luma coefficients smaller than the original. However, ALF coefficients have no obvious numerical regularity. As shown in Figure1 and Figure2, X-axis is the coefficients index of luma component(0~12). Y-axis is the coefficients of each filter. Different line represents different ALF filters. The four sub-figures represent coefficients distribute of QP 22,27,32,37, respectively. It can be seen that ALF coefficients are irregular. Thus, ALF differential coding mode is rarely selected. However, because of the existing differential coding mode, encoder side has to calculate the RDcost of differential coding 260 times (5 times iteration * 2 times for nonlinear/linear decision * 26 times for filter merge and coefficient derivation) for one frame. And a flag is needed to identify the coding method. The different coding mode may also cause the dependency between different filters. It is not hardware friendly.

Figure 1 Coefficients distribute of MarketPlace Frame32 under RA configuration

Figure 2 Coefficients distribute of BQTerrace Frame156 under LDB configuration

This contribution proposes to remove differential coding mode in ALF. It can reduce RDcost computation by 260 times for one frame in encoder side, reduce the overhead of ALF and remove the dependency between different ALF filters.

Proposed draft text changes

The following shows the proposed draft text changes based on Draft 5 v8 when using the proposed method:

7.3.5.3	Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_use_fixed_filter_flag | u(1) |
| if( alf_luma_use_fixed_filter_flag ) { |  |
| alf_luma_fixed_filter_set_idx | tb(v) |
| alf_luma_fixed_filter_pred_present_flag | u(1) |
| if( alf_luma_fixed_filter_pred_present_flag ) { |  |
| for( i = 0; i < NumAlfFilters; i++ ) |  |
| alf_luma_fixed_filter_pred_flag[ i ] | u(1) |
| } |  |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if( alf_luma_coeff_delta_flag ) { |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| alf_luma_coeff_flag[ sfIdx ] | u(1) |
| } |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.6.3	Adaptive loop filter data semantics

…

alf_luma_coeff_delta_flag equal to 1 indicates that alf_luma_coeff_delta_prediction_flag is not signalled. alf_luma_coeff_delta_flag equal to 0 indicates that alf_luma_coeff_delta_prediction_flag may be signalled. equal to 1 indicates that alf_luma_coeff_flag is signalled. alf_luma_coeff_delta_flag equal to 0 indicated that alf_luma_coeff_flag is not signalled.

alf_luma_coeff_delta_prediction_flag equal to 1 specifies that the signalled luma filter coefficient deltas are predicted from the deltas of the previous luma coefficients. alf_luma_coeff_delta_prediction_flag equal to 0 specifies that the signalled luma filter coefficient deltas are not predicted from the deltas of the previous luma coefficients. When not present, alf_luma_coeff_delta_prediction_flag is inferred to be equal to 0.

alf_luma_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma filter coefficient signalling. The value of alf_luma_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

…

alf_luma_coeff_delta_sign[ sfIdx ][ j ] specifies the sign of the j-th luma coefficient of the filter indicated by sfIdx as follows:

- If alf_luma_coeff_delta_sign[ sfIdx ][ j ] is equal to 0, the corresponding luma filter coefficient has a positive value.

- Otherwise (alf_luma_coeff_delta_sign[ sfIdx ][ j ] is equal to 1), the corresponding luma filter coefficient has a negative value.

When alf_luma_coeff_delta_sign[ sfIdx ][ j ] is not present, it is inferred to be equal to 0.

The variable filtCoeff[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filtCoeff[ sfIdx ][ j ] = alf_luma_coeff_delta_abs[ sfIdx ][ j ] * 	(7-69)
	( 1 − 2 * alf_luma_coeff_delta_sign[ sfIdx ][ j ] )

When alf_luma_coeff_delta_prediction_flag is equal 1, filtCoeff[ sfIdx ][ j ] with sfIdx = 1..alf_luma_num_filters_signalled_minus1 and j = 0..11 are modified as follows:

filtCoeff[ sfIdx ][ j ] += filtCoeff[ sfIdx − 1 ][ j ]		(7-70)

The luma filter coefficients AlfCoeffL[ adaptation_parameter_set_id ] with elements AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as follows:

AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] = filtCoeff[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]	(7-71)

When alf_luma_use_fixed_filter_flag is equal to 1 and alf_luma_fixed_filter_pred_flag[ filtIdx ] is equal to 1, the following applies for filtIdx = 0..NumAlfFilters − 1 and j = 0..11:

AlfCoeffL[ adaptation_parameter_set_id ][ filtIdx ][ j ] += 	(7-72)
	AlfFixFiltCoeff[ AlfClassToFiltMap[ alf_luma_fixed_filter_set_idx ][ filtIdx ] ][ j ]

Simulation results

The experiments were performed based on VTM-5.0 software [2] with VTM configuration files under JVET common test condition [3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method compared to VTM-5.0 anchor are summarized in Table I. The average BD-rates of 0.00%, 0.01%, and 0.xx% are observed for AI, RA, and LDB cases, respectively. It can be noted that the proposed methods can reduce the encoder and decoder computational complexity with minor coding loss.

Table I. Experimental result of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class E | 0.01% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | -0.03% | 101% | 102% |
| Class A2 | 0.01% | 0.07% | -0.01% | 100% | 101% |
| Class B | 0.02% | -0.01% | -0.02% | 101% | 103% |
| Class C | 0.00% | 0.02% | 0.06% | 100% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.00% | 100% | 103% |
| Class D | 0.00% | 0.01% | 0.09% | 99% | 101% |
| Class F | 0.00% | 0.06% | 0.05% | 94% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.29% | 0.28% | 100% | 100% |
| Class C | 0.01% | 0.00% | -0.11% | 100% | 102% |
| Class E | 0.02% | 0.41% | 0.29% | 87% | 86% |
| Overall | 0.00% | 0.22% | 0.15% | 96% | 100% |
| Class D | 0.02% | -0.38% | 0.05% | 100% | 96% |
| Class F | -0.05% | 0.04% | -0.24% | 85% | 100% |

Conclusions

In this contribution, a simplification on coefficient coding in the adaptive loop filter (ALF) is proposed. The average BD-rates of 0.00%, 0.01%, and 0.00% are observed for AI, RA, and LDB cases, respectively. It can be noted that the proposed methods can reduce the RDcost calculation by 260 times for one frame in encoder side and remove the dependency between different filter coefficients with minor coding loss. Therefore, it is recommended to adopt this proposal to simply the current adaptive loop filter coefficient coding.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).