| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0430-v2 |
| --- | --- |

| Title: | Non-CE5: Modification of clipping value signalling for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution presents a simplification of clipping value signalling for ALF in VTM-5.0. In VVC-5.0, ALF clipping parameter is coded by K-th order Exponential Golomb in non-linear ALF. This contribution proposes to use fixed length code and truncated unary code for both luma and chroma ALF clipping parameters. The test 1 replaces K-th order Exponential Golomb code with fixed length code. The test 2 replaces K-th order Exponential Golomb code with truncated unary code. The test 3 replaces K-th order Exponential Golomb code with truncated unary code while reordering the clip parameter according to the statistical probability of different clip parameter index. All the three tests remove 4 syntax elements and corresponding semantics used to derive the order K of K-th order Exponential Golomb code. Compared to VTM-5.0, it is reported that the test 1 shows average BD-rates of -0.01%, -0.02%, and 0. 00% for AI, RA, and LDB configurations, respectively. The test 2 shows average BD-rates of -0.02%, -0.02%, and 0.01% for AI, RA, and LDB configurations, respectively. And the test 3 shows average BD-rates of -0.02%, -0.03%, and -0.02% for AI, RA, and LDB configurations, respectively.

Introduction

In VVC5.0 [1], ALF parameters are signalled in adaptation parameter set (APS) [2]. In one APS, up to 25 sets of luma filter coefficients and clipping value indices, and up to 1 set of chroma filter coefficients and clipping value indices are signalled for non-linear ALF [3]. The value of filter coefficients is in the range of -128 to 127. And the value of clipping value index is in the range of 0 to 3.

The clipping parameters are signalled per filter coefficient. Therefore, up to 12 clipping indices for 1 set of luma filter and up to 6 clipping indices for 1 set of chroma filter are signalled by using K-th order Exponential Golomb code. 12 * 25 clipping indices for luma filters and 6 clipping indices for chroma filter would be signalled in the worst case. As K-th order Exponential Golomb code is not efficient for clipping parameters which has a small range of values, this contribution proposes to use fixed length code and truncated unary code for both luma and chroma ALF clipping parameters.

Proposed Method

For ALF clipping parameter coding for both luma and chroma,

Test 1: Replaces K-th order Exponential Golomb code with fixed length code u(2).

Test 2: Replaces K-th order Exponential Golomb code with truncated unary code tu(v) with maximum value 3.

Test 3: Replaces K-th order Exponential Golomb code with truncated unary code tu(v) with maximum value 3. The index 0, 1, 2, 3 is coded by tu(0), tu(2), tu(1), tu(3), respectively

All the three tests can remove the following syntax elements and corresponding semantics used to derive the order K of K-th order Exponential Golomb code.

- alf_luma_clip_min_eg_order_minus1

- alf_luma_clip_eg_order_increase_flag[ i ]

- alf_chroma_clip_min_eg_order_minus1

- alf_chroma_clip_eg_order_increase_flag[ i ]

Test 1 (fixed length code)

The following shows the proposed draft text changes based on Draft 5 v8 when using the test 1:

7.3.5.3	Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v)u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v)u(2) |
| } |  |
| } |  |
| … |  |

7.4.6.3	Adaptive loop filter data semantics

…

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

…

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

…

Test 2 (truncated unary code)

The following shows the proposed draft text changes based on Draft 5 v8 when using the test 2:

7.3.5.3	Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v)tu(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v)tu(v) |
| } |  |
| } |  |
| … |  |

7.4.6.3	Adaptive loop filter data semantics

…

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The maximum value maxVal of the truncated unary binarization tu(v) is set to 3.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

…

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The maximum value maxVal of the truncated unary binarization tu(v) is set to 3.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

…

Simulation results

The experiments were performed based on VTM-5.0 software [3] with VTM configuration files under JVET common test condition [4]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed methods compared to VTM-5.0 anchor are summarized in Table 1, Table 2 and Table3. For the test 1, the average BD-rates of -0.01%, -0.02%, and 0.xx% are observed for AI, RA, and LDB configurations, respectively. For the test 2, the average BD-rates of -0.02%, -0.02%, and 0.xx% are observed for AI, RA, and LDB configurations, respectively. For the test 3, the average BD-rates of -0.02%, -0.03%, and 0.xx% are observed for AI, RA, and LDB configurations, respectively. Thus, it can be noted that the proposed methods can simplify the coding process of ALF clipping parameter while improve the coding efficiency.

Table 1. Experimental result of the proposed method (Test 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | -0.01% | 0.00% | 0.00% | 101% | 107% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.02% | -0.01% | -0.01% | 101% | 107% |
| Class E | -0.02% | -0.01% | -0.02% | 101% | 104% |
| Overall | -0.01% | -0.01% | -0.01% | 101% | 103% |
| Class D | -0.02% | 0.01% | -0.02% | 100% | 106% |
| Class F | -0.02% | -0.01% | -0.01% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | -0.04% | 101% | 103% |
| Class A2 | -0.01% | -0.05% | -0.02% | 108% | 105% |
| Class B | -0.01% | 0.00% | -0.02% | 99% | 101% |
| Class C | -0.04% | -0.11% | -0.12% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.05% | 100% | 102% |
| Class D | -0.04% | 0.00% | 0.04% | 99% | 99% |
| Class F | -0.03% | 0.05% | 0.01% | 94% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.21% | 0.02% | 100% | 100% |
| Class C | 0.03% | 0.10% | -0.21% | 102% | 110% |
| Class E | -0.02% | 0.39% | -0.30% | 88% | 95% |
| Overall | 0.00% | 0.22% | -0.14% | 97% | 100% |
| Class D | -0.07% | -0.55% | 1.51% | 100% | 101% |
| Class F | 0.04% | 0.05% | 0.82% | 84% | 100% |

Table 2. Experimental result of the proposed method (Test 2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 100% |
| Class C | -0.03% | -0.01% | -0.01% | 101% | 109% |
| Class E | -0.03% | -0.02% | -0.03% | 102% | 102% |
| Overall | -0.02% | -0.01% | -0.01% | 100% | 102% |
| Class D | -0.03% | 0.01% | -0.02% | 100% | 104% |
| Class F | -0.02% | -0.01% | 0.00% | 101% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.15% | 0.01% | 112% | 108% |
| Class A2 | -0.03% | -0.04% | -0.02% | 100% | 102% |
| Class B | -0.01% | 0.01% | -0.11% | 99% | 100% |
| Class C | -0.04% | 0.00% | -0.13% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.07% | 101% | 102% |
| Class D | -0.02% | -0.06% | 0.07% | 99% | 98% |
| Class F | -0.03% | 0.03% | 0.04% | 94% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.26% | 0.13% | 100% | 100% |
| Class C | 0.00% | 0.08% | -0.12% | 100% | 104% |
| Class E | 0.07% | 0.09% | -0.14% | 86% | 86% |
| Overall | 0.01% | 0.15% | -0.02% | 96% | 100% |
| Class D | -0.05% | -0.39% | 0.37% | 100% | 100% |
| Class F | 0.01% | -0.18% | -0.20% | 85% | 100% |

Table 3. Experimental result of the proposed method (Test 3)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | 0.00% | 0.00% | 100% | 102% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.03% | -0.01% | -0.01% | 98% | 99% |
| Class E | -0.03% | -0.01% | -0.03% | 102% | 104% |
| Overall | -0.02% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.03% | 0.01% | -0.01% | 99% | 101% |
| Class F | -0.01% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | 0.05% | 101% | 103% |
| Class A2 | -0.04% | 0.05% | 0.01% | 104% | 102% |
| Class B | -0.02% | -0.02% | -0.07% | 99% | 101% |
| Class C | -0.07% | -0.09% | -0.11% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.06% | -0.04% | 100% | 101% |
| Class D | -0.04% | 0.02% | 0.04% | 99% | 98% |
| Class F | -0.02% | 0.04% | 0.06% | 94% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.14% | 0.02% | 99% | 100% |
| Class C | -0.02% | -0.23% | -0.12% | 100% | 103% |
| Class E | -0.04% | 0.13% | -0.29% | 85% | 82% |
| Overall | -0.02% | 0.01% | -0.10% | 96% | 100% |
| Class D | -0.09% | -0.04% | 0.38% | 100% | 97% |
| Class F | 0.08% | 0.06% | -0.09% | 84% | 100% |

Conclusions

In this contribution, a simplification of clipping value signalling for ALF in VTM-5.0 is proposed. This contribution proposes to use fixed length code and truncated unary code for both luma and chroma ALF clipping parameters. The proposed methods can simplify the coding process of ALF clipping parameter while improve the coding efficiency Therefore, it is recommended to adopt this proposal to simplify the current non-linear ALF design.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- J. Taquet, P. Onno, C. Gisquet, G. Laroche, “CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0242, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).