| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0431-v2 |
| --- | --- |

| Title: | Non-CE4: On Merge data signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Xu Chen Haitao Yang | Tel: Email: | chenhuanbang@huawei.com anderson.chen@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In current VVC, there are 5 merge modes, including regular merge mode, merge mode with motion vector different (MMVD), sub-block merge mode, combined inter-intra prediction (CIIP) and triangular prediction mode (TPM). They are syntax redundancies for some of the merge modes in case of specific on/off combinations of other merge modes. A sanity checking of all the possible configurations for these merge modes is conducted. A modified conditional parsing of merge data syntax is proposed to remove the redundancies. Simulation results reportedly show -0.20%/-0.22% luma BR-rate for RA/LB configurations, compared to VTM-5.0 with TPM off.

Problem statements

In current VVC, there are 5 merge modes, including regular merge mode, merge mode with motion vector different (MMVD), sub-block merge mode, combined inter-intra prediction (CIIP) and triangular prediction mode (TPM). They are syntax redundancies for some of the merge modes in case of specific on/off combinations of other merge modes.

In following cases, the syntaxes are redundant:

- If MMVD, sub-block merge mode, CIIP and TPM are not allowed, regular_merge_flag is redundant;

- If sub-block merge mode, CIIP and TPM are not allowed, regular_merge_flag is false and MMVD is allowed, mmvd_merge_flag is redundant;

- If CIIP and TPM are not allowed, regular_merge_flag and mmvd_merge_flag are false and sub-block merge mode is allowed, merge_subblock_flag is redundant;

- If TPM is not allowed, regular_merge_flag, mmvd_merge_flag and merge_subblock_flag are false and CIIP is allowed, ciip_flag is redundant.

Proposed method

In following cases, these merge modes are inferred in following cases:

- If MMVD, sub-block merge mode, CIIP and TPM are not allowed, regular_merge_flag is inferred to be 1;

- If sub-block merge mode, CIIP and TPM are not allowed, regular_merge_flag is false and MMVD is allowed, mmvd_merge_flag is inferred to be 1;

- If CIIP and TPM are not allowed, regular_merge_flag and mmvd_merge_flag are false and sub-block merge mode is allowed, merge_subblock_flag is inferred to be 1;

- If TPM is not allowed, regular_merge_flag, mmvd_merge_flag and merge_subblock_flag are false and CIIP is allowed, ciip_flag is inferred to be 1.

The parsing conditions and inferred values for each mode are summarized in Table 1.

Table 1 Parsing conditions and inferred values for each mode

| Mode | Syntax | Allowed flag | Parsing condition | Inferred value if not present |
| --- | --- | --- | --- | --- |
| Regular merge | regular_merge_flag | / | allowMMVD + allowSubblock + allowCIIP + allowTPM > 0 | general_merge_flag |
| MMVD | mmvd_merge_flag | allowMMVD = sps_mmvd_enabled_flag | !regular_merge_flag && allowMMVD && allowSubblock + allowCIIP + allowTPM > 0 | allowMMVD && general_merge_flag && !regular_merge_flag |
| sub-block merge mode | merge_subblock_flag | allowSubblock = MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 | !regular_merge_flag && !mmvd_merge_flag && allowSubblock && allowCIIP + allowTPM > 0 | allowSubblock && general_merge_flag && !regular_merge_flag && !mmvd_merge_flag |
| CIIP | ciip_flag | allowCIIP = sps_ciip_enabled_flag && !cu_skip_flag && (cbWidth*cbHeight) >= 64 && cbWidth < 128 && cbHeight < 128 | !regular_merge_flag && !mmvd_merge_flag && !merge_subblock_flag && allowCIIP && allowTPM | allowCIIP && general_merge_flag && !regular_merge_flag && !mmvd_merge_flag && !merge_subblock_flag |
| TPM | / | allowTPM = sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 && (cbWidth*cbHeight) >= 64 | / | allowTPM && general_merge_flag && !regular_merge_flag && !mmvd_merge_flag && !merge_subblock_flag && !ciip_flag |

Experiment results

The proposed methods has been implemented on top of VTM5.0. TPM is turned off by command line for both anchor and test.

Table 2 Coding performance of the proposal over VTM5.0–TPM

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0-TPM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.28% | -0.28% | 100% | 100% |
| Class A2 | -0.19% | -0.29% | -0.23% | 100% | 101% |
| Class B | -0.18% | -0.16% | -0.12% | 101% | 102% |
| Class C | -0.22% | -0.18% | -0.20% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.22% | -0.20% | 100% | 101% |
| Class D | -0.19% | -0.39% | -0.25% | 100% | 101% |
| Class F | -0.14% | -0.12% | -0.19% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0-TPM |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.35% | -0.60% | 100% | 102% |
| Class C | -0.20% | -0.43% | -0.30% | 100% | 101% |
| Class E | -0.35% | -0.14% | -0.04% | 101% | 104% |
| Overall | -0.22% | -0.32% | -0.36% | 100% | 102% |
| Class D | -0.28% | -0.59% | -0.85% | 100% | 100% |
| Class F | -0.03% | -0.90% | -1.29% | 101% | 103% |

Conclusions

In current VVC, there are 5 merge modes, including regular merge mode, merge mode with motion vector different (MMVD), sub-block merge mode, combined inter-intra prediction (CIIP) and triangular prediction mode (TPM). They are syntax redundancies for some of the merge modes in case of specific on/off combinations of other merge modes. A sanity checking of all the possible configurations for these merge modes is conducted. A modified conditional parsing of merge data syntax is proposed to remove the redundancies.

Reference

- VTM-5.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Working Draft

The text to be added is mark as green and the text to be removed is marked as red with strikethrough.

- 7.3.7.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag ¦ ¦ cbWidth * cbHeight != 32allowMMVD + allowSubblock + allowCIIP + allowTPM > 0 ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mmvd_enabled_flag && cbWidth * cbHeight != 32allowMMVD && allowSubblock + allowCIIP + allowTPM > 0 ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 allowSubblock && allowCIIP + allowTPM > 0 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 allowCIIP && allowTPM ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

- 7.4.8.7 Merge data semantics

The variables allowMMVD, allowSubblock, allowCIIP, and allowTPM are derived as follows:

allowMMVD = sps_mmvd_enabled_flag		(7-122)

allowSubblock = MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8	(7-122)

allowCIIP = sps_ciip_enabled_flag && !cu_skip_flag && (cbWidth*cbHeight) >= 64 && cbWidth < 128  && cbHeight < 128		(7-122)

allowTPM = sps_triangle_enabled_flag && slice_type == B && MaxNumTriangleMergeCand >= 2 && (cbWidth*cbHeight) >= 64		(7-122)

regular_merge_flag[ x0 ][ y0 ] equal to 1 specifies that regular merge mode is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When regular_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:to be general_merge_flag[ x0 ][ y0 ].

- If all the following conditions are true, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flag is equal to 0.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- Otherwise, regular_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

mmvd_merge_flag[ x0 ][ y0 ] equal to 1 specifies that merge mode with motion vector difference is used to generate the inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mmvd_merge_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_mmvd_enabled_flagallowMMVD is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- cbWidth*cbHeight is equal to 32.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, mmvd_merge_flag[ x0 ][ y0 ] is inferred to be equal to 0.

mmvd_cand_flag[ x0 ][ y0 ] specifies whether the first (0) or the second (1) candidate in the merging candidate list is used with the motion vector difference derived from mmvd_distance_idx[ x0 ][ y0 ] and mmvd_direction_idx[ x0 ][ y0 ]. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When mmvd_cand_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

mmvd_distance_idx[ x0 ][ y0 ] specifies the index used to derive MmvdDistance[ x0 ][ y0 ] as specified in Table 7-12. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-12 – Specification of MmvdDistance[ x0 ][ y0 ] based on mmvd_distance_idx[ x0 ][ y0 ].

| mmvd_distance_idx[ x0 ][ y0 ] | MmvdDistance[ x0 ][ y0 ] |  |
| --- | --- | --- |
|  | slice_fpel_mmvd_enabled_flag = = 0 | slice_fpel_mmvd_enabled_flag = = 1 |
| 0 | 1 | 4 |
| 1 | 2 | 8 |
| 2 | 4 | 16 |
| 3 | 8 | 32 |
| 4 | 16 | 64 |
| 5 | 32 | 128 |
| 6 | 64 | 256 |
| 7 | 128 | 512 |

mmvd_direction_idx[ x0 ][ y0 ] specifies index used to derive MmvdSign[ x0 ][ y0 ] as specified in Table 7-13. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-13 – Specification of MmvdSign[ x0 ][ y0 ] based on mmvd_direction_idx[ x0 ][ y0 ]

| mmvd_direction_idx[ x0 ][ y0 ] | MmvdSign[ x0 ][ y0 ][0] | MmvdSign[ x0 ][ y0 ][1] |
| --- | --- | --- |
| 0 | +1 | 0 |
| 1 | -1 | 0 |
| 2 | 0 | +1 |
| 3 | 0 | -1 |

Both components of of the merge plus MVD offset MmvdOffset[ x0 ][ y0 ] are derived as follows:

MmvdOffset[ x0 ][ y0 ][ 0 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][0]	(7-124)

MmvdOffset[ x0 ][ y0 ][ 1 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][1]	(7-125)

merge_subblock_flag[ x0 ][ y0 ] specifies whether the subblock-based inter prediction parameters for the current coding unit are inferred from neighbouring blocks. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_subblock_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.as follows:

- If all the following conditions are true, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- allowSubblock is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, merge_subblock_flag[ x0 ][ y0 ] is inferred to be equal to 0.

merge_subblock_idx[ x0 ][ y0 ] specifies the merging candidate index of the subblock-based merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_subblock_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.as follows:

- If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- allowCIIP is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is equal to 1, the variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to INTRA_PLANAR.

The variable MergeTriangleFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice. is derived as follows:

- If all the following conditions are true, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 1:

- sps_triangle_enabled_flagallowTPM is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumTriangleMergeCand is greater than or equal to 2.

- cbWidth * cbHeight is greater than or equal to 64.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- mmvd_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeTriangleFlag[ x0 ][ y0 ] is set equal to 0.

merge_triangle_split_dir[ x0 ][ y0 ] specifies the splitting direction of merge triangle mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_split_dir[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

[Ed. (BB): Add semantics of what value corresponds to horizontal and which corresponds to vertical direction as well as consider adding the flag suffix.]

merge_triangle_idx0[ x0 ][ y0 ] specifies the first merging candidate index of the triangular shape based motion compensation candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_idx0[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

merge_triangle_idx1[ x0 ][ y0 ] specifies the second merging candidate index of the triangular shape based motion compensation candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_triangle_idx1[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

merge_idx[ x0 ][ y0 ] specifies the merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_idx[ x0 ][ y0 ] is not present, it is inferred as follows:

- If mmvd_merge_flag[ x0 ][ y0 ] is equal to 1, merge_idx[ x0 ][ y0 ] is inferred to be equal to mmvd_cand_flag[ x0 ][ y0 ].

- Otherwise (mmvd_merge_flag[ x0 ][ y0 ] is equal to 0), merge_idx[ x0 ][ y0 ] is inferred to be equal to 0.