| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0432-v1 |
| --- | --- |

| Title: | AHG10: LMCS encoder improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Sean McCarthy, Taoran Lu, Peng Yin, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {fpu, tlu, pyin}@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes non-normative encoder algorithm optimizations for LMCS to improve coding performance for SDR and enable LMCS for HLG. The specific changes proposed are:

- support LMCS for HDR-HLG CTC content in RA and AI test conditions;

- replace the fixed 5x5 neighborhood size used to calculate local spatial variance with a variable neighborhood size that increases or decreases based on picture resolution;

- simplify the method of allocating codewords based on histogram and local spatial variance; and,

- provide LMCS encoder configuration parameters in accordance with “Guidelines for VVC reference software development.”

The coding performance for SDR CTC using the proposed encoder implementation compared to VTM5.0 anchor (LMCS enabled) in terms of BDRate for PSNR Y, U, and V and processing times is:

- AI     {-0.30%,   0.33%,  0.32%}, Enc: 101%, Dec: 100%

- RA    {-0.16%, -0.11%, -0.12%}, Enc:  98%, Dec:  99%

- LDB  {-0.20%, -0.37%, -0.59%}, Enc: 100%, Dec: 99%

The coding performance for HLG CTC using the proposed encoder implementation compared to VTM5.0 anchor (LMCS disabled) in terms of BDRate for PSNR Y, U, and V and processing times is:

- AI     {-0.17%, 2.04%, 1.99%}, Enc: 103%, Dec: 103%

- RA    {-1.03%, 2.28%, 1.94%}, Enc: 100%, Dec: 103%

# Introduction

In VTM5.0, the non-normative encoder implementation for LMCS (luma mapping with chroma residue scaling) only supports SDR and HDR-PQ signals [1][2][3]. It does not support HDR-HLG signal.

At the 14th meeting of JVET, it was agreed to use PSNR Y, U, V metrics for HLG [4], which enabled development and testing of a non-normative encoder implementation for LMCS to include HLG.

This contribution proposes several non-normative encoder changes to improve performance for SDR and allow LMCS to be enabled for HLG.

# Proposed LMCS encoder model

JVET-N1002 [1] provides a description of the current non-normative reference implementation in the VTM5.0 encoder to estimate LMCS model parameters.

This contribution proposes the following changes to the non-normative reference implementation to estimate LMCS model parameters for SDR and HLG:

## Resolution-adaptive determination of local spatial variance

In the current VTM5.0 implementation for SDR, the local spatial variance of luma sample values for each luma sample location in the picture is calculated using a 5x5 neighborhood centered on the current position. It is proposed that the fixed 5x5 neighborhood be replaced with a neighborhood size that depends on the resolution of the picture so that the neighborhood size increases with increasing picture resolution. The proposed derivation of the neighborhood size for SDR and HLG is as follows:

winSize=min(width,height)240*2+1		(1)

## Codeword allocation and adaptation

In current VTM5.0, the number of codewords per bin is allocated based on the average of the local spatial variances over all luma values within each bin.

It is proposed to replace the current codeword allocation implementation with the following four-step process:

- allocate an equal number of codewords per bin for all valid bins;

- adjust codeword allocation so that more codewords are allocated to bins with lower average local spatial variance and fewer codewords are allocated to bins with higher average local variance;

- if the total number of codewords allocated exceeds the maximum number of allowed codewords. reduce the total number of codewords allocated;

- set LMCS slice type, high bit rate, and chroma adjust adaptation parameters based on the relative histogram and average local spatial variances of luma before and after reshaping.

In step 1, the number of allocated codewords for each bin given an initial total number of codewords and bin numbers is derived as:

binCW[i]=roundtotalCWendIdx-startIdx+1		(2)

Where binCW[i] is the number of codewords allocated to the ith bin, totalCW is the total number of codewords allowed, and startIdx and endIdx are the index values for the first and last valid bins.

In step 2, the adjustment applied for each initial number of codewords allocated for each valid bin in step 1 is computed as:

if normVar<1.0,

binCW[i]=binCWi+delta1,0.8≤normVar<0.9binCWi+delta2,normVar<0.8	(3)

else if normVar>1.0,

binCW[i]=binCWi-delta1,1.1<normVar≤1.2binCWi-delta2,normVar>1.2	(4)

where normVar=binVar[i]/meanVar, delta1=round10*hist, delta2=round20*hist.  The meanings of the variables are: binVar[i] is the average local spatial variance for the luma values in the ith bin; meanVar is the mean of the average local spatial variances across all valid bins; and hist is the percentage of pixels in the ith bin over the total number of pixels, clipped to the range of [0, 0.4].

In step 3, codeword checking is performed to restrict total codeword less than or equal to the maximum allowed number of codewords. If the total codeword exceeds the maximum value, the number of codewords is reduced by equal amount starting from the first bin.

In step 4, LMCS slice type, high bit rate, and chroma adjust adaptation parameters are set. Slice type adaption refers to enabling LMCS for the follow slice type combinations: intra and inter; for Tid0 only; or for inter only. High bit rate adaption refers to adjusting the number of code words allocated to bins for QP values less than or equal to 22. Chroma adjust adaptation refers to disabling or enabling chroma residue scaling. Adaptation decisions are based on a series of threshold comparisons. For example, if the average local spatial variance of the original picture is large, or if the average local variance of the mapped picture is large compared to the original, then LMCS is disabled for intra. As another example, if the standard deviation of the chroma component is large compared to the standard deviation of the luma component, then disable chroma residue scaling.

One encoder bug was found in original LMCS encoder design. A redundant rounding process was applied for chroma lambda computation in CRS [6]. The proposed LMCS model applies the fix by removing the rounding operation.

## LMCS encoder configuration parameters

In accordance with JVET-N1003 [5], “Guidelines for VVC reference software development,” the LMCS encoder configuration parameters listed in Table 1 are changed as proposed in Table 2.

Table 1 LMCS encoder configuration parameter settings in VTM5.0

| Parameter Name | Default Value | Description |
| --- | --- | --- |
| LumaReshapeEnable | 1 | 0: disable; 1: enable |
| ReshapeSignalType | 0 | 0: SDR; 1: PQ; 2: HLG |

Table 2 LMCS encoder configuration parameter settings for proposed solution

| Parameter Name | Default Value | Description |
| --- | --- | --- |
| LumaReshapeEnableLMCSEnable | 1 | 0: disable; 1: enable |
| ReshapeSignalTypeLMCSSignalType | 0 | 0: SDR; 1: PQ; 2: HLG |
| LMCSUpdateCtrl | 0 | 0: RA; 1: AI; 2: LDB/LDP |
| LMCSAdpOption | 0 | 0: automatic model; 1: LMCS enabled for intra and inter, and 66 codewords allocated for each bin for QP<=22; 2: LMCS enabled only for Tid0 (for all QP); 3: LMCS enabled only for inter, and 66 codewords allocated for each bin for QP<=22; 4: LMCS enabled for inter (for all QP). |
| LMCSInitialCW | 0 | user-defined total number of codewords |

Note: LMCSEnable and LMCSSignalType replace the configuration parameters LumaReshapeEnable and ReshapeSignalType in VTM5.0, respectively.

# Simulation results

Table 3 shows the testing results on SDR CTC [3] with the proposed LMCS encoder implementation compared to SDR CTC anchor (LMCS enabled).

Table 3 Test results for SDR CTC over VTM5.0 (LMCS enabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -1.91% | -2.13% | 90% | 101% |
| Class A2 | -1.07% | -1.38% | -1.34% | 91% | 104% |
| Class B | -0.20% | 0.75% | 0.84% | 104% | 102% |
| Class C | -0.21% | 1.84% | 1.81% | 107% | 96% |
| Class E | 0.05% | 1.58% | 1.55% | 112% | 95% |
| Overall | -0.30% | 0.33% | 0.32% | 101% | 100% |
| Class D | -0.27% | 2.67% | 2.40% | 106% | 98% |
| Class F | -0.07% | 0.23% | 0.25% | 108% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | -0.96% | -0.80% | 96% | 103% |
| Class A2 | -0.70% | 1.96% | 1.86% | 98% | 100% |
| Class B | -0.19% | -0.71% | -0.80% | 99% | 97% |
| Class C | 0.01% | -0.26% | -0.27% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.11% | -0.12% | 98% | 99% |
| Class D | -0.09% | -0.82% | -0.87% | 100% | 93% |
| Class F | -0.04% | -0.21% | -0.18% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.42% | -1.24% | -1.63% | 100% | 102% |
| Class C | -0.02% | 0.92% | 1.05% | 104% | 96% |
| Class E | -0.08% | -0.64% | -1.03% | 95% | 99% |
| Overall | -0.20% | -0.37% | -0.59% | 100% | 99% |
| Class D | 0.06% | -0.97% | -0.93% | 101% | 96% |
| Class F | 0.03% | -0.83% | -0.24% | 98% | 101% |

Table 4 shows the testing results on HLG CTC [4] with the proposed LMCS encoder implementation compared to HLG CTC anchor (LMCS disabled).

Table 4 Test results for HLG CTC over VTM5.0 (LMCS disabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H (HLG) | -0.17% | 2.04% | 1.99% | 103% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class H (HLG) | -1.03% | 2.28% | 1.94% | 100% | 103% |

# Conclusion

This proposal further improves LMCS encoder algorithm and software in several aspects: 1) enables LMCS for HLG signal type; 2) simplifies derivation process of LMCS model; 3) improves coding performance; and 4) provides configurable LMCS parameters for user tuning. It is recommended to adopt the proposed non-normative encoder implementation into VTM.

# Acknowledgements

We would like to thank NHK and LG for crosschecking the simulation results.

# References

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1002, Geneva, CH, 19–27 Mar. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

- A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-N1011, Geneva, CH, 19–27 Mar. 2019.

- G. Sullivan, J.-R. Ohm, “Meeting Report of the 14th Meeting of the Joint Video Experts Team (JVET)”, JVET-N_Notes_dD, Geneva, CH, 19–27 Mar 2019

- F. Bossen, X. Li, K. Suehring, “Guidelines for VVC reference software development”, JVET-N1003, Geneva, CH, 19-27 Mar, 2019.

- T. Lu, P. Yin, E. Francois, F. Hiron, “Non-CE2: alternative solutions for chroma residual scaling factors derivation for dual tree”, JVET-O0429, Gothenburg, SE, 3–12 July 2019

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).