| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0433 |
| --- | --- |

| Title: | AHG15: chroma quantization parameters QpC table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangjun Pu, Taoran Lu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 940850 | Tel: Email: | {fpu, tlu, pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes syntax and semantics for signalling user-defined chroma quantization parameter QpC tables for any signal type. The proposed syntax and semantics support signalling of either one QpC table to be shared by both chroma components or a pair of QpC tables, one for each chroma component. The proposed syntax and semantics can be used for any signal type, including SDR and HDR. This contribution additionally proposes that default chroma quantization parameter QpC tables be defined to provide an option to simplify signalling and reduce the number of bits required to signal QpC tables.  Three default QpC tables are proposed to support the following use cases: SDR Y'CbCr, HDR PQ Y'CbCr and HDR HLG Y'CbCr.(The proposed syntax and semantics allow additional default QpC tables, for example default tables for ICTCP, to be specified later if desired.)

The maximum number of bits required to signal QpC table(s) for each option is as follows:

- two QpC tables (one for each chroma component): 170 bits

- one QpC table (shared by both chroma components): 86 bits

- default QpC table (shared by both components): 2 bits for SDR and 4 bits for HDR

# Introduction

JVET-N0221 [1] proposed that a default chroma quantization parameter QpC table for HDR content be added to VVC to complement the existing default chroma quantization parameter QpC table inherited from HEVC for SDR content.  In JVET-N0221, it was noted that early JCT-VC testing showed that the default SDR QpC table when used with HDR content resulted in chroma artefacts, particularly at low bit rates and in achromatic regions [2][3].  JVET-N0221 proposed that the addition of a default QpC table for HDR would resolve such artefacts.

During discussions of JVET-N0221 at the fourteenth meeting of JVET, it was noted that there is no evidence that the default SDR QpC table is optimal even for SDR content, and that it might be better to signal QpC tables rather than relying on default tables.

The current contribution proposes unified syntax and semantics for signalling QpC tables for any signal type, including SDR and HDR. The proposed syntax and semantics enable the following options:

- Signalling that either chroma-specific QpC tables are present for each chroma component or that one shared QpC table is present for both chroma components

- Signalling that no QpC table is present and that a default QpC table should be used

- In the case that a default QpC table is indicated, signalling which of several default QpC table options should be used.

The current contribution proposes three default QpC tables, one for SDR and two for HDR (one each for HLG and PQ). The proposed values for the default SDR QpC table are the same as those in the current VVC specification [4].  The proposed values for the default HDR HLG QpC table are derived based on chromaQPOffset as proposed in JVET-J0015 CfP [5], which uses a method similar to that of the chromaQPOffset derivation in the HDR PQ CTC anchor [6][7][8]. For HDR PQ signals, this contribution proposes the QpC table proposed in JVET-N0221[1].

# Proposal

## Explicitly-signaled QpC tables

Each explicitly-signaled QpC table is coded by signalling the start index (start_idx) and end index (end_idx) for qPi. The value between start_idx and end_idx is delta coded from the previous QpC value.  The delta value is signaled in the syntax. The current contribution proposes that the delta value be restricted to be either 0 or 1.

Pseudo code indicating how to derive QpC[ ] given a qPi value is shown below:

if qPi < start_idx

QpC[qPi] = qPi;

else if qPi <= end_idx

QpC[qPi]  = QpC[qPi – 1] + delta;  //delta is signaled in the syntax, only values of 0 and 1 are allowed

else (qPi > end_idx) {

delta_end = end_idx − QpC[end_idx] ;

QpC[qPi]  = qPi  −  delta_end;

}

## Proposed syntax and semantics

The proposed syntax is shown in Table 1.

Table 1 Proposed syntax in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| …… |  |
| sps_default_qpc_table_flag | u(1) |
| if( sps_default_qpc_table_flag ) |  |
| sps_default_qpc_idc | ue(v) |
| else { |  |
| sps_separate_qpc_table_flag | u(1) |
| sps_qpc_cb_start_idx | u(7) |
| sps_qpc_cb_end_idx | u(7) |
| for( i = sps_qpc_cb_start_idx; i <= sps_qpc_cb_end_idx; i++ ) |  |
| sps_qpc_cb_delta[ i ] | u(1) |
| if( sps_separate_qpc_table_flag ) { |  |
| sps_qpc_cr_start_idx | u(7) |
| sps_qpc_cr_end_idx | u(7) |
| for( i = sps_qpc_cr_start_idx; i <= sps_qpc_cr_end_idx; i++ ) |  |
| sps_qpc_cr_delta[ i ] | u(1) |
| } |  |
| } |  |
| …… |  |
| } |  |

sps_default_qpc_table_flag equal to 1 specifies that a default chroma quantization parameter QpC table specified by sps_default_qpc_idc is used. sps_default_qpc_table_flag equal to 0 specifies that default QpC table is not used.

sps_default_qpc_idc indicates which default QpC table is used when sps_default_qpc_table_flag equal to 1. sps_default_qpc_idc equal to 0 indicates the default SDR Y’CbCr QpC table. sps_default_qpc_idc equal to 1 indicates the default HDR PQ Y’CbCr QpC table. sps_default_qpc_idc equal to 2 indicates the default HDR HLG Y’CbCr QpC table. sps_default_qpc_idc greater than or equal to 3 is reserved for future use. When sps_default_qpc_idc is not present, it is inferred to be equal to 0.

sps_separate_qpc_table_flag equal to 1 specifies that two separate QpC tables are used for Cb and Cr samples. sps_separate_qpc_table_enable_flag equal to 0 specifies that one QpC table is used for Cb and Cr samples. When sps_separate_qpc_table_flag is not present, it is inferred to be equal to 0.

The variable QpCb[ i ], with i = 0..69 refers to the QpC table used for Cb samples. The variable QpCr[ i ], with i = 0..69 refers to the QpC table used for Cr samples. When sps_separate_qpc_table_flag equal to 0,  QpCr[ i ] = QpCb[ i ] with i = 0..69.

sps_qpc_cb_start_idx specifies the starting index from which the syntax array sps_qpc_cb_delta is signaled. The value of sps_qpc_cb_start_idx shall be in the range of 0 to 69, inclusive.

sps_qpc_cb_end_idx specifies the ending index to which the syntax array sps_qpc_cb_delta is signaled. The value of sps_qpc_cb_end_idx shall not be less than sps_qpc_cb_start_idx and shall be in the range of 0 to 69, inclusive.

sps_qpc_cb_delta[ i ] specifies the delta values between QpCb[ i ] and QpCb[i − 1] for Cb components. The value of sps_qpc_cb_delta[ i ] shall be in the range of 0 to 1.

The variable QpCb[ i ], with i = 0..69, is derived as follows:

- For i = 0..sps_qpc_cb_start_idx, QpCb [ i ] is set equal i.

- For i = sps_qpc_cb_start_idx + 1..sps_qpc_cb_end_idx, the following applies:

QpCb[ i ] = QpCb[i − 1]+ sps_qpc_cb_delta[ i ]

- For i = sps_qpc_cb_end_idx + 1..69, the foolowing applies:

deltaEnd = sps_qpc_cb_end_idx− QpCb[sps_qpc_cb_end_idx]

QpCb[ i ] =i − deltaEnd

The syntax elements for the Cr component, sps_qpc_cr_start_idx, sps_qpc_cr_end_idx, and sps_qpc_cr_delta[ i ], have the same semantics as the Cb component.

## Default QpC tables

### SDR

For SDR signals, this contribution proposes the default SDR QpC table in the current VVC specification, as shown in Table 2.

Table 2–- Specification of QpC as a function of qPi for ChromaArrayType equal to 1 when sps_default_qpc_idc = 0

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi–− 6 |

### HDR HLG

For HDR HLG signals, this contribution proposes the QpC table show in Table 3 and Figure 1. The QpC table values were derived based on chromaQPOffset as proposed in JVET-J0015 CfP [5], which uses a method similar to the chromaQPOffset derivation in the HDR PQ CTC anchor [6][7][8].

Subjective viewing confirmed that the proposed default HDR HLG QpC table reduces chroma patch artefacts at very low bitrate (QP=37) in achromatic regions, such as the white snow and the feathers in FlyingBirds. The results are crosschecked by InterDigital. The BDRate compared to VTM5.0 is shown in Table 4.

Table 3 -- Specification of QpC as a function of qPi for ChromaArrayType equal to 1 when sps_default_qpc_idc = 2

| qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 34 | 35 | 35 | 36 | 36 | 36 | 37 | 37 |

| qPi | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | >62 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | 38 | 39 | 39 | 40 | 41 | 42 | 43 | 43 | 44 | 45 | 46 | 47 | 47 | 48 | 49 | 50 | 51 | 51 |

Figure 1 -- Relationship between qPi and derived QpC (YCbCr case)

Table 4 – BD rate for AI and RA for HLG CTC

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Over vtm5.0 HLG |  |  |
|  | Y | U | V |
| Class H (HLG) | 0.13% | -1.26% | -1.21% |
|  | Random Access Main 10 |  |  |
|  | Over vtm5.0 HLG |  |  |
|  | Y | U | V |
| Class H (HLG) | 0.09% | -0.86% | -0.56% |

### HDR PQ

For HDR PQ signals, this contribution proposes the QpC table proposed in JVET-N0221, as shown in Table 5.  The proposed default HDR PQ QpC was subjectively crosschecked by Technicolor [9]

Table 5 -- Specification of QpC as a function of qPi for ChromaArrayType equal to 1 when sps_default_qpc_idc= 1

| qPi | < 21 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | = qPi | 20 | 21 | 21 | 22 | 22 | 22 | 23 | 23 | 23 | 24 | 24 | 24 | 25 | 25 | 25 | 26 | 26 | 27 |

| qPi | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | > 55 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QpC | 27 | 28 | 29 | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 18 |

# Conclusion

This contribution proposes syntax and semantics for signalling chroma quantization parameter QpC tables. This proposal provides additional flexibility to optimize encoding while also supporting low bitrate overhead options and no additional complexity within the prediction, transform, and quantization loops.

# Acknowledgements

We would like to thank InterDigital for the crosscheck.

# References

- T. Lu, F. Pu, P. Yin, S, McCarthy, W. Husak, T. Chen, “Chroma Quantization Parameter QpC Table for HDR Signal”, JVET-N0221, Geneva, CH, Mar. 2019

- D. Baylon, Z. Gu, A. Luthra, K. Minoo, P. Yin, F. Pu, T. Lu, T. Chen, W. Husak, Y. He, L. Kerofsky, Y. Ye, B. Yi, “Response to Call for Evidence for HDR and WCG Video Coding: ARRIS, Dolby and InterDigital”, Doc. m36264, July 2015, Warsaw, Poland

- K.Andersson, P. Hermansson, J. Samuelsson, J. Strom, M. Petersson, “Report for CE1.a (Chroma QP)”, Doc. m37179, Oct. 2015, Geneva, Switzerland.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-N1001-v8, Geneva, CH, Mar. 2019.

- X. Xiu, P. Hanhart, R .Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, US, April, 2018.

- K.Andersson, P. Hermansson, J. Samuelsson, J. Strom, M. Petersson, “Report for CE1.a (Chroma QP)”, Doc. m37179, Oct. 2015, Geneva, Switzerland.

- A. Segall, E. François, S. Iwamura, D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-L1011, Macao, CN, 3–12 Oct. 2018.

- Supplement 15 to ITU-T H-series Recommendations: “Conversion and coding practices for HDR/WCG Y’CbCr 4:2:0 video with PQ transfer characteristics”

- F. Hiron, C. Chevance, “Crosscheck of JVET-N0221 (Chroma Quantization Parameter Qpc Table for HDR Signal) ”, JVET-N0793, Geneva, CH, Mar. 2019

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).