| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0435 |
| --- | --- |

| Title: | AHG17: on pic_struct |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Fangjun Pu, Taoran Lu, Peng Yin, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 940850 | Tel: Email: | {Sean.McCarthy,pyin}@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modifications to the existing VVC syntax and syntax values to enable repetition of an arbitrary number of frames. The existing VVC syntax element, pic_struct, is defined to indicate whether a decoded and displayed picture should be repeated, and also whether a picture should be displayed as a frame or as one or more fields.  In the current draft of the VVC draft specification, pic_struct supports only frame doubling and tripling, which limits its usefulness for display of video on high scan rate displays, such as 120 Hz displays. In this contribution, it is proposed to:

- add a new pic_struct value that indicates frame repetition; and

- add a new syntax element to indicate the number of frames to be repeated.

An alternative proposal is also described which would provide identical functionality, but instead of adding a new pict_struct value, the existing two pic_struct values that indicate frame doubling and tripling would be replaced by one pic_struct value that indicates frame repetition.

# Introduction

In the current VVC draft specification [1], pic_struct is used to indicate whether a picture should be displayed as a frame or as one or more fields and whether a decoded picture should be displayed two times, frame doubling, or three times, frame tripling. Frame doubling can be used to facilitate the display, for example, of 25 Hz progressive-scan video on a 50 Hz progressive-scan display or 30 Hz progressive-scan video on a 60 Hz progressive-scan display. Using frame doubling and frame tripling in alternating combination on every other frame can be used to facilitate the display of 24 Hz progressive-scan video on a 60 Hz progressive-scan display. The interpretation of pic_struct is specified in Table D.2, which is copied from the current VVC draft specification.

The problem is that the current pic_struct semantics cannot support use cases such as the display of 24 Hz or 30 Hz progressive-scan video on a 120 Hz progressive scan display.  Instead of simply adding frame quadrupling, quintupling, sextupling, etc. to support the numerous various combinations of video and display scan rates that can be anticipated, it is proposed that it would be simpler and more flexible to explicitly signal the number of pictures to be repeated.

The primary proposal and an alternative proposal described in this contribution provide identical functionality.

Table D.2 – Interpretation of pic_struct

| Value | Indicated display of picture | Restrictions |
| --- | --- | --- |
| 0 | (progressive) Frame | field_seq_flag shall be equal to 0 |
| 1 | Top field | field_seq_flag shall be equal to 1 |
| 2 | Bottom field | field_seq_flag shall be equal to 1 |
| 3 | Top field, bottom field, in that order | field_seq_flag shall be equal to 0 |
| 4 | Bottom field, top field, in that order | field_seq_flag shall be equal to 0 |
| 5 | Top field, bottom field, top field repeated, in that order | field_seq_flag shall be equal to 0 |
| 6 | Bottom field, top field, bottom field repeated, in that order | field_seq_flag shall be equal to 0 |
| 7 | Frame doubling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 8 | Frame tripling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 9 | Top field paired with previous bottom field in output order | field_seq_flag shall be equal to 1 |
| 10 | Bottom field paired with previous top field in output order | field_seq_flag shall be equal to 1 |
| 11 | Top field paired with next bottom field in output order | field_seq_flag shall be equal to 1 |
| 12 | Bottom field paired with next top field in output order | field_seq_flag shall be equal to 1 |

# Proposal

## Primary Proposal – add a new pic_struct value

In this contribution, it is proposed to add a new pic_struct interpretation value, pic_struct equal to 13, to Table D.2 in the VVC draft specification to indicate frame repetition greater than tripling. It is also proposed to add a new syntax element, num_frame_repetition_minus3, to indicate the number of frames to be repeated, as shown in Table 2.

Table 2

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| if( pic_struct == 13) | u(4) |
| num_frame_repetition_minus3 | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |

| Value | Indicated display of picture | Restrictions |
| --- | --- | --- |
| 0 | (progressive) Frame | field_seq_flag shall be equal to 0 |
| 1 | Top field | field_seq_flag shall be equal to 1 |
| 2 | Bottom field | field_seq_flag shall be equal to 1 |
| 3 | Top field, bottom field, in that order | field_seq_flag shall be equal to 0 |
| 4 | Bottom field, top field, in that order | field_seq_flag shall be equal to 0 |
| 5 | Top field, bottom field, top field repeated, in that order | field_seq_flag shall be equal to 0 |
| 6 | Bottom field, top field, bottom field repeated, in that order | field_seq_flag shall be equal to 0 |
| 7 | Frame doubling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 8 | Frame tripling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 9 | Top field paired with previous bottom field in output order | field_seq_flag shall be equal to 1 |
| 10 | Bottom field paired with previous top field in output order | field_seq_flag shall be equal to 1 |
| 11 | Top field paired with next bottom field in output order | field_seq_flag shall be equal to 1 |
| 12 | Bottom field paired with next top field in output order | field_seq_flag shall be equal to 1 |
| 13 | Frame repetition extension | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |

num_frame_repetition_minus3 plus 4 indicates that when fixed_pic_rate_within_cvs_flag is equal to 1, the frame should be displayed num_frame_repetition_minus3 plus 4 times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-73.

## Alternative Proposal – replace and re-use existing pic_struct values

Alternatively, it is proposed to:

- use existing pic_struct interpretation value, pic_struct equal to 7, to indicate frame repetition rather than frame doubling, which is the interpretation in the current VVC draft specification;

- use existing pic_struct interpretation values, pic_struct equal to 8, 9, 10, 11, to indicate the field pairing configurations that are currently indicated by pic_struct equal to 9, 10, 11, and 12, in the current VVC draft specification; and

- add a new syntax element, num_frame_repetition_minus1, to indicate the number of frames to be repeated, as shown in Table 3.

Table 3 – Alternative proposal

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| if( pic_struct == 7) | u(4) |
| num_frame_repetition_minus1 | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |

| Value | Indicated display of picture | Restrictions |
| --- | --- | --- |
| 0 | (progressive) Frame | field_seq_flag shall be equal to 0 |
| 1 | Top field | field_seq_flag shall be equal to 1 |
| 2 | Bottom field | field_seq_flag shall be equal to 1 |
| 3 | Top field, bottom field, in that order | field_seq_flag shall be equal to 0 |
| 4 | Bottom field, top field, in that order | field_seq_flag shall be equal to 0 |
| 5 | Top field, bottom field, top field repeated, in that order | field_seq_flag shall be equal to 0 |
| 6 | Bottom field, top field, bottom field repeated, in that order | field_seq_flag shall be equal to 0 |
| 7 | Frame repetition | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 8 | Top field paired with previous bottom field in output order | field_seq_flag shall be equal to 1 |
| 9 | Bottom field paired with previous top field in output order | field_seq_flag shall be equal to 1 |
| 10 | Top field paired with next bottom field in output order | field_seq_flag shall be equal to 1 |
| 11 | Bottom field paired with next top field in output order | field_seq_flag shall be equal to 1 |

num_frame_repetition_minus1 plus 2 indicates that when fixed_pic_rate_within_cvs_flag is equal to 1, the frame should be displayed num_frame_repetition_minus1 plus 2 times consectutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-73.

# Conclusion

This contribution proposes modifications to existing VVC syntax and syntax values to enable repetition of an arbitrary number of frames. The modifications facilitate display of video in cases in which the video and display have different scan rates.  The modifications support a wider range of video and display scan rates than are supported in the current VVC draft specification.

It is recommended that the primary proposal of adding a new pic_struct interpretation value and new syntax element to indicate the number of frames to be repeated be adopted in VVC.  The primary proposal has the advantage that interpretation of pic_struct would be consistent for VVC, HEVC, and AVC, which would reduce the potential for error when transcoding.

Alternatively, it is recommended that the proposal to replace and re-use existing pic_struct interpretation values be adopted in VVC.  The alternative proposal has the advantage that frame doubling and tripling would not be treated as special cases of frame repetition. It also has the advantage that pic_struct equal to 12 would be available for future use.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6)”, JVET-N1001-v8, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).