| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0437-v1 |
| --- | --- |

| Title: | Non-CE5: Unification of non-linear ALF luma/chroma clipping parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution presents a unification method of luma and chroma clipping parameters for nonlinear ALF in VTM-5.0. In VVC-5.0, there are different clipping parameters for luma and chroma component. This contribution proposes to use the same clipping parameters for luma and chroma. For Y component, the proposed method shows average BD-rates of 0.00%, -0.xx%, and 0.xx% for AI, RA, and LDB configurations, respectively. For U component, the proposed method shows average BD-rates of -0.02%, -0.xx%, and 0.xx% for AI, RA, and LDB configurations, respectively. For V component, the proposed method shows average BD-rates of -0.02%, -0.xx%, and 0.xx% for AI, RA, and LDB configurations, respectively.

Introduction

In VVC5.0 [1], ALF parameters are signalled in adaptation parameter set (APS) [2]. In one APS, up to 25 sets of luma filter coefficients and clipping value indices, and up to 1 set of chroma filter coefficients and clipping value indices are signalled for non-linear ALF [3]. The value of filter coefficients is in the range of -128 to 127. And the value of clipping value index is in the range of 0 to 3.

There are two sets of clipping parameters in current nonlinear ALF. For luma component, the clipping set is {1024, 181, 32, 6}. For chroma component, the clipping set is {1024, 161, 25, 4}. The clipping parameter sets AlfClip are calculated in encoder and decoder side, by the following Equal,

For luma component,

AlfClipL [j]= Round( 2( BitDepthY * ( 4 − i ) / 4 ) )

For chroma component,

AlfClipC [j]= Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − j ) / 3 ) )

Where, j (0~3) is the clipping value index.

Proposed Method

There is no need to use separate clipping parameters for luma and chroma. This proposal proposes to use {1024, 181, 32, 6} for both luma and chroma.

Proposed draft text changes

The following shows the proposed draft text changes based on Draft 5 v8 when using the proposed method:

- 7.4.6.3Adaptive loop filter data semantics

……

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

The chroma filter clipping values AlfClipC[ adaptation_parameter_set_id ] with elements AlfClipC[ adaptation_parameter_set_id ][ j ], with j = 0..5 are derived as follows:

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2( BitDepthY * ( 4 − alf_chroma_clip_idx [ j ] ) / 4 ) )	         (7-87)

AlfClipC[ adaptation_parameter_set_id ][ j ] = Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − alf_chroma_clip_idx[ j ] ) / 3 ) )	(7-87)

……

Simulation results

The experiments were performed based on VTM-5.0 software [3] with VTM configuration files under JVET common test condition [4]. The experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed methods compared to VTM-5.0 anchor are summarized in Table 1. It can be noted that the proposed methods can unify the ALF luma and chroma design and simplify the derivation process of ALF clipping parameter while improve the coding efficiency in AI and RA configurations.

Table 1. Experimental result of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.05% | 101% | 101% |
| Class A2 | 0.00% | -0.04% | 0.00% | 99% | 102% |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 98% |
| Class C | 0.00% | -0.02% | -0.04% | 100% | 103% |
| Class E | 0.00% | -0.02% | 0.00% | 100% | 98% |
| Overall | 0.00% | -0.02% | -0.02% | 100% | 100% |
| Class D | 0.00% | -0.01% | -0.03% | 99% | 102% |
| Class F | 0.00% | 0.00% | 0.04% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.13% | 100% | 102% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.02% | 0.01% | 0.01% | 99% | 100% |
| Class C | -0.01% | -0.07% | -0.12% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.22% | 0.33% | 98% | 98% |
| Class F | -0.01% | 0.09% | 0.02% | 94% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.02% | 0.11% | 0.12% | 101% | 107% |
| Class E | 0.00% | 0.56% | -0.56% | 85% | 84% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.02% | -0.48% | 0.89% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

In this contribution, a unification method of luma and chroma clipping parameters for nonlinear ALF in VTM-5.0 is proposed. For Y component, the proposed method shows average BD-rates of 0.00%, -0.xx%, and 0.xx% for AI, RA, and LDB configurations, respectively. For U component, the proposed method shows average BD-rates of -0.02%, -0.xx%, and 0.xx% for AI, RA, and LDB configurations, respectively. For V component, the proposed method shows average BD-rates of -0.02%, -0.xx%, and 0.xx% for AI, RA, and LDB configurations, respectively. This contribution can unify the ALF luma and chroma design and simplify the derivation process of ALF clipping parameter while improving the coding efficiency in AI and RA configurations. Therefore, it is recommended to adopt this proposal to unify the current luma and chroma design in nonlinear ALF.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- J. Taquet, P. Onno, C. Gisquet, G. Laroche, “CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0242, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).