| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0440-v2 |
| --- | --- |

| Title: | Non-CE4: A modified method to derive the temporal merge candidate for long-term reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geon-Won Lee Geun-Bae Kim Sang-Guk Cha Da-Yun Nam Jong-Ki Han | Tel: Email: | +82-2-3408-3739 geonwonlee@sju.ac.kr gbkim@sju.ac.kr sgsept29@sju.ac.kr ndy9709@sju.ac.kr hjk@sejong.edu |
| Source: | Sejong University |  |  |

_____________________________

- Abstract

This contribution proposes a modified method to set the target picture when the long-term reference picture is considered. In the merge mode of VTM 5.0, because the target picture is set to the picture whose index is 0 in the reference picture list, if the type of the picture of index 0 is different from that of the reference picture of the co-located picture, the temporal merge candidate is not used. In this contribution, when the co-located block refers to the long-term reference picture, the target picture is set to a picture in the reference picture list whose type is long-term, even if the index of the picture is not 0. The proposed method can make a temporal merge candidate when the type of the reference picture having index 0 is short-term while that of the reference picture of the co-located picture is long-term.

Introduction

In the merge mode of VTM 5.0 [1] and VVC Draft 5 [2], when the co-located picture refers to the long-term reference picture, if the type of the target picture indicated by index 0 in the reference picture list is short-term, then TMVP is not used. Else if the type of the picture having index 0 is the long-term, MV of the co-located block is used as one (TMVP) of the merge candidates without scaling. Until the 14th JVET meeting, because CTC does not consider the long-term picture type of the reference pictures, realistic cases have not been considered sufficiently. This contribution proposes a modified method to make the temporal merge candidate when the long-term reference pictures are used.

Proposed method

In this contribution, when the co-located picture refers to the long-term reference picture, the long-term type picture is set to the target picture, if any long-term type picture is included in the reference picture list of the current picture.

Figure 1 shows an example of setting the target picture to make the temporal merge candidate in the proposed method, where the collocated block (T0) refers to a picture (POC 45) whose type is long-term. In VTM 5.0, TMVP is not used as a merge candidate, because the type of the target picture (POC 98, indicated by index 0) is short-term. In the proposed method, the target picture is set to a picture (POC 0, indicated by index 3), because the type of the picture is long-term. In Figure 1, ColMV is used as the temporal merge candidate for the target picture (POC 0) without scaling. If there are many long-term pictures in the reference picture list, the picture indicated by the smallest index number is used as the target picture.

Figure 1. An example of selecting the target picture to make the temporal merge candidate when long term reference picture is considered.

Conclusion

This contribution proposes a modified method to set the target picture when the long-term reference picture is considered. Because CTC [3] does not consider the long-term picture type, we don’t provide the evaluation results of the proposed method under CTC. If we use a realistic configuration for the simulation where the long-term reference pictures are used, we can expect that the proposed method increases the coding efficiency significantly, because many long-term reference pictures are used more frequently in specific applications.

References

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 5)”, Document of Joint Video Experts Team (JVET), JVET-N1001-v8, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).