| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0441-v2 |
| --- | --- |

| Title: | Non-CE4: Short-tap interpolation filter for bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Xiaozhen Zheng, Shanshe Wang, Siwei Ma | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd., |  |  |

_____________________________

- Abstract

This contribution presents a short-tap interpolation based bi-prediction to reduce the memory bandwidth consumption of conventional LDB configuration compared to LDP configuration. This proposal proposes to use 4-tap interpolation filter for small blocks of normal bi-prediction and affine bi-prediction. Compare to conventional LDP configuration, the proposed method can achieve 5.83% coding gain with almost the same memory bandwidth consumption.

Introduction

Motion compensation (MC) is one of the largest memory bandwidth consumers in decoder side. Providing a reasonable limit on MC bandwidth requirement is important for the implementations of video coding standard.

Memory bandwidth requirement for MC is usually determined by motion compensation block size, the type of prediction (e.g. uni-directional or bi-directional) and the tap of interpolation filter. In the current VVC (VTM-5.0), the 4x4 inter CUs and the 4x8/8x4 bi-prediction are disallowed. As shown in Table1 and Table 2, the worst case memory bandwidth is calculated according to the method in [2]. The worst case memory bandwidth is caused by 4x16/16x4 bi-prediction CUs. As for affine mode, because of the 6-tap luma interpolation filter and 8x8 affine block MV spread limitation, the worst case is the same as normal inter mode.

Table 1. Memory bandwidth consumption (per-pixel) of normal inter mode

| Block Size | Prediction mode | Luma (8 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- |
| 4x8/8x4 | Uni | 5.16 | 2.18 | 7.34 |
| 4x16/16x4 | Bi/Triangle | 7.9 | 3.44 | 11.34 |
| 8x8 | Bi/Triangle | 7.03 | 3.06 | 10.09 |
| 4x32/32x4 | Bi/Triangle | 6.70 | 2.97 | 9.67 |
| 8x16/16x8 | Bi/Triangle | 5.39 | 2.41 | 7.80 |
| 4x64/64x4 | Bi/Triangle | 6.10 | 2.73 | 8.84 |
| 4x128/128x4 | Bi/Triangle | 5.80 | 2.62 | 8.42 |

Table 2. Memory bandwidth consumption (per-pixel) of Affine mode

| Block Size | Prediction mode | δx | δy | Thred4 | Luma (6 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MxN | Affine Uni | 2 | 2 | 15x11 | 5.06 | 2.18 | 7.24 |
| MxN | Affine Bi | 2 | 2 | 15x15 | 7.03 | 3.06 | 10.09 |

Proposed Method

In order to achieve better coding performance as well as keep almost same memory bandwidth consumption as low delay P configuration, this contribution propose to use shorter tap interpolation filter for some small block of bi-prediction.

For normal bi-prediction and triangle prediction, we propose to use 4 tap chroma interpolation filter for luma component with 4x16/16x4/8x8/4x32/32x4/8x16/16x8/4x64/64x4/4x128/12x8 block sizes (width*height<=128 or width=4 or height=4).

For Affine bi-prediction, we propose to use 4 tap chroma interpolation filter for Luma component while reduce the threshold of affine MV spread limitation.

The memory bandwidth consumption for the proposed method is summarized in Table 3 and 4. It can be noted that the bandwidth consumption (peri-pixel) for the proposed short-tap lowdelay B is almost same as the bandwidth consumption (peri-pixel) for lowdelay P configuration (7.59 / per-pixel vs. 7.34 / per-pixel).

Table 3. Memory bandwidth consumption (per-pixel) of normal inter mode (Proposed method)

| Block Size | Prediction mode | Luma (4 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- |
| 4x16/16x4 | Bi/Triangle | 4.15 | 3.44 | 7.59 |
| 8x8 | Bi/Triangle | 3.78 | 3.06 | 6.84 |
| 4x32/32x4 | Bi/Triangle | 3.83 | 2.97 | 6.80 |
| 8x16/16x8 | Bi/Triangle | 3.27 | 2.41 | 5.67 |
| 4x64/64x4 | Bi/Triangle | 3.66 | 2.73 | 6.40 |
| 4x128/128x4 | Bi/Triangle | 3.58 | 2.62 | 6.20 |

Table 4. Memory bandwidth consumption (per-pixel) of Affine mode (Proposed method)

| Block Size | Prediction mode | δx | δy | Thred4 | Luma (4 tap) | Chroma (4 tap) | All |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MxN | Affine Bi | 0 | 1 | 11x12 | 4.13 | 3.06 | 7.19 |

In order to evaluate computation complexity for 8 tap uni-prediction and 4 tap bi-prediction at decoder side, the contribution also summarise the number of multiplication, addition and shift operation for 8 tap interpolation for uni-prediction and 4 tap interpolation for bi-prediction in Table 5. Please note that the data in Table 5 also includes the number for U and V chroma component. The statistics results in Table 5 show that 8 tap uni-prediction and 4 tap bi-prediction have almost same computation complexity when we consider the worst case at decoder side.

Table 5. Computation complexity (per-pixel) for 8 tap-uni and 4 tap-bi

| Block Size | 8 tap uni |  |  | 4 tap bi |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mult. | Add. | Shift | Mult. | Add. | Shift |
| 8x4 | 24.67 | 24.67 | 3.67 | N/A | N/A | N/A |
| 4x8 | 19.00 | 19.00 | 2.83 | N/A | N/A | N/A |
| 16x4 | 24.67 | 24.67 | 3.67 | 24.00 | 25.00 | 7.00 |
| 4x16 | 16.17 | 16.17 | 2.42 | 18.00 | 19.00 | 5.50 |
| 8x8 | 19.00 | 19.00 | 2.83 | 20.00 | 21.00 | 6.00 |
| 32x4 | 24.67 | 24.67 | 3.67 | 24.00 | 25.00 | 7.00 |
| 4x32 | 14.75 | 14.75 | 2.21 | 17.00 | 18.00 | 5.25 |
| 16x8 | 19.00 | 19.00 | 2.83 | 20.00 | 21.00 | 6.00 |
| 8x16 | 16.17 | 16.17 | 2.42 | 18.00 | 19.00 | 5.50 |
| 64x4 | 24.67 | 24.67 | 3.67 | 24.00 | 25.00 | 7.00 |
| 4x64 | 14.04 | 14.04 | 2.10 | 16.50 | 17.50 | 5.13 |
| 128x4 | 24.67 | 24.67 | 3.67 | 24.00 | 25.00 | 7.00 |
| 4x128 | 13.69 | 13.69 | 2.05 | 16.25 | 17.25 | 5.06 |

Simulation results

The experiments were performed based on VTM-5.0 software [3] with VTM configuration files under JVET common test condition [4]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method compared to VTM-5.0 anchor are summarized in Table 5. The average BD-rates of 0.22%, and 0.xx% are observed for RA, and LDB cases, respectively. And the proposed short-tap LDB can get xxx% coding gain compared to traditional LDP configuration. It can be noted that the proposed method can provide much better performance than traditional LDP configuration without increase memory bandwidth consumption and computation complexity at decoder side.

Table 5. Experimental result of the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 RA |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.49% | -0.29% | 99% | 101% |
| Class A2 | 0.13% | -0.16% | -0.16% | 99% | 100% |
| Class B | 0.22% | -0.06% | -0.21% | 98% | 98% |
| Class C | 0.52% | 0.21% | 0.02% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.22% | -0.09% | -0.16% | 98% | 100% |
| Class D | 1.02% | 0.69% | 0.93% | 97% | 95% |
| Class F | 0.17% | 0.21% | 0.16% | 93% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 LDB |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.30% | 0.42% | 0.36% | 98% | 100% |
| Class C | 0.91% | 0.71% | 0.88% | 100% | 99% |
| Class E | 0.35% | -0.04% | -0.44% | 85% | 86% |
| Overall | 0.52% | 0.40% | 0.33% | 95% | 99% |
| Class D | 2.51% | 2.17% | 3.03% | 99% | 97% |
| Class F | -0.06% | 0.07% | -0.35% | 84% | 85% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-5.0 LDP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -7.21% | -9.33% | -9.28% | 136% | 100% |
| Class C | -4.32% | -5.76% | -5.89% | 133% | 100% |
| Class E | -5.53% | -5.62% | -6.30% | 147% | 102% |
| Overall | -5.83% | -7.21% | -7.41% | 138% | 99% |
| Class D | -3.25% | -4.03% | -4.23% | 129% | 101% |
| Class F | -4.25% | -4.31% | -5.37% | 127% | 103% |

Conclusions

In this contribution, shorter tap interpolation filter for small blocks of normal bi-prediction and affine bi-prediction is proposed. Compare to conventional LDP configuration, the proposed method can achieve xxx% coding gain with almost the same memory bandwidth consumption. Therefore, we propose to add a new short-tap B frame type to current VVC working draft by using 4-tap chroma filter for luma component’s interpolation.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- Y. He, “BoG report on CE2 related contributions”, JVET-M0862, Jan. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).