| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0443 |
| --- | --- |

| Title: | CE8-related: Flipping of reference blocks for Intra Block Copy (IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anubhav Singh Akhil Konda | Tel: Email: | anubhav.23@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

In the VVC pipeline [1], Intra Block Copy (IBC) gives a considerable gain for Class F and Class SCC sequences. IBC, uses references from the current picture for prediction of a particular block. The current implementation of IBC uses matching of blocks in their original orientation, which sometimes may not result in optimum match. There are some patterns and shapes (especially in SCC/ Class F) which differ only in orientation. To efficiently match these shapes, this contribution proposes to use original reference blocks as well as three flipped versions of the blocks for matching, viz. flipping along vertical axis, along horizontal axis and along both the axes. Furthermore the contribution also proposes an efficient signalling of orientation. The Luma gains observed in class F and Class SCC sequences are -1.13% and -1.41% respectively for AI.

# Introduction

IBC uses current picture itself for finding prediction reference for a given block [2]. Due to the repetitive pattern and shapes in Class F and Class SCC sequences (Screen Content), IBC provides a considerable gain in these sequences. Also the pattern and shapes in screen content may occur as mirror image of each other at a block level, as shown in Fig. 1. It shows the flipped version being a better reference for the current block.

Figure 1. Example from SlideShow_1280x720_20.yuv

Current IBC pipeline does not consider these variations in shapes and patterns. It was observed in our experiments, that ~30-40% of the blocks match better with one of the flipped version of reference blocks.

It was also observed that some blocks may find a rotated version of reference more suitable, but due to the complexity of rotation operation (application of filters) it may add considerable encoding and decoding complexity [3].

# Proposed Method

In the proposed method, instead of using only the original blocks as prediction references, three extra versions of the reference block are introduced. Total four orientations are evaluated for each block, to find the best match. The blocks are rotated based on either horizontal or vertical axis as shown in Fig. 2.

- Original reference block

- Flipped along horizontal axis

- Flipped along vertical axis

- Flipped along both axes (equivalent to 180 degree rotation)

Figure 2. Flipping process for reference block (a) Original block (b) Flipped along horizontal axis (c) Flipped along vertical axis and (d) Flipped along both axes

For any given block, while calculating the cost of IBC mode, all 4 versions are evaluated for the best match. It has been observed that ~30-40% of the times the best match is one of the flipped version [3]. As Screen content, generally has fixed and rigid shapes, getting a match in one of these becomes more probable as can be seen from Fig. 3.

Figure 3. Flipped reference cropped from SlideShow_1280x720_20.yuv along with original

Furthermore, flipping will require signalling at CU level. This proposal also provides a method to encode flags efficiently.

# Results

Table 1 shows the simulations results of the proposed method over VTM 5.0. The results show performance for Class F and Class SCC sequences in All Intra mode only. The rest of the results are awaited. In the current implementation, the flag is encoded using fixed length binarization and without using any context bins.

Table 1. Simulation results with proposed method for AI.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F | -1.13% | -1.21% | -1.27% | 110% | 101% |
| Class SCC | -1.41% | -1.36% | -1.29% | 108% | 106% |

Table 2. Simulation results with proposed method for RA.

|  |  |  | Flipping without signalling |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
| Class SCC |  |  |  |  |  |

The results with All the Classes and RA are running. Shall update in next version

# Conclusion

The document proposes a method to improve the coding of Screen content by making IBC more efficient. It exploits one of the basic characteristics of screen content, i.e. that the blocks are rigid and have repetitive pattern which may be different only in orientation. Along with the signalling for the orientation flag this contribution achieves a gain of -1.13% and -1.41% for Class F and Class SCC respectively in AI, without adding significant complexity in decoder.

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001, B. Bross, J. Chen, S. Liu

- “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

- Z. Zhang, V. Sze, "Rotate intra block copy for still image coding", Proc. IEEE Int. Conf. Image Process. (ICIP), pp. 4102-4106, Sep. 2015.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).