| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0444 |
| --- | --- |

| Title: | Non-CE6: On LFNST index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Jacob Ström, Per Wennersten Torshamnsgatan 23 16480 Stockholm, Sweden | Tel: Email: | +46 10 714 64 37 {christopher.hollmann, davood.saffar, per.wennersten, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to modify the coding of the LFNST index, so that it no longer is part of the coding unit syntax, but part of the transform unit syntax. It is claimed that this change increases readability of the standard while having no impact on compression performance.

# Problem Statement

In the current version 5 of the Versatile Video Coding (VVC) draft the index for the low-frequency non-separable transform (LFNST) is coded on a coding unit level. However, as the LFNST is performed on a transform unit level, it is proposed to change the coding of the LFNST, so it is done on a transform unit level.

This suggested change does not change how the LFNST index is coded and thus does not have any impact on compression efficiency but can increase the logical structure of the specification.

Simulations for AI and RA showed that the resulting bit streams are bit-exact to VTM-5.0 CTC bit streams. LD was not tested, as LFNST is not part of the CTC for LD. A patch containing the software changes is attached to the contribution.

# Conclusion

It is proposed to adopt this proposal into the next version of the VVC draft.

# Changes to the specification

Changes can be seen in the attached file (see tracked changes). Changes were made in four places:

- 7.3.7.5		Coding unit syntax

- 7.3.7.10		Transform unit syntax

- 7.4.8.5		Coding unit semantics

- 7.4.8.10		Transform unit semantics

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.