| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0445 |
| --- | --- |

| Title: | [AHG17] Signaling of uniform virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amith Dsouza Chirag Pujara Raj Narayana Gadde Woong-Il Choi Kwang Pyo Choi | Tel: Email: | amith.ds@samsung.com chirag.p@samsung.com raj.gadde@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This document proposes method to reduce the signaling overhead for virtual boundaries as has been adopted in the VVC Standard. It is asserted that the current method of signaling of virtual boundaries results in higher bit requirements. In this document, a method to signal uniform virtual boundaries is described. This method is especially useful for omnidirectional videos, wherein the virtual boundaries will be uniformly distributed. Bits required to signal the virtual boundaries is significantly reduced when the proposed method is used.

# Introduction

Signaling of virtual boundary from JVET-N0438 [1] was adopted to reduce visual artefacts at synthetic boundaries. One of the use cases for virtual boundaries is 360° video. For a 360° video, the projection format may have one or more discontinuous edges. Application of loop-filters across these discontinuous edges result in visible artefacts in the rendered viewports. In-loop filters can be disabled across the signalled virtual boundaries. The virtual boundaries are currently signalled in VVC [2] as absolute distance from the top-left of the picture in units of 8 samples. For most of the projections with multiple faces, for example Cubemap based projection formats, virtual boundaries are generally present at uniform distance.

# Uniform Virtual Boundaries

One of the common projection formats used to represent 360° videos is cube map based projection formats in 3x2 layout. In this arrangement of faces only one synthetic boundary is present at the centre of the frame. Using the current method to signal the virtual boundary would require signaling of the offset of the virtual boundary offset with respect to the top of the frame. The number of bits required for signaling this virtual boundary as per the current VVC draft is equal to Ceil( Log2( pic_height_in_luma_samples ) − 3 ). In case of cube map with face size equal to 1024 samples arranged in a 3x2 layout, the number of bits required to signal a single virtual boundary would be 12 bits. The worst case bit requirements for this method in case of three vertical and three horizontal boundaries in the above example would be 55 bits (2 + 8x3 + 2 + 9x3).  An alternative method of signaling the virtual boundaries when the virtual boundaries are uniformly spaced is given below. Using the below syntax, the number of bits require for signaling the virtual boundaries would be only 6 bits and would be independent of the number of such boundaries(constrained to 3 vertical and 3 horizontal boundaries). The worst case bit requirement for the alternate signaling would still be 6 bits only.

## Syntax

The modified syntax is highlighted in Yellow below.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| pps_uniform_ver_virtual_boundaries | u(1) |
| if(!pps_uniform_ver_virtual_boundaries) { |  |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) { |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| } |  |
| } |  |
| pps_num_hor_virtual_boundaries | u(2) |
| pps_uniform_hor_virtual_boundaries | u(1) |
| if(!pps_uniform_hor_virtual_boundaries) { |  |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) { |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Semantics

…

pps_uniform_ver_virtual_boundaries equal to 1 specifies that the vertical virtual boundaries are distributed uniformly across the picture. pps_uniform_ver_virtual_boundaries equal to 0 specifies that the vertical virtual boundaries may or may not be uniformly distributed across the picture and the explicit signaling of the virtual boundary location follows.

If pps_uniform_ver_virtual_boundaries equal to 1, the location of the vertical virtual boundary PpsVirtualBoundariesPosX[ i ] is derived as follows:

for (i=0;i< pps_num_ver_virtual_boundaries ;i++)

PpsVirtualBoundariesPosX[i] = (i+1) * (((pic_width_in_luma_samples >> 3)/(pps_num_ver_virtual_boundaries+1))<< 3)

Otherwise,

PpsVirtualBoundariesPosX[ i ] = pps_virtual_boundaries_pos_x[ i ] * 8

pps_uniform_hor_virtual_boundaries equal to 1 specifies that the horizontal virtual boundaries are distributed uniformly across the picture. pps_uniform_hor_virtual_boundaries equal to 0 specifies that the horizontal virtual boundaries may or may not be uniformly distributed across the picture and the explicit signaling of the virtual boundary location follows.

If pps_uniform_hor_virtual_boundaries equal to 1, the location of the horizontal virtual boundary PpsHorinzontalBoundariesPosY[ i ] is derived as follows:

for (i=0;i< num_uni_hor_virtual_boundaries;i++)

PpsVirtualBoundariesPosY[i] = (i+1) * (((pic_height_in_luma_samples >> 3)/(pps_num_hor_virtual_boundaries+1))<< 3)

Otherwise

PpsVirtualBoundariesPosY[ i ] = pps_virtual_boundaries_pos_y[ i ] * 8

…

# Conclusion

This document presents a method to signal uniform virtual boundaries in the frame. Using the proposed method of signalling the virtual boundaries would significantly reduce the number of bits required. It is recommended to adopt this signaling method for virtual boundaries in VVC.

# References

- “AHG12: Loop filter disabled across virtual boundaries,” JVET-N0438, S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v9, B. Bross, J. Chen, and S. Liu

# Patent rights declaration(s)

Samsung Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).