| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0446-v3 |
| --- | --- |

| Title: | Non-CE6: Reduced MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Per Wennersten, Jacob Ström Torshamnsgatan 23 16480 Stockholm, Sweden | Tel: Email: | +46 10 714 64 37 {christopher.hollmann, davood.saffar, per.wennersten, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this document it is proposed to remove the DCT-8 from the MTS tool, leaving the encoder with two options for the primary transform: DCT-2 and DST-7. It is claimed that this can be done without any loss in compression efficiency. Test results are reported as -0.01% (AI-Y), -0.01% (RA-Y) and 0.06% (LD-Y) without a measurable difference in encoder and decoder run times for AI and RA, while the encoder run time for LD is reduced to 98%. As a consequence, it is claimed that DCT-8 of size 32 can be removed from the standard.

An additional test with MTS allowed for inter predicted blocks results reportedly in 0.11% (RA-Y) and 0.13% (LD-Y). In the additional test the encoder run times are reported to be 96% (RA) and 92% (LD).

In version 2 of this document complete results for the additional test with inter MTS as well as the necessary changes to the specification were added.

In version 3 of this document an additional aspect regarding Subblock Transform (SBT) was described, which does not affect test results.

# Introduction

This document proposes to remove the DCT-8 from the Multiple Transform Selection (MTS) tool. Currently, the MTS tool allows four different transform combinations:

- (DST-7, DST-7)

- (DCT-8, DST-7)

- (DST-7, DCT-8)

- (DCT-8, DCT-8)

This proposal reduces the number of allowed candidates to one: (DST-7, DST-7). This can be done without losing compression efficiency.

This technology has previously been proposed [1, 2] and tested in a CE [3]. However, with the inclusion of the Low Frequency Non-Separable Transform (LFNST) at the JVET meeting in March 2019 in Geneva the importance of having many different MTS modes has been reduced compared to previous versions of the draft standard.

In Table 1 the historical development of the performance of this modification is presented.

As a further aspect, it is proposed to restrict the usage of DCT-8 in the Subblock Transforms (SBT) tool to the maximum size of 16. The DCT-8 of size 32 is used in SBT for example if a 64x32 CU uses a vertical half-size SBT, resulting in a TB of 32x32 using DCT-8 as its horizontal transform if cu_sbt_pos_flag = 0. This is, however, a constellation that does not occur in the current CTC and thus has no impact on

test results.

Table 1 Performance development of reduced MTS

| Meeting | Document | BDR-Y AI | EncT AI | BDR-Y RA | EncT RA |
| --- | --- | --- | --- | --- | --- |
| Macao 2018 | JVET-L0489 [1] | 0.30% | 64% | 0.13% | 89% |
| Marrakech 2019 | JVET-M0304 [2] | 0.27% | 64% | 0.11% | 95% |
| Geneva 2019 | JVET-N0053 [3] | 0.20% | 75% | 0.18% | 97% |
| Gothenburg 2019 | JVET-O0446 | -0.01% | 100% | -0.01% | 100% |

# Test Results

Detailed results can be found in the accompanying Excel sheets.

In Table 2 a summary of performed tests is shown.

Table 2 Overview of tests

| Test | Anchor | BD-Rate |  |  | Encoder run time |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AI-Y | RA-Y | LD-Y | AI | RA | LD |
| M1: Reduced MTS | VTM-5.0 | -0.01% | -0.01% | 0.06% | 100% | 100% | 98% |
| M2: Reduced MTS + Inter MTS | VTM-5.0 + inter MTS |  | 0.11% | 0.13% |  | 96% | 92% |

## M1: Reduced MTS – Common Test Conditions

Results according to CTC [4].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.05% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | -0.08% | -0.02% | 100% | 101% |
| Class B | -0.01% | -0.06% | -0.06% | 100% | 99% |
| Class C | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class E | -0.02% | -0.01% | -0.01% | 99% | 100% |
| Overall | -0.01% | -0.03% | -0.02% | 100% | 100% |
| Class D | -0.02% | 0.09% | -0.06% | 100% | 100% |
| Class F | -0.02% | -0.02% | 0.07% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.21% | 0.10% | 100% | 95% |
| Class A2 | -0.01% | 0.08% | -0.04% | 100% | 98% |
| Class B | 0.00% | -0.02% | -0.17% | 100% | 100% |
| Class C | -0.02% | -0.33% | -0.15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.12% | -0.08% | 100% | 99% |
| Class D | -0.02% | -0.05% | 0.22% | 100% | 100% |
| Class F | 0.01% | 0.04% | -0.06% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.36% | -0.07% | 97% | 99% |
| Class C | 0.03% | 0.05% | -0.08% | 99% | 101% |
| Class E | 0.15% | 0.00% | 0.15% | 98% | 100% |
| Overall | 0.06% | 0.17% | -0.02% | 98% | 100% |
| Class D | 0.00% | 0.13% | 1.06% | 98% | 102% |
| Class F | -0.04% | -0.38% | -0.58% | 99% | 100% |

## M2: Reduced MTS with inter MTS enabled

In this test both anchor and the tested software use MTS for inter predicted blocks. This was enabled with the additional command line parameter --MTS=3. As this has no effect on the All Intra configuration, results are omitted.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 + inter MTS (MTS = 3) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.04% | 0.02% | 98% | 102% |
| Class A2 | 0.22% | 0.05% | -0.20% | 96% | 99% |
| Class B | 0.09% | -0.16% | -0.27% | 95% | 105% |
| Class C | 0.10% | -0.20% | 0.04% | 95% | 109% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | -0.11% | -0.12% | 96% | 104% |
| Class D | 0.00% | 0.04% | -0.17% | 95% | 116% |
| Class F | 0.02% | -0.04% | -0.09% | 99% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 + inter MTS (MTS = 3) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.23% | -0.27% | -0.28% | 90% | 101% |
| Class C | 0.12% | -0.33% | -0.57% | 88% | 104% |
| Class E | 0.00% | -0.59% | 0.95% | 100% | 107% |
| Overall | 0.13% | -0.37% | -0.07% | 92% | 103% |
| Class D | 0.15% | -0.91% | -0.16% | 89% | 101% |
| Class F | 0.02% | -0.15% | -1.06% | 94% | 97% |

# Changes to the specification

Red highlighted text is removed, green highlighted text is added.

In 8.7.4.1 General:

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32 and nTbW and nTbH are not both equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or both sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-965)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-966)

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-17 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-16 depending on tu_mts_idx[ xTbY ][ yTbY ].

- Otherwise, trTypeHor and trTypeVer are set to the value of tu_mts_idx[ xTbY ][ yTbY ].

Furthermore, table 8-16 and equations 8-989, 8-990, 8-991 and 8-992 can be removed

In 9.5.3.1 General:

In table 9-11:

| transform_unit( ) | tu_mts_idx[ ][ ] | TR FL | cMax = 4, cRiceParam = 0 cMax = 1 |
| --- | --- | --- | --- |

In 9.5.4.2.1 General:

In table 9-17:

| tu_mts_idx[ ][ ] | cqtDepth | 6 na | 7 na | 8 na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

# Conclusion

We propose to adopt the reduced MTS into the VVC draft and the VTM software.

# References

[1] C. Hollmann, P. Wennersten, J. Ström, R. Sjöberg. CE6-related: Transform Simplification, JVET-L0489, Macao, China, October 2018.

[2] J. Lainema. CE6-related: 2-mode MTS with shape adaptive transform selection, JVET-M0304, Marrakech, Morocco, January 2019.

[3] J. Lainema. CE6: 2-mode MTS (CE6-2.1), JVET-N0053, Geneva, Switzerland, March 2019.

[4] F. Bossen et al. JVET common test conditions and software reference configurations for SDR video, JVET-N1010, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).