| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0448 |
| --- | --- |

| Title: | Non-CE2: Inverse LUT carriage for LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Edouard François, Christophe Chevance, Franck Hiron, Rémi Jullian | Email: | pierre.andrivon@interdigital.com edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution relates to the luma mapping with chroma scaling (LMCS). LMCS involves at the decoder an inverse mapping as well as a chroma residual scaling. In addition, in inter slices a forward mapping is also applied to luma prediction samples. By design, in current VTM, the inverse mapping curve is non-uniformly sampled, while the forward mapping curve is. Consequently, the inverse mapping requires extra-processing such as a binary search algorithm also employed for chroma residual scaling. This contribution proposes to carry a uniformly sampled inverse mapping function. The forward mapping function is derived from carried inverse mapping function. It is asserted that the proposition simplifies the intra picture decoding, inverse mapping and chroma residual scaling processes. Performance for both SDR and HDR is reported to be comparable to the current LMCS design using a carried forward mapping function. Encoding and decoding times are also reportedly similar to VTM-5.0 anchor with LMCS enabled.

# Introduction

LMCS requires the application of in-loop forward (FwdMap) and inverse mapping/reshaping (InvMap) functions which are implemented in VTM5.0 as look-up tables (LUT).

JVET-M0427 [1] proposes a sketch illustrating the location of these mapping functions in the coding loop. The main difference between intra and inter slices is that in intra slices, only the inverse mapping is applied. The forward mapping only applies in inter slices to the luma prediction signal.

Figure 1. Usage of in-loop forward and inverse functions (JVET-M0427)

In VVC draft 5 (JVET-N1001-v4 [2]), the forward mapping function is carried in the bitstream as a uniformly sampled piece-wise linear (PWL) function. The advantage of using uniformly sampled function of length R is that the mapping of a sample value Y can be easily performed on-the-fly by a straightforward access to look-up-tables of size N, using an index computed from the value Y shifting by K bits.

This process may be described as follows:

- Computation of the index k = Y / R = Y >> K

- Ymap = scale[ k ] * Y + A[ k ]

where A is a pre-built look-up-table of size (N+1), derived as follows (for k=0 to N):

- A[ k ] = FwdMap[ k*R ] – k * R * scale[ k ]

Figure 2. Construction of the forward mapping function

Basically, the successive pivot points (for each uniformly sampled step) ordinate value differences representative of the forward mapping curve are coded. Bounds of the forward mapping curve are carried as indices in the bitstream as well (R as a minimum and 5R as a maximum in the figures).

The decoder reconstructs the forward mapping function from these syntax elements. The inverse mapping function is derived from reconstructed forward mapping function. It is built as the symmetric function of FwdMap related to the line defined by y = x.

Figure 3. Construction of the inverse mapping function

# Problem statement: forward mapping function carriage issues

From the decoder standpoint, Figure 1 shows that intra pictures decoding requires only inverse mapping function however bitstream carries a forward mapping function only applied for decoding inter coded pictures.

In case of on-the-fly implementation, as the inverse mapping function is non-uniformly sampled, the application of the non-uniformly sampled inverse mapping function requires a pixel-wise binary search to determine in which portion of the inverse PWL portion the sample to inverse-map is located. This stands for extra computation especially for intra coded pictures and to some extent to any intra picture profile used in consumer electronics and professional equipment.

Besides, as explained in JVET-N0299 [4], chroma residue scaling (CRS) also involves the determination of an index related to the inverse mapping function and consequently it requires a binary search as well.

# Proposition: inverse mapping function carriage

The proposition aims at reducing inverse mapping processes complexity, by enabling the carriage of a uniformly sampled inverse mapping function (rather than the forward mapping function). In that case, the index identification involved in the inverse mapping and in the CRS processes is computed thanks to shifts only (removed pixel-wise binary search).

In the proposed implementation, the inverse mapping function is computed at the encoder from the forward mapping function as proposed in JVET-M0640. The (non-uniformly sampled) forward mapping function required for decoding inter coded pictures is computed from carried inverse mapping function.

## Option 1: inverse mapping function replacing forward mapping function

In option1, the current syntax is slightly adjusted. Semantics must be adjusted to incorporate simplifications as well.

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = 0; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

It is noted that the inverse mapping function starts at abscissa 0 (see figure above). lmcs_min_bin_idx is used to determine the ordinate value of the first pivot point of the inverse mapping function i.e. lmcs_min_bin_idx specifies the minimum bin index used in the reconstruction of the forward luma mapping function. The proposed reconstruction process is provided in an attached document.

## Option 2: inverse mapping function or forward mapping function carriage

In option 2, a new syntax element slice_lmcs_inverse_flag is added (at the SPS, slice/tile or lmcs_data() levels) to enable switching between the forward or inverse mapping model.

| slice_header() | Descriptor |
| --- | --- |
| ... |  |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(5) |
| slice_lmcs_inverse_flag | u(1) |
| if( !( qtbtt_dual_tree_intra_flag && slice_type = = I ) ) |  |
| slice_chroma_residual_scale_flag | u(1) |
| } |  |
| ... |  |

When set to 0, slice_lmcs_inverse_flag indicates that the mapping function carried in lmcs_data() is forward. When set to 1, it indicates that the mapping function carried in lmcs_data() is inverse.

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = LmcsMinBinIdx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

where the variable LmcsMinBinIdx is set to lmcs_min_bin_idx when slice_lmcs_inverse_flag is equal to 0. The variable LmcsMinBinIdx is set to 0 when slice_lmcs_inverse_flag is equal to 1.

Semantics and proposed decoding process changes are implemented in a redline version of VVC draft 5 attached to the present document.

Option 2 can be enabled in the provided source code with the following macro:

IDCC_LMCS_INVLUT	1

IDCC_LMCS_INVLUT_EXT_CLIP 0

## Option 3: option 2 + preserving last slope before clipping

Due to the uniformization process of the inverse mapping function (i.e. selecting pivot points on the uniform sampling abscissa), the uniformly sampled inverse mapping function approximates the non-uniformly sampled inverse mapping function. This approximation creates a light decrease of performance on one sequence of the SDR CTCs (i.e. BQSquare in RA mode). The issue arises for the last portion of the PWL inverse mapping curve for which the last derived slope may be significantly different from the equivalent slope of the non-uniform curve due to clipping (illustrated by the small dashed lines between circles in next figure).

In option 3, a change in the derivation and coding of the last pivot point is proposed. The change consists in deriving the last pivot point by extending the last segment of the inverse curve (see figure below). A clipping is applied for this last section to the original non-uniform maximum bound. This clipping value (see next figure) is carried in the bitstream and corresponds to maxBinIdx of the forward mapping curve. Thus, both maxBinIdx of inverse and forward mapping curves are carried in the bitstream when the inverse LUT is carried in the bitstream. Consequently, values represented by last slopes of the inverse mapping curves are superimposed with those of the original non-uniform inverse mapping curve.

Figure 4. Limiting inverse mapping function to initial maximum bound

An exemplary syntax is proposed in the following array:

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| if( slice_lmcs_inverse_flag ) |  |
| lmcs_delta_fwd_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

lmcs_delta_fwd_max_bin_idx has the same semantic as current lmcs_delta_max_bin_idx semantic present in VVC draft 5. It is noted that lmcs_delta_max_bin_idx in the present document is representative of the inverse mapping function maxBinIdx when the inverse mapping function is carried in the bitstream.

Option 3 can be enabled in the provided source code with the following macro:

IDCC_LMCS_INVLUT	1

IDCC_LMCS_INVLUT_EXT_CLIP 1

# Results

Results are provided in the following tables, compared to VTM5.0 with LMCS tool enabled, for option 2 and option 3. For HDR content, the anchor is VTM5.0 lQP = 0 LMCS = 1.  By design, option 1 performance is similar to option 2 performance.

## Option 2 performance

### SDR

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (LMCS enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,12% | 0,15% | 0,01% | 101% | 100% |
| Class A2 | 0,00% | -0,08% | -0,06% | 100% | 100% |
| Class B | -0,06% | 0,02% | 0,05% | 99% | 99% |
| Class C | -0,04% | -0,13% | -0,02% | 99% | 99% |
| Class E | -0,02% | -0,06% | -0,09% | 99% | 100% |
| Overall | -0,05% | -0,02% | -0,01% | 99% | 99% |
| Class D | -0,09% | 0,37% | 0,06% | 99% | 98% |
| Class F | -0,08% | 0,05% | 0,01% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (LMCS enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | 0,15% | 0,15% | 100% | 101% |
| Class A2 | -0,01% | 0,08% | 0,05% | 100% | 99% |
| Class B | -0,04% | 0,14% | 0,07% | 100% | 100% |
| Class C | -0,03% | -0,18% | -0,09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | 0,04% | 0,04% | 100% | 100% |
| Class D | 0,10% | 0,39% | 0,44% | 101% | 100% |
| Class F | -0,03% | -0,04% | -0,08% | 101% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (LMCS enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | 0,39% | 0,28% | 101% | 100% |
| Class C | -0,03% | 0,42% | -0,16% | 100% | 100% |
| Class E | -0,05% | 0,45% | 0,27% | 100% | 99% |
| Overall | -0,04% | 0,42% | 0,13% | 100% | 99% |
| Class D | 0,25% | 0,45% | 0,65% | 102% | 100% |
| Class F | -0,03% | 0,04% | -0,01% | 99% | 100% |

### HDR - AI

|  |  |  | wPSNR |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V |
| Class H1 | -0,04% | -0,03% | -0,04% | 0,13% | 0,25% |
| Class H2 |  |  |  |  |  |
| Overall | -0,04% | -0,03% | -0,04% | 0,13% | 0,25% |

### HDR - RA

|  |  |  | wPSNR |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V |
| Class H1 | -0,24% | -0,01% | -0,04% | 0,16% | -0,07% |
| Class H2 |  |  |  |  |  |
| Overall | -0,24% | -0,01% | -0,04% | 0,16% | -0,07% |

## Option 3 performance

### SDR

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (LMCS enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,04% | 0,09% | 0,01% | 99% | 100% |
| Class A2 | 0,00% | -0,08% | 0,01% | 99% | 100% |
| Class B | 0,00% | -0,05% | -0,03% | 98% | 101% |
| Class C | 0,00% | -0,15% | -0,03% | 100% | 100% |
| Class E | 0,00% | -0,08% | -0,05% | 100% | 100% |
| Overall | -0,01% | -0,06% | -0,02% | 99% | 100% |
| Class D | -0,02% | 0,09% | -0,11% | 101% | 101% |
| Class F | -0,04% | 0,09% | 0,01% | 99% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (LMCS enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | 0,07% | 0,13% | 101% | 101% |
| Class A2 | 0,00% | 0,08% | 0,06% | 101% | 99% |
| Class B | 0,00% | -0,01% | -0,10% | 99% | 100% |
| Class C | 0,00% | -0,04% | 0,02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | 0,02% | 0,01% | 100% | 100% |
| Class D | 0,03% | 0,08% | 0,24% | 100% | 100% |
| Class F | -0,02% | -0,04% | -0,05% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 (LMCS enabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | 0,36% | 0,08% | 101% | 100% |
| Class C | 0,03% | 0,10% | -0,12% | 100% | 100% |
| Class E | 0,06% | 0,52% | 0,46% | 100% | 99% |
| Overall | 0,03% | 0,31% | 0,11% | 100% | 100% |
| Class D | 0,01% | 0,07% | 1,29% | 98% | 100% |
| Class F | 0,00% | 0,17% | 0,18% | 99% | 100% |

### HDR - AI

|  |  |  | wPSNR |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V |
| Class H1 | -0,04% | -0,03% | -0,04% | 0,13% | 0,25% |
| Class H2 |  |  |  |  |  |
| Overall | -0,04% | -0,03% | -0,04% | 0,13% | 0,25% |

### HDR - RA

|  |  |  | wPSNR |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V |
| Class H1 | -0,24% | -0,01% | -0,04% | 0,16% | -0,07% |
| Class H2 |  |  |  |  |  |
| Overall | -0,24% | -0,01% | -0,04% | 0,16% | -0,07% |

# Acknowledgments

Thank you to Panasonic for cross-checking.

# References

[1] JVET-M0427, T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby), "CE12: Mapping functions (test CE12-1 and CE12-2)", Marrakech meeting

[2] JVET-N1001, B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 5)", Geneva meeting

[3] JVET-M0640, E. Francois (Technicolor), "CE12-related: in-loop reshaping with approximate inverse mapping function", Marrakech meeting

[4] JVET-N0299, J. Zhao, S. Kim (LGE), "On Luma Dependent Chroma Residual Scaling of In-loop Reshaper "

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).