| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0450 |
| --- | --- |

| Title: | Separating context coded and bypass coded data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Currently VVC draft standard specifies that context coded bins and bypass coded bins are both coded by arithmetic coder and the resulting data for both are interweaved in the bitstream. The contribution proposes to separate context coded data from bypass coded data in the bitstream. It is asserted that by separating bypass coded data decoding of bypass bins is simplified and decoding of context coded bins does not depend on bypass bins anymore.

To avoid sending length field indicating start of the bypass coded part, bypass part is coded after the context coded part in byte reversed manner.

Average overhead for separating bypass bins is 4.5 bits on average (3.5 bits on average for additional byte alignment and 1 extra stop bit) if bypass data always exists. Overhead is zero if bypass data does not exist.

# Proposal

CABAC encodes both context coded and bypass coded bins using arithmetic coder and puts resulting data to the same bitstream as illustrated in Figure 1. Bypass bin coding is simpler than that of context coded bins, but bypass bins still needs to go through arithmetic coding engine and arithmetic coder and decoder state must be updated. Also, when context coded bin is preceded by bypass coded symbol the decoder must resolve the bypass coded symbol before context coded bin can be decoded.

These two issues can be removed by separating bypass coded bins into their own sub-bitstream. Effectively bypass coding then becomes just FLC/VLC coding. For decoder to know where to find bypass data, the encoder must signal the position in the bitstream the same way as WPP row or tile entry point in signalled. However, there are only two sub-streams in this case, so the bypass part can be coded in reverse order following the context coded part and therefore entry point signaling can be avoided. This arrangement is illustrated in Figure 2. It can be calculated than overhead for adding one extra byte alignment plus one extra stop bit is 4.5 bits on average compared to VVC reference.

It is possible to further reduce the overhead by reversing bit order within the bytes in the bypass part. This way the last coded byte of the context coded part and the last coded byte of the bypass coded part can be often combined into a single byte. The average overhead can be calculated to be 1.125 bits compared to VVC reference.

Figure 1- Arithmetic coding in VVC

Figure 2 - Separate context coded and bypass coded streams

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).