| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0452 |
| --- | --- |

| Title: | AHG12: On tile and brick partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Muhammed Coban, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Email: | {yjchang, mcoban, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The ambiguities on the semantics of single_tile_in_pic_flag and uniform_tile_spacing_flag are identified and fixed in this proposal. If the height of a tile in unit of CTU is smaller than or equal to 2, there might be potential bugs when parsing the syntax elements brick_split_flag and uniform_brick_spacing_flag. These potential bugs are also discussed and addressed in this proposal.

# Introduction

The current VVC syntax design supports a two-stage picture partition scheme, which first determines whether to partition a picture into smaller tiles, and then further determine whether to partition a tile into smaller bricks. The syntax design of the tile and brick partitioning process in the VVC Draft 5 [1] is shown in the table below:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. single_tile_in_pic_flag; u(1). if( !single_tile_in_pic_flag ) {. uniform_tile_spacing_flag; u(1). if( uniform_tile_spacing_flag ) {. tile_cols_width_minus1; ue(v). tile_rows_height_minus1; ue(v). } else {. num_tile_columns_minus1; ue(v). num_tile_rows_minus1; ue(v). for( i = 0; i < num_tile_columns_minus1; i++ ). tile_column_width_minus1[ i ]; ue(v). for( i = 0; i < num_tile_rows_minus1; i++ ). tile_row_height_minus1[ i ]; ue(v). }. brick_splitting_present_flag; u(1). for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) {. brick_split_flag[ i ]; u(1). if( brick_split_flag[ i ] ) {. uniform_brick_spacing_flag[ i ]; u(1). if( uniform_brick_spacing_flag[ i ] ). brick_height_minus1[ i ]; ue(v). else {. num_brick_rows_minus1[ i ]; ue(v). for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ). brick_row_height_minus1[ i ][ j ]; ue(v). }. }. }. ….

# Improvement on syntax and semantics

Issue 1: In the current syntax design, there are ambiguities on the semantics of single_tile_in_pic_flag and  uniform_tile_spacing_flag as shown below:

- When single_tile_in_pic_flag is equal to 1, there shall be only one tile in each picture as indicated in semantics. However, single tile can be also achieved even if single_tile_in_pic_flag is equal to 0 by setting tile_rows_height_minus1 = PicHeightInCtbsY – 1 and tile_cols_width_minus1 = PicWidthInCtbsY – 1, or by setting num_tile_columns_minus1 = 0 and num_tile_rows_minus1 = 0.

- When uniform_tile_spacing_flag is equal to 1, the tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture. However, the same uniform distribution of tile column and tile row boundaries across the picture can be also achieved even if uniform_tile_spacing_flag is equal to 0 by setting num_tile_columns_minus1 = PicHeightInCtbsY – 1 and num_tile_rows_minus1 = PicWidthInCtbsY – 1, or by setting num_tile_columns_minus1 = 0 and num_tile_rows_minus1 = 0.

Therefore, it is proposed to avoid to concurrently set tile_rows_height_minus1 = PicHeightInCtbsY − 1 and tile_cols_width_minus1 = PicWidthInCtbsY – 1 as single_tile_in_pic_flag = 0. It is further proposed to avoid that num_tile_columns_minus1 and num_tile_rows_minus1 are concurrently set as 0​, or concurrently set as PicHeightInCtbsY – 1 and PicWidthInCtbsY – 1 respectively. The proposed syntax changes to VVC Draft 5 is as described in the following table with yellow change mark:

Section 7.4.3.4 in VVC Draft 5 version 8: single_tile_in_pic_flag equal to 1 specifies that there is only one tile in each picture referring to the PPS. single_tile_in_pic_flag equal to 0 specifies that there is more than one tile in each picture referring to the PPS. NOTE – In absence of further brick splitting within a tile, the whole tile is referred to as a brick. When a picture contains only a single tile without further brick splitting, it is referred to as a single brick. It is a requirement of bitstream conformance that the value of single_tile_in_pic_flag shall be the same for all PPSs that are activated within a CVS. uniform_tile_spacing_flag equal to 1 specifies that tile column boundaries and likewise tile row boundaries are distributed uniformly across the picture and signalled using the syntax elements tile_cols_width_minus1 and tile_rows_height_minus1. uniform_tile_spacing_flag equal to 0 specifies that tile column boundaries and likewise tile row boundaries may or may not be distributed uniformly across the picture and signalled using the syntax elements num_tile_columns_minus1 and num_tile_rows_minus1 and a list of syntax element pairs tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ]. When not present, the value of uniform_tile_spacing_flag is inferred to be equal to 1. tile_cols_width_minus1 plus 1 specifies the width of the tile columns excluding the right-most tile column of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. The value of tile_cols_width_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_cols_width_minus1 is inferred to be equal to PicWidthInCtbsY – 1. tile_rows_height_minus1 plus 1 specifies the height of the tile rows excluding the bottom tile row of the picture in units of CTBs when uniform_tile_spacing_flag is equal to 1. If single_tile_in_pic_flag is equal to 0 and tile_cols_width_minus1 is equal to PicWidthInCtbsY – 1, the value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 2, inclusive. Otherwise, the value of tile_rows_height_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_rows_height_minus1 is inferred to be equal to PicHeightInCtbsY – 1. num_tile_columns_minus1 plus 1 specifies the number of tile columns partitioning the picture when uniform_tile_spacing_flag is equal to 0. The value of num_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_columns_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_columns_minus1 is inferred as specified in clause 6.5.1. num_tile_rows_minus1 plus 1 specifies the number of tile rows partitioning the picture when uniform_tile_spacing_flag is equal to 0. If uniform_tile_spacing_flag is equal to 0 and num_tile_columns_minus1 is equal to 0, the value of num_tile_rows_minus1 shall be in the range of 1 to PicHeightInCtbsY − 1, inclusive. Otherwise, if uniform_tile_spacing_flag is equal to 0 and num_tile_columns_minus1 is equal to PicWidthInCtbsY − 1, the value of num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 2, inclusive. Otherwise, the value of num_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. If single_tile_in_pic_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0. Otherwise, when uniform_tile_spacing_flag is equal to 1, the value of num_tile_rows_minus1 is inferred as specified in clause 6.5.1.

Issue 2: If the height of a tile in unit of CTU is smaller than or equal to 2, there are potential bugs when parsing the syntax elements brick_split_flag and uniform_brick_spacing_flag as described below:

- If the height of the i-th tile in unit of CTU is equal to 1, i.e., RowHeight[i] = 1, there is a bug when brick_split_flag[i] equal to 1 is signaled.

- If the height of the i-th tile in unit of CTU is equal to 2, i.e., RowHeight[i] = 2, there is a bug or an ambiguous on semantics when uniform_brick_spacing_flag[i] equal to 0 is signaled.

Therefore, it is proposed to infer brick_split_flag[ i ] to be zero if RowHeight[ i ] = 1​. It is further proposed to infer uniform_brick_spacing_flag[ i ] to be 1 and brick_height_minus1[ i ] to be 0 if RowHeight[ i ] = 2 since it must be uniform brick split​. It is proposed to avoid num_brick_rows_minus1[ i ] to be set as RowHeight[ i ] − 1 since brick_height_minus1 = 0 can already cover this case. The proposed syntax changes to VVC Draft 5 is as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. brick_splitting_present_flag; u(1). for( i = 0; brick_splitting_present_flag && RowHeight[i] > 1 && i < NumTilesInPic; i++ ) {. brick_split_flag[ i ]; u(1). if( brick_split_flag[ i ] && RowHeight[i] > 2) {. uniform_brick_spacing_flag[ i ]; u(1). if( uniform_brick_spacing_flag[ i ] ). brick_height_minus1[ i ]; ue(v). else {. num_brick_rows_minus1[ i ]; ue(v). for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ). brick_row_height_minus1[ i ][ j ]; ue(v). }. }. }. …. Section 7.4.3.4 in VVC Draft 5 version 8: brick_splitting_present_flag equal to 1 specifies that one or more tiles of pictures referring to the PPS may be divided into two or more bricks. brick_splitting_present_flag equal to 0 specifies that no tiles of pictures referring to the PPS are divided into two or more bricks. brick_split_flag[ i ] equal to 1 specifies that the i-th tile is divided into two or more bricks. brick_split_flag[ i ] equal to 0 specifies that the i-th tile is not divided into two or more bricks. When not present, the value of brick_split_flag[ i ] is inferred to be equal to 0. uniform_brick_spacing_flag[ i ] equal to 1 specifies that horizontal brick boundaries are distributed uniformly across the i-th tile and signalled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that horizontal brick boundaries may or may not be distributed uniformly across i-th tile and signalled using the syntax element num_brick_rows_minus1[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ]. When not present, the value of uniform_brick_spacing_flag[ i ] is inferred to be equal to 1. brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. If brick_height_minus1 [ i ] is not present and RowHeight[ i ] is equal to 2, the value of brick_height_minus1[ i ] is inferred to be equal to 0. Otherwise, When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] – 1. num_brick_rows_minus1[ i ] plus 1 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1[ i ] shall be in the range of 1 to RowHeight[ i ] – 1 2, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus1[ i ] is inferred to be equal to 0. Otherwise, when uniform_brick_spacing_flag[ i ] is equal to 1, the value of num_brick_rows_minus1[ i ] is inferred as specified in 6.5.1.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, the 14th JVET meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).