| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0453 |
| --- | --- |

| Title: | AHG12: On rectangular slice signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Muhammed Coban, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Email: | {yjchang, mcoban, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

When the number of bricks in picture is equal to 2, and/or the number of slices in picture is equal to 1, potential bugs are discovered in the current rectangular slice signaling syntax at PPS level. This contribution discusses and addresses the issues.

# Introduction

The current VVC syntax design supports to identify the locations of each rectangular slice in the picture. The syntax table related to rectangular slices in the VVC Draft 5 [1] at PPS and slice header is shown in the table below:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. single_brick_per_slice_flag; u(1). if( !single_brick_per_slice_flag ). rect_slice_flag; u(1). if( rect_slice_flag && !single_brick_per_slice_flag ) {. num_slices_in_pic_minus1; ue(v). for( i = 0; i <= num_slices_in_pic_minus1; i++ ) {. if( i > 0 ). top_left_brick_idx[ i ]; u(v). bottom_right_brick_idx_delta[ i ]; u(v). }. }. …. Section 7.3.2.4 in VVC Draft 5 version 8: slice_header( ) {; Descriptor. …. if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ). slice_address; u(v). if( !rect_slice_flag && !single_brick_per_slice_flag ). num_bricks_in_slice_minus1; ue(v). ….

# Improvement on rectangular slice syntax

When the number of bricks in picture is equal to 2, and/or the number of slices in picture is equal to 1, several potential bugs are discovered in the current rectangular slice signaling syntax design, and are discussed as follows:

- If the number of bricks in picture is equal to 2, i.e., NumBricksInPic = 2, the number of slices must be either 1 or 2. When single_brick_per_slice_flag is equal to 0, it implies there is only one slice in picture including 2 bricks. A bug therefore can be discovered if the number of slices is signaled as an integer value larger than 1.

- If the number of slices in picture is equal to 1, i.e., num_slices_in_pic_minus1 = 0, a bug occurs when bottom_right_back_idx_delta[0] is improperly set as an integer number smaller than NumBricksInPic - 1.

Therefore, it is proposed to implicitly infer num_slices_in_pic_minus1 to be 0 if single_brick_per_slice_flag is disabled and NumBricksInPic is equal to 2. Because the number of slices is set as 1, it is meaningless to signal slice ID in the PPS level. Therefore, it is further proposed to infer rect_slice_flag to be 0 as single_brick_per_slice_flag is disabled and NumBricksInPic is equal to 2. Since there is only one slice in picture when NumBricksInPic is equal to 2, the syntax element num_bricks_in_slice_minus1 in slice header must be inferred to be 1 accordingly. To deal with another potential bug in the case of num_slices_in_pic_minus1 equal to 0, it is proposed to infer top_left_brick_idx[0] and bottom_right_brick_idx_delta[0] to be 0 and NumBricksInPic – 1, respectively.

The syntax changes to VVC Draft 5 version 8 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. single_brick_per_slice_flag; u(1). if( !single_brick_per_slice_flag && NumBricksInPic > 2 ). rect_slice_flag; u(1). if( rect_slice_flag && !single_brick_per_slice_flag && NumBricksInPic > 2 ) {. num_slices_in_pic_minus1; ue(v). for( i = 0; i <= num_slices_in_pic_minus1 && num_slices_in_pic_minus1 > 0; i++ ) {. if( i > 0 ). top_left_brick_idx[ i ]; u(v). bottom_right_brick_idx_delta[ i ]; u(v). }. }. …. Section 7.3.5.1 in VVC Draft 5 version 7: slice_header( ) {; Descriptor. slice_pic_parameter_set_id; ue(v). if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ). slice_address; u(v). if( !rect_slice_flag && !single_brick_per_slice_flag && NumBricksInPic > 2 ). num_bricks_in_slice_minus1; ue(v). …; ue(v). Section 7.4.3.4 in VVC Draft 5 version 8: rect_slice_flag equal to 0 specifies that bricks within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that bricks within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When not present and single_brick_per_slice_flag is equal to 1, rect_slice_flag is inferred to be equal to 1. When not present and single_brick_per_slice_flag is equal to 0, rect_slice_flag is inferred to be equal to 0. num_slices_in_pic_minus1 plus 1 specifies the number of slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When not present and single_brick_per_slice_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to NumBricksInPic − 1. When not present and single_brick_per_slice_flag is equal to 0, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. top_left_brick_idx[ i ] specifies the brick index of the brick located at the top-left corner of the i-th slice. The value of top_left_brick_idx[ i ] shall not be equal to the value of top_left_brick_idx[ j ] for any i not equal to j. When not present, the value of top_left_brick_idx[ i ] is inferred to be equal to i. The length of the top_left_brick_idx[ i ] syntax element is Ceil( Log2( NumBricksInPic ) bits. bottom_right_brick_idx_delta[ i ] specifies the difference between the brick index of the brick located at the bottom-right corner of the i-th slice and top_left_brick_idx[ i ]. When single_brick_per_slice_flag is equal to 1, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to 0. When not present, single_brick_per_slice_flag is equal to 0 and num_slices_in_pic_minus1 is equal to 0, the value of bottom_right_brick_idx_delta[ i ] is inferred to be equal to NumBricksInPic – 1. The length of the bottom_right_brick_idx_delta[ i ] syntax element is Ceil( Log2( NumBricksInPic − top_left_brick_idx[ i ] ) ) bits. Section 7.4.6 in VVC Draft 5 version 8: num_bricks_in_slice_minus1, when present, specifies the number of bricks in the slice minus 1. The value of num_bricks_in_slice_minus1 shall be in the range of 0 to NumBricksInPic − 1, inclusive. When rect_slice_flag is equal to 0 and single_brick_per_slice_flag is equal to 1, the value of num_bricks_in_slice_minus1 is inferred to be equal to 0. When not present, single_brick_per_slice_flag is equal to 0 and NumBricksInPic is equal to 2, the value of num_bricks_in_slice_minus1 is inferred to be equal to 1.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, the 14th JVET meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).