| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0454 |
| --- | --- |

| Title: | AHG12: On sub-bitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Muhammed Coban, Vadim Seregin, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Email: | {yjchang, mcoban, vseregin, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal clarifies the signaling of bitstream extraction should be disabled when the number of slices in picture is equal to 1. The derivation of the number of the bricks in the extracted sub-bitstream is suggested to be added into VVC Draft. A new syntax element is further proposed to allow the number of rectangular slices extracted from the bitstream to be non-power of two.

# Introduction

The current VVC syntax design can extracts rectangular slices from the bitstream to form a new picture. The syntax table related to the bitstream extraction in the VVC Draft 5 [1] at PPS and slice header is shown in the table below:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. if( rect_slice_flag ) {. signalled_slice_id_flag; u(1). if( signalled_slice_id_flag ) {. signalled_slice_id_length_minus1; ue(v). for( i = 0; i <= num_slices_in_pic_minus1; i++ ). slice_id[ i ]; u(v). }. }. …. Section 7.3.2.4 in VVC Draft 5 version 8: slice_header( ) {; Descriptor. …. if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ). slice_address; u(v). …. Section 7.3.2.4 in VVC Draft 5 version 8: signalled_slice_id_flag equal to 1 specifies that the slice ID for each slice is signalled. signalled_slice_id_flag equal to 0 specifies that slice IDs are not signalled. When rect_slice_flag is equal to 0, the value of signalled_slice_id_flag is inferred to be equal to 0. signalled_slice_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element slice_id[ i ] when present, and the syntax element slice_address in slice headers. The value of signalled_slice_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_slice_id_length_minus1 is inferred to be equal to Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) − 1. slice_id[ i ] specifies the slice ID of the i-th slice. The length of the slice_id[ i ] syntax element is signalled_slice_id_length_minus1 + 1 bits. When not present, the value of slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive. Section 7.4.6.1 in VVC Draft 5 version 8: slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0. If rect_slice_flag is equal to 0, the following applies: - The slice address is the brick ID as specified by Equation 6-8. - The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits. - The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive. Otherwise (rect_slice_flag is equal to 1), the following applies: - The slice address is the slice ID of the slice. - The length of slice_address is signalled_slice_id_length_minus1 + 1 bits. - If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

# Issue in signalled_slice_id_length_minus1

The syntax element signalled_slice_id_length_minus1 specifies the number of bits used to represent the syntax element slice_id[ i ] in PPS and the syntax element slice_address in slice headers. If signalled_slice_id_flag is equal to 0, signalled_slice_id_length_minus1 would be inferred to be Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) – 1. When the number of slices in picture is equal to 1, i.e., num_slices_in_pic_minus1 is equal to 0, it derives signalled_slice_id_length_minus1 to be a negative number, i.e., -1. It is an unfavorable design to allow a PPS-level syntax element to be a negative number. Therefore, it is proposed to skip the signaling of slice_id[ i ] in PPS and slice_address in slice headers when num_slices_in_pic_minus1 is equal to 0. The syntax elements slice_id[0] and slice_address are both inferred to be 0. Because there is no bitstream extraction in this case, signalled_slice_id_flag shall be inferred to be 0 when num_slices_in_pic_minus1 is equal to 0. The condition check of slice_address is also modified accordingly. The syntax changes to VVC Draft 5 version 8 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. if( rect_slice_flag && num_slices_in_pic_minus1 > 0 ) {. signalled_slice_id_flag; u(1). if( signalled_slice_id_flag ) {. signalled_slice_id_length_minus1; ue(v). for( i = 0; i <= num_slices_in_pic_minus1; i++ ). slice_id[ i ]; u(v). }. }. …. Section 7.3.5.1 in VVC Draft 5 version 8: slice_header( ) {; Descriptor. slice_pic_parameter_set_id; ue(v). if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 && num_slices_in_pic_minus1 > 0 ). slice_address; u(v). if( !rect_slice_flag && !single_brick_per_slice_flag ). num_bricks_in_slice_minus1; ue(v). …; ue(v). Section 7.4.3.4 in VVC Draft 5 version 8: signalled_slice_id_flag equal to 1 specifies that the slice ID for each slice is signalled. signalled_slice_id_flag equal to 0 specifies that slice IDs are not signalled. When rect_slice_flag is equal to 0 not present, the value of signalled_slice_id_flag is inferred to be equal to 0.

# Issue in NumBricksInPic

If the syntax elements rect_slice_flag and signalled_slice_id_flag are both enabled, a subset of the slices in picture is extracted from the bitstream to a new shorter bitstream. The number of bricks in the new bitstream should be changed accordingly. The current specification misses the description to derive the number of bricks. On top of VVC Draft 5 version 8, the description in the following table is suggested to be added into PPS semantics to update the number of bricks after the sub-bitstream is decided. It is suggested to further clarify whether there are other changes, if any, are required to be added.

NumBricksInPic is originally as derived in subclause 6.5.1. If rect_slice_flag and signalled_slice_id_flag are both equal to 1 and the number of the extracted slices is smaller than num_slices_in_pic_minus1 + 1, a smaller sub-bitstream is extracted from the bitstream. The variable NumBricksInPic for this new sub-bitstream is updated as follows: for( i = 0; i <= num_slices_in_pic_minus1; i++ ) { if ( The i-th slice is not extracted from the bitstream ) { NumBricksInPic = NumBricksInPic – NumBricksInSlice[ i ] } }

# Issue in slice_address

In the current design, if rect_slice_flag is equal to 1, there are two conditions to derive the range of the values for the syntax element slice_address at slice header:

- If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive.

- Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) − 1, inclusive.

When signalled_slice_id_flag is equal to 0, it restricts the number of rectangular slices to be num_slices_in_pic_minus1 + 1, which may be an integer of non-power of two. However, it is not reasonable and impractical to restrict the maximum number of rectangular slices extracted from the bitstream to be powers of two, when signalled_slice_id_flag is equal to 1. Therefore, a new syntax element signalled_slice_id_diff is added to PPS level to derive the value of non-power of two for the maximum number of the extracted slices, where max_slice_id_diff could be a value in a range of 0 to 2( signalled_slice_id_length_minus1 ) – 1. If signalled_slice_id_flag is equal to 1, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) – max_slice_id_diff – 1, inclusive. The syntax changes to VVC Draft 5 version 8 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 5 version 8: pic_parameter_set_rbsp( ) {; Descriptor. …. if( rect_slice_flag ) {. signalled_slice_id_flag; u(1). if( signalled_slice_id_flag ) {. signalled_slice_id_length_minus1; ue(v). if ( signalled_slice_id_length_minus1 > 0 ). max_slice_id_diff; u(v). for( i = 0; i <= num_slices_in_pic_minus1; i++ ). slice_id[ i ]; u(v). }. }. …. Section 7.4.3.4 in VVC Draft 5 version 8: signalled_slice_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element slice_id[ i ] when present, and the syntax element slice_address in slice headers. The value of signalled_slice_id_length_minus1 shall be in the range of 0 to 15, inclusive. When not present, the value of signalled_slice_id_length_minus1 is inferred to be equal to Ceil( Log2( num_slices_in_pic_minus1 + 1 ) ) − 1. max_slice_id_diff specifies the difference value of 2( signalled_slice_id_length_minus1 + 1 ) – 1 subtracted from the maximum number of slice_address. The maximum number of slice_address is derived by 2( signalled_slice_id_length_minus1 + 1 ) – 1 – max_slice_id_diff. The value of max_slice_id_diff is in the range of 0 to 2( signalled_slice_id_length_minus1 ) – 1. When not present, it is inferred to be 0. slice_id[ i ] specifies the slice ID of the i-th slice. The length of the slice_id[ i ] syntax element is signalled_slice_id_length_minus1 + 1 bits. When not present, the value of slice_id[ i ] is inferred to be equal to i, for each i in the range of 0 to num_slices_in_pic_minus1, inclusive. Section 7.4.6.1 in VVC Draft 5 version 8: slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0. If rect_slice_flag is equal to 0, the following applies: - The slice address is the brick ID as specified by Equation 6-8. - The length of slice_address is Ceil( Log2 ( NumBricksInPic ) ) bits. - The value of slice_address shall be in the range of 0 to NumBricksInPic − 1, inclusive. Otherwise (rect_slice_flag is equal to 1), the following applies: - The slice address is the slice ID of the slice. - The length of slice_address is signalled_slice_id_length_minus1 + 1 bits. - If signalled_slice_id_flag is equal to 0, the value of slice_address shall be in the range of 0 to num_slices_in_pic_minus1, inclusive. Otherwise, the value of slice_address shall be in the range of 0 to 2( signalled_slice_id_length_minus1 + 1 ) – 1 – max_slice_id_diff, inclusive.

In another related issue, the current design is quite unclear and ambiguous to allow the maximum number of the extracted rectangular slices to be larger than the number of the slices in the picture. It is suggested to avoid this flexibility without clear purpose and clear use case. Restrictions to signalled_slice_id_length_minus1 and/or the range of slice_address should be added into the specification in order to avoid this flexibility.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, the 14th JVET meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).