| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0455r2 |
| --- | --- |

| Title: | Non-CE8: On IBC merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Yan Zhang, Chun-Chi Chen, Wei-Jung Chien, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Email: | {yjchang, yzh, chunchic, wchien, martak}@qti. qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The current VVC includes 4 different types of merge modes, e.g. regular merge mode, subblock merge mode, triangle merge mode, and IBC merge mode, to address different contents such as translational motion, motions in triangle shape, intra-picture motion (for screen content use case), and general motion. It was already agreed to utilize three independent syntax elements to separately specify the maximum number of merging candidates for regular merge mode, subblock merge mode, and triangle merge mode. This proposal further adds one more syntax element in slice header to independently specify the maximum number of merging candidates for IBC merge mode, as the same as what regular merge mode, subblock merge mode, and triangle merge mode already do.

# Introduction

There are four types of merge modes in the current VVC design, including regular merge mode, subblock merge mode, triangle merge mode, and IBC merge mode. Each mode can be effective in different use cases, such as subblock merge mode for translational motion, triangle merge mode for the motions in triangle shape blocks, IBC merge mode for intra-picture motion (mainly for screen content), and regular merge mode for the general motions. It was agreed in the previous meetings to use three independently slice-level syntax elements as marked in the following table to separately specify the maximum number of merging candidates for regular merge mode, subblock merge mode, and triangle merge mode:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( slice_type != I ) { |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| slice_fpel_mmvd_enabled_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 ) |  |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_merge_cand | ue(v) |
| … |  |

Issue 1: IBC merge mode uses the syntax element six_minus_max_num_merge_cand originally designed for regular merge mode to specify the maximum number of IBC merge candidates. In order to independently indicate the maximum number of merging candidates of IBC merge mode, this contribution proposes to add a new syntax element as the same as other merge modes already have.

Issue 2: This proposal further discusses whether to separate the merge index signaling that are currently shared by regular merge mode and IBC merge mode.

It is claimed the proposed design can not only unify the derivation of the maximum numbers of different merge modes, but support flexibility to determine the complexity at encoder side.

# Issue on maximum number of IBC merging candidates

Proposed 1.1: It is proposed to add a slice-level syntax element constant_minus_max_num_ibc_merge_cand to specify the maximum number of IBC merging candidates supported in the slice subtracted from a constant N, where N is a pre-assigned fixed constant such as 5 or 6. The maximum number of IBC merging candidates, MaxNumIntraBlockCopyMergeCand, is derived as follows:

MaxNumIntraBlockCopyMergeCand = N – constant_minus_max_num_ibc_merge_cand

It should be understood that the range of the value of MaxNumIntraBlockCopyMergeCand could have various designs as follows:

- Solution 1: The range of MaxNumIntraBlockCopyMergeCand is the same in (I, P/B)-slices, i.e., the range of MaxNumIntraBlockCopyMergeCand of (I, P/B) is equal to (0 ~ N, 0 ~ N) or (1 ~ N, 1 ~ N), inclusive.

- Solution 2 (Suggested): The ranges of MaxNumIntraBlockCopyMergeCand are different in (I, P/B)-slices, i.e., the range of MaxNumIntraBlockCopyMergeCand of (I, P/B) is equal to (1 ~ N, 0 ~ N) or (0 ~ N, 1 ~ N), inclusive.

In I-slice, we suggest to follow the current design which set the range of MaxNumIntraBlockCopyMergeCand to be 1 ~ N, inclusive, and excludes the value of MaxNumIntraBlockCopyMergeCand to be zero. In P/B slice, we suggest to align the current design as well that always sets the value of MaxNumMergeCand to be larger than 0, but instead allow the values of MaxNumSubblockMergeCand and MaxNumTriangleMergeCand to be zero; that is, it is proposed to allow the values of MaxNumIntraBlockCopyMergeCand to be in the range of 0 ~ N, inclusive.

Proposed 1.2: Since IBC merge mode disallows one temporal merging candidate as compared to the regular merge mode which allows to use it, if available, it is suggested to set N to be 5.

It is summarized in the following table:

|  | VTM5.0 I-Slice | VTM5.0 P/B-Slice | Proposed I-Slice | Proposed P/B-slice |
| --- | --- | --- | --- | --- |
| MaxNumMergeCand | N/A | 1 ~ 6 | N/A | 1 ~ 6 |
| MaxNumSubblockMergeCand | N/A | 0 ~ 5 | N/A | 0 ~ 5 |
| MaxNumTriangleMergeCand | N/A | 0, 2 ~ MaxNumMergeCand | N/A | 0, 2 ~ MaxNumMergeCand |
| MaxNumIntraBlockCopyMergeCand | MaxNumMergeCand, i.e., 1 ~ 6 |  | 1 ~ 5 | 0 ~ 5 |

The proposed syntax change to VVC Draft is shown in the table below with yellow change mark:

Subclause 7.3.5.1 slice_header( ) {; Descriptor. …. if ( slice_type != I ) {. six_minus_max_num_merge_cand; ue(v). if( sps_affine_enabled_flag ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). slice_fpel_mmvd_enabled_flag; u(1). if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 ). max_num_merge_cand_minus_max_num_triangle_cand; ue(v). } else if ( sps_ibc_enabled_flag ). six_minus_max_num_merge_cand; ue(v). if ( sps_ibc_enabled_flag ). five_minus_max_num_ibc_merge_cand; ue(v). …. Subclause 7.3.7.7 merge_data( x0, y0, cbWidth, cbHeight ) {; Descriptor. if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) {. if( MaxNumMergeCand > 1 ). if(MaxNumIntraBlockCopyMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). …. Subclause 7.4.6.1 five_minus_max_num_ibc_merge_cand specifies the maximum number of IBC merging motion vector prediction (MVP) candidates supported in the slice subtracted from 5. The maximum number of IBC merging MVP candidates, MaxNumMergeCand is derived as follows: MaxNumIntraBlockCopyMergeCand = 5 − constant_minus_max_num_ibc_merge_cand If slice_type is I-slice, the value of MaxNumMergeCand shall be in the range of 1 to 5, inclusive. Otherwise, the value of MaxNumMergeCand shall be in the range of 0 to 5, inclusive.

We have noticed that a plenty of contributions, e.g. JVET-O0162, O0331, O0480, and O0574, have addressed an ambiguous issue when the number of merge candidates is equal to 1 in the current IBC design that AMVP shares the same motion vector candidate list. It is commented that the proposed syntax element, constant_minus_max_num_ibc_merge_cand, could better design with any one of methods provided from different proposals JVET-O0162, O0331, O0480, and O0574, and thus suggested to make decisions by considering both issues discussed.

# Clarification on merging indexes

Another issue is also discussed to clarify whether different merge modes for different purposes should share the same syntax element to signal the index of the merge candidates or not. In current design, regular merge mode, sub-block merge mode and triangle merge mode have respective syntax element to signal the merging index, and, however, the regular merge mode and IBC merge mode share the same syntax elements, i.e., merge_idx.

Proposed 2: The design to share the same syntax element for two coding mode with different use cases should be avoided unless it is guaranteed there is a strong use case or connection. It should be notified that there are two designs to separate the signaling of IBC merging index from the signaling of regular merging index.

- Solution 1: One is to add a new syntax element merge_ibc_idx to identify the index of IBC merging candidates.

- Solution 2 (Suggested): Another one is to simply add a new context to merge_idx, and a first context is used when regular merge mode is enabled, and a second context is used when IBC merge mode is enabled.

The proposed syntax change to VVC Draft is shown in the table below with yellow change mark:

Solution 1: Add a new syntax element merge_ibc_idx to specify the index of IBC merging candidates 7.3.7.7 Merge data syntax merge_data( x0, y0, cbWidth, cbHeight ) {; Descriptor. if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) {. if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ] ibc_merge_idx[x0][y0]; ae(v). …. 7.4.8.7 Merge data semantics Ibc_merge_idx[ x0 ][ y0 ] specifies the IBC merging candidate index of the merging candidate list where x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When ibc_merge_idx[ x0 ][ y0 ] is not present, it is inferred as 0. 8.6.2.2 Derivation process for luma motion vector for merge mode mvL[ 0 ] = mergeMvCandList[ merge_idx ibc_merge_idx[ xCb ][ yCb ] ][ 0 ] (8-978) mvL[ 1 ] = mergeMvCandList[merge_idx ibc_merge_idx [ xCb ][ yCb ] ][ 1 ] (8-979) Table 9-9 – Syntax elements and associated binarizations Syntax structure; Syntax element; Binarization. Process; Input parameters. merge_data( ); merge_idx[ ][ ]; TR; cMax = MaxNumMergeCand – 1, cRiceParam = 0. ibc_merge_idx[ ][ ]; TR; cMax = MaxNumIntraBlockCopyMergeCand – 1, cRiceParam = 0. Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins. Syntax element; binIdx. 0; 1; 2; 3; 4; >= 5. ibc_merge_idx[ ][ ]; 0; bypass; bypass; bypass; bypass; na.

Solution 2: Add a new context to merge_idx, and a first context is used when regular merge mode is enabled, and a second context is used when IBC merge mode is enabled Table 9-9 – Syntax elements and associated binarizations Syntax structure; Syntax element; Binarization. Process; Input parameters. merge_data( ); …; ..; …. merge_idx[ ][ ]; TR; cMax = MaxNumMergeCand – 1, cMax = (CuPredMode[ x0 ][ y0 ] = = MODE_IBC) ? MaxNumIntraBlockCopyMergeCand – 1 : MaxNumMergeCand – 1, cRiceParam = 0. Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins. Syntax element; binIdx. 0; 1; 2; 3; 4; >= 5. merge_idx[ ][ ]; 0 (CuPredMode[ x0 ][ y0 ] = = MODE_IBC) ? 0 : 1; bypass; bypass; bypass; bypass; na.

# Simulation results

The following experiments were conducted using the VTM-5.0 under the common test conditions [1]:

Experiment 1:

Anchor: VTM5.0 with MaxNumIntraBlockCopyMergeCand = 6

Tested method: VTM5.0 with MaxNumIntraBlockCopyMergeCand = 5

Experiment 2:

Anchor: VTM5.0

Tested method: VTM5.0 with one new context added to merge_idx

The simulation results are shown in Table 1 (All-intra configuration) and Table 2 (Random access configuration).

Table 1: Summary results of experiment 1 (MaxNumIntraBlockCopyMergeCand = 5 compared with anchor: MaxNumIntraBlockCopyMergeCand = 6)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.02% | 0.05% | 0.04% | 99% | 100% |
| Class TGM420 | 0.03% | 0.03% | 0.04% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.04% | 0.13% | 0.00% | 99% | 99% |
| Class TGM420 | -0.05% | -0.06% | -0.12% | 98% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | 0.07% | 0.00% | 98% | 99% |
| Class TGM420 | -0.06% | -0.08% | -0.03% | 97% | 98% |

Table 2: Summary results of experiment 2. (Proposed 2 – Solution 2 compared with anchor VTM5.0)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | 0.09% | -0.03% | 100% | 100% |
| Class TGM420 | -0.04% | -0.03% | -0.03% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.07% | 0.02% | 99% | 98% |
| Class TGM420 | -0.12% | -0.05% | -0.09% | 99% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0_IBC1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.02% | -0.12% | -0.27% | 98% | 98% |
| Class TGM420 | -0.01% | -0.11% | 0.03% | 98% | 96% |

# Conclusion

A new syntax element constant_minus_max_num_ibc_merge_cand is proposed to indicate the maximum number of IBC merge candidates, MaxNumIntraBlockCopyMergeCand. This new parsing index benefits to the video codec design especially video encoder to trade-off the complexity and bitrate. This also improves the syntax design to align with other types of merge modes on how to specify the maximum number of merging candidates. Methods to separate the signaling of IBC merging index from the signaling of regular merging index are also discussed to address the practical issue that current CTC cannot identify. It is recommended to adopt the proposals into VVC standard.

# Reference

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, the 14th JVET meeting, Mar. 2019, Geneva, CH.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).