| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0460 |
| --- | --- |

| Title: | On lossless coding for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier, Fabrice Le Léannec, Franck Galpin 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Email: | tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a lossless coding mode for VVC. Lossless mode is included in the VTM software but the encoder fails with --TransquantBypassEnable=1 as reported by ticket #205 and is not included in the Draft Text as reported by ticket #333.
This contribution resolves both tickets.

It is reported that compared to HM16.19, lossless mode in VTM-5.0 shows gain of -6.08 % in compression ratio in All Intra configuration with 3422% encoding time increase and 177 % decoding time increase, XXX % (XXX%/XXX%) in RA and XXX % (XXX%/XXX%) in LDB.

# Introduction

As in HEVC, a lossless mode that bypass the transform and quantization steps may be enabled with the PPS-level flag “transquant_bypass_enabled_flag”.
Then if the PPS flag is enabled, a CU-level flag is coded “cu_transquant_bypass_flag” to indicate that the quantization, transform process and loop filter are bypassed for the current CU. 
As reported by ticket #205, the VTM-5.0 software crash when enabling lossless coding, this is because many tools/features included in the software are not compatible with lossless coding.

# Proposal

To support lossless coding, Luma mapping is disabled at slice level if PPS-level transquant_bypass_enabled_flag is true. Then Zero-Out transform is not used in case a CU is coded with lossless mode. LFNST and SBT may produce zero coefficient and Joint coding of Cb-Cr is not inversible so are also be disabled at CU-level if the CU-level cu_transquant_bypass_flag is true.

# Simulation results

The simulations show the results of VTM-5.0 software including the proposed changes versus the HM16.19.  The common test conditions and reference configurations specified in [1] are followed except that QP=0 is used.

Table 1: All Intra results over HM16.19

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 3.4 | 3.5 | -4.61% |
| Class B | 1.4 | 1.5 | -5.03% |
| Class C | 1.3 | 1.3 | -5.21% |
| Class D | 1.2 | 1.3 | -4.61% |
| Class E | 1.7 | 1.9 | -10.96% |
| Overall (w/o F) | 2.4 | 2.6 | -6.08% |
| Overall (w/ F) | 2.5 | 2.8 | -9.13% |
| Class F | 3.0 | 4.0 | -21.34% |
| Enc Time[%] | 3422% |  |  |
| Dec Time[%] | 177% |  |  |

Table 2: Random Access results over HM16.19

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 3.5 | 3.7 | -4.78% |
| Class B | 1.5 | 1.6 | -5.95% |
| Class C | 1.5 | 1.6 | -6.50% |
| Class D | 1.6 | 1.7 | -5.37% |
| Class E |  |  |  |
| Overall (w/o F) | #DIV/0! | #DIV/0! | #DIV/0! |
| Overall (w/ F) | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | 28.7 | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | 149% |  |  |

Table 3: Low Delay results over HM16.19

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A |  |  |  |
| Class B | 1.5 | #VALUE! | #VALUE! |
| Class C | 1.5 | 1.6 | -6.45% |
| Class D | 1.6 | 1.7 | -5.36% |
| Class E | 1.9 | 2.1 | -11.30% |
| Overall (w/o F) | #DIV/0! | #VALUE! | #VALUE! |
| Overall (w/ F) | #DIV/0! | #VALUE! | #VALUE! |
| Class F | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | 129% |  |  |

# Conclusion

This contribution proposes to include the suggested modifications to the software and the Draft text to support lossless coding.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Oct. 2019

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).