| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O461 |
| --- | --- |

| Title: | AHG13: On chroma tool evaluation in Dual Tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tangi Poirier, Fabrice Le Léannec, Franck Galpin, Edouard François 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Email: | tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to report separated Y,U and V BD rate numbers by separating luma and chroma rates in addition to numbers computed with global luma and chroma rate when Dual Tree is used.
CCLM and Joint CB-Cr coding tools are evaluated with this method in All Intra configuration, results are reported as follows: CCLM tool off leads to +1.86% increase  in BD-rate for Y, +15.80% for U and 15.73% for V, with the proposed evaluation method CCLM tool off leads to +0.01% for Y, +26.92% for U and 26.95% for V. Joint Cb-Cr coding tool off leads to 0.15% increase in BD-rate for Y, +1.61% for U and +2.01% for V, with the proposed evaluation method Joint Cb-Cr tool off leads to -0.01% decrease in BD-rate for Y, +2.92% increase for U and 3.31% for V.

# Problem statement

BD rate results are currently reported separately for luma and chroma channels. While trends are often similar for each channel, it is not uncommon for results to show divergence. At the last meeting it was proposed in [2] to compute combined YUV PSNR, however for chroma specific tools it does not reflect the actual performance for chroma BD rate. Typically, CCLM will reduce the rate and increase the PSNR for chroma but does not change the luma, or luma BD rate will decrease due to the reduce rate for chroma.

# Proposal

In this contribution, it is proposed to add a separate evaluation of luma and chroma BD rate by computing separately the luma and chroma rates in case of Dual Tree. PSNR before loop-Filtering is used to be agnostic to shared process for both luma and chroma channels.

# Simulation results

Simulation are conducted following the common tests conditions described in [1] on only 1 frame. Anchor is VTM-5.0, and tool-off test are run for CCLM and Joint Cb-Cr coding.

Table 1: CCLM off results in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5.44% | 38.89% | 33.90% | 100% | 101% |
| Class A2 | 1.94% | 15.79% | 8.80% | 96% | 97% |
| Class B | 0.96% | 11.08% | 16.18% | 98% | 98% |
| Class C | 1.53% | 13.08% | 14.96% | 97% | 100% |
| Class E | 0.17% | 4.22% | 4.79% | 98% | 101% |
| Overall | 1.86% | 15.80% | 15.73% | 98% | 99% |
| Class D | 0.71% | 10.44% | 11.93% | 98% | 102% |
| Class F (optional) | 2.66% | 16.56% | 17.28% | 99% | 99% |

Table 2: CCLM off results with the proposed method in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 61.24% | 53.22% | 100% | 101% |
| Class A2 | 0.03% | 23.84% | 18.21% | 96% | 97% |
| Class B | 0.02% | 20.16% | 26.94% | 98% | 98% |
| Class C | -0.01% | 28.04% | 30.28% | 97% | 100% |
| Class E | -0.05% | 5.42% | 5.03% | 98% | 101% |
| Overall | 0.01% | 26.92% | 26.95% | 98% | 99% |
| Class D | -0.04% | 14.63% | 17.94% | 98% | 102% |
| Class F (optional) | -0.01% | 44.48% | 58.35% | 99% | 99% |

Table 3:Joint Cb-Cr off results in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 1.71% | 0.58% | 98% | 102% |
| Class A2 | 0.16% | 3.29% | 0.81% | 96% | 99% |
| Class B | 0.10% | 0.82% | 1.96% | 98% | 97% |
| Class C | 0.27% | 2.09% | 2.76% | 97% | 104% |
| Class E | 0.04% | 0.50% | 3.71% | 97% | 101% |
| Overall | 0.15% | 1.61% | 2.01% | 97% | 100% |
| Class D | 0.09% | 2.56% | 5.33% | 96% | 104% |
| Class F (optional) | 0.01% | 3.22% | 5.14% | 98% | 96% |

Table 4: Joint Cb-Cr off results with the proposed method in All Intra configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 1.75% | 1.01% | 98% | 102% |
| Class A2 | 0.01% | 3.00% | 1.80% | 96% | 99% |
| Class B | 0.00% | 2.04% | 3.28% | 98% | 97% |
| Class C | -0.04% | 4.84% | 5.28% | 97% | 104% |
| Class E | -0.12% | 2.93% | 4.54% | 97% | 101% |
| Overall | -0.01% | 2.92% | 3.31% | 97% | 100% |
| Class D | -0.02% | 3.76% | 5.78% | 96% | 104% |
| Class F (optional) | -0.06% | 5.87% | 9.12% | 98% | 96% |

# Conclusion

A method to evaluate chroma specific tools is proposed in this contribution. This method computes the rate separately for luma and chroma in Dual Tree and compute the BD rates with this different rate for luma and chroma.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019

- F. Bossen, “On reporting combined YUV BD rates”, JVET-N0341, Geneva, CH, Mar.2019

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).