| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0462 |
| --- | --- |

| Title: | AHG12: Separated luma and chroma wavefront for Dual Tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier, Fabrice Le Léannec, Franck Galpin 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Email: | tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes a separated wavefront parallel processing (WPP) for Luma and Chroma CTU in Dual Tree. The first test proposes a separation of CABAC processing for luma and chroma, the method shows 0.01 % loss in AI. The second test combines usual WPP for parsing CTU lines together with the first method. It reportedly brings 0.26% BDrate difference over VTM5 in AI.
The third test proposes to separate the coding of SAO syntax for Luma and Chroma on top of test 1, the method shows XXX% in AI.

# Introduction

In [2], it has been proposed to use one CTU delay for wavefront parallel processing instead of two CTU delay as in HEVC to allow more parallel processing capabilities. It is asserted that Intra coded pictures have a higher rate and needs more CABAC cycles to be parsed.

In VTM-5.0, some contexts are used for both Luma Chroma CUs in case of Dual Tree such as the contexts used for the following syntax elements: Split flags, IBC flag, QtRootCbf, TransquantBypass flag, LFNST index.

# Proposal

In this contribution, a parallel processing of Luma and Chroma CTUs is proposed by separating the contexts for Luma and Chroma CU that are shared between component types in VTM5. There is no CABAC context probability value propagation between luma and chroma in the test 1. So Luma and Chroma syntax may be parsed in parallel, as shown in Figure 1.

Figure 1: Proposed separated Luma and Chroma processing for Dual Tree

This proposal can be easily combined with the usual wavefront parallel processing. In this case a one CTU line? delay propagation is used for both Luma and chroma CTUs, and there is no context probability value propagation between Luma and Chroma CTUs in test 2, as shown in Figure 2.

Figure 2: Proposed Separated CABAC context propagation for Luma and Chroma CTUs

To go further with the separation of luma and chroma, test 3 is built on top of test 1 where SAO syntax is separated between luma and chroma, this means that sao_merge_left_flag and sao_mege_above_flag are coded indepently (thus one flag by channel type) for Luma and chroma instead of being shared.

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-N1010 [1]. VTM-5.0 reference software is used with VTM configuration files.

Table 1: separated luma and chroma processing in All Intra

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.65% | -0.58% | 99% | 97% |
| Class A2 | 0.00% | -0.22% | -0.16% | 101% | 98% |
| Class B | 0.01% | -0.16% | -0.19% | 103% | 100% |
| Class C | 0.00% | 0.06% | 0.04% | 103% | 99% |
| Class E | 0.01% | 0.02% | -0.03% | 103% | 97% |
| Overall | 0.01% | -0.18% | -0.17% | 102% | 98% |
| Class D | 0.03% | 0.23% | 0.11% | 103% | 99% |
| Class F (optional) | -0.05% | -0.14% | 0.03% | 99% | 97% |

Table 2:Results of test 2 (WPP + separated luma chroma) in All Intra

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.54% | -1.01% | -0.64% | 102% | 100% |
| Class A2 | 0.23% | -0.65% | -0.13% | 100% | 98% |
| Class B | 0.20% | -0.43% | -0.24% | 101% | 99% |
| Class C | 0.13% | -0.01% | 0.09% | 99% | 94% |
| Class E | 0.30% | -0.14% | -0.36% | 100% | 97% |
| Overall | 0.26% | -0.42% | -0.24% | 100% | 98% |
| Class D | 0.12% | 0.22% | 0.09% | 99% | 93% |
| Class F (optional) | 0.10% | -0.16% | -0.18% | 97% | 104% |

Table 3: Results of test 3 (separated luma chroma + separated SAO) in All Intra

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | -0.18% | 0.76% | 103% | 98% |
| Class C | 0.01% | -0.07% | 0.48% | 104% | 98% |
| Class E | 0.01% | 0.03% | 0.07% | 103% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.05% | 0.15% | 0.12% | 104% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Note: a bug was found at the encoder for test 3, results are updated with the fixed version.

# Conclusion

This contribution proposes a new method to allow high level parallelism for Intra coded slices with Dual Tree. It may be combined with wavefront parallel processing without noticeable change in coding efficiency.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019

- T. Ikai, “AHG12: One CTU delay wavefront parallel processing”, JVET-N0150, Geneva, CH,          Mar.2019

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).