| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0465 |
| --- | --- |

| Title: | Non-CE6 : simplified SBT mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, Y. Chen, T. Poirier, K. Naser 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné | Tel: Email: | +33256856196 fabrice.leleannec@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the coding of the sub-block transform mode. In VTM-5.0, the cu_sbt_quad_flag, cu_sbt_horizontal_flag and cu_sbt_pos_flag syntax elements are context-based coded. This contribution proposes to code them in bypass mode.

The proposed simplification reportedly brings +0.01% Luma BD-rate loss in RA and 0.02% Luma BD-rate loss in LDB, over VTM-5.0 anchor.

It is suggested to adopt the proposed simplification into next VTM-6.0.

# Introduction

In VTM-5.0, the sub-block transform (SBT) mode is used to split an INTER coding unit into 2 TUs in a binary way. One of the 2 TUs has zero-residual, and  TU has non-zero residual. The SBT mode is signaled as follows.

- The ‘cu_sbt_flag’ flag indicates the use of SBT for a given INTER coding unit. It is context-based coded.

- The ‘cu_sbt_quad_flag’ flag indicates the type of CU partitioning into transform units: symmetric or asymmetric binary split. It is context-based coded.

- The ‘cu_sbt_horizonttal_flag’ flag indicates the orientation of binary split used for the considered CU: horizontal or vertical. It is context-based coded.

- The ‘cu_sbt_pos_fag’ flag indicates which TU has non-zero residual.

In this contribution, it is proposed to keep the coding of the first above flag ‘cu_sbt_flag’ unchanged, and to code the 3 other SBT-related flags in bypass mode.

# Experimental Results

The performances of the proposed simplification are shown by the following Table 1. As can be seen, +0.01% BD-rate loss is obtained in RA in Luma, and +0.02% in LDB.

Table 1: performance obtained with the proposed simplified SBT mode coding.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.17% | 0.09% | 97% | 102% |
| Class A2 | 0.01% | -0.02% | 0.07% | 99% | 104% |
| Class B | 0.02% | 0.01% | -0.01% | 94% | 101% |
| Class C | 0.01% | -0.17% | -0.13% | 94% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.01% | 96% | 100% |
| Class D | 0.02% | -0.12% | 0.24% | 97% | 91% |
| Class F | 0.03% | 0.02% | 0.01% | 98% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.23% | 0.24% | 103% | 105% |
| Class C | 0.05% | -0.25% | -0.21% | 104% | 114% |
| Class E | -0.02% | 0.10% | -0.14% | 104% | 110% |
| Overall | 0.02% | 0.04% | 0.00% | 104% | 109% |
| Class D | -0.06% | -0.69% | 0.35% | 103% | 111% |
| Class F | 0.04% | -0.04% | 0.06% | 108% | 110% |

# Proposed modification to VVC draft 5

Table 2 depicts the modifications brought by this contribution to the coding unit syntax table of VVC.

Table 2: proposed modification to the coding unit syntax table of VVC draft 5

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| …. // INTRA prediction syntax |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| …. // INTER prediction syntax |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| cu_sbt_quad_flag | u(1) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_horizontal_flag | u(1) |
| cu_sbt_pos_flag | ae(v) |
| cu_sbt_pos_flag | u(1) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

The following table shows the modification brought to the table 9-17 of VVC draft 5 of JVET-N1001, which specifies the CABAC context assigned to each bin of the VVC bit-stream syntax.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| cu_sbt_flag | ( cbWidth * cbHeight < 256 ) ? 1 : 0 | na | Na | na | na | na |
| cu_sbt_quad_flag | 0 Bypass | na | Na | na | na | na |
| cu_sbt_horizontal_flag | ( cbWidth = = cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 Bypass | na | Na | na | na | na |
| cu_sbt_pos_flag | 0 Bypass | na | Na | na | na | na |
| … | … | … | … | … | … | … |

# Conclusion

This contribution proposes to simplify the coding of the SBT mode, by coding only the first flag in context-based mode and coding all three remaining bins in bypass mode.

It is reported that +0.01% luma BD-rate loss over VTM-5.0 is obtained in Random Access with the proposed simplification and +0.02% loss is obtained in LDB.

It is suggested to adopt the proposed simplification into future VTM-6.0.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).