| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0467 |
| --- | --- |

| Title: | Non-CE3 : simplified MRL and ISP mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, Y. Chen, T. Poirier 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné | Tel: Email: | +33256856196 fabrice.leleannec@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the coding of the multiple reference line (MRL) index and the Intra Sub-Partition (ISP) mode.

In VTM-5.0, the MRL index is coded by the syntax element intra_luma_ref_idx, made of two context-based coded bins. This contribution’s method 1 proposes to code the second bin in bypass mode. It reportedly brings +0.01% Luma BD-rate loss in RA and +0.01% Luma BD-rate loss in AI over VTM-5.0.

Moreover, the ISP mode is signaled through two context-based coded flags, intra_subpartitions_mode_flag and intra_subpartitions_split_flag. The proposed method 2 codes the second flag intra_subpartitions_split_flag in bypass mode. When the method 2 is used on top of the method 1, it leads to 0.01% Luma BD-rate loss in RA and 0.02% loss in AI over VTM-5.0.

It is suggested to adopt the two proposed simplifications into next VTM-6.0.

# Introduction

In VTM-5.0, multiple reference line intra prediction is used to predict an INTRA block from a chosen line of reference samples. The intra_luma_ref_idx syntax element indicates the reference line used for intra prediction. It is made of two context-based coded bins. First bin indicates that a reference line is different from the default one (index 0) . If true, then the second bin indicates if the reference line index is equal to 1 or 3.

The method 1 proposed in this contribution codes the second bin of intra_luma_ref_idx in bypass mode.

Next, the second proposed method (method 2) consists in the following.

In VTM-5.0, the ISP mode consists in partitioning an INTRA CU into 2 or 4 TUs, depending on the CU size. The ISP mode is signaled through 2 flags: intra_subpartitions_mode_flag and intra_subpartitions_split_flag. The flag intra_subpartitions_mode_flag indicates the use of ISP. If true, then the flag intra_subpartitions_split_flag inidicates the horizontal or vertical split orientation.

The method 2 proposed in this contribution codes the second flag intra_subpartitions_split_flag in bypass mode.

# Experimental Results

The performances of the proposed simplification method 1 are shown by the following Table 1. As can be seen, +0.01% BD-rate difference over VTM-5.0 is obtained in RA in Luma, and +0.01% is obtained in AI.

Table 1: performances obtained with the first proposed simplification (method 1)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.04% | 102% | 101% |
| Class A2 | 0.00% | -0.05% | -0.02% | 102% | 103% |
| Class B | 0.01% | -0.06% | 0.03% | 100% | 105% |
| Class C | 0.01% | -0.07% | 0.04% | 97% | 103% |
| Class E | 0.01% | -0.09% | -0.04% | 100% | 101% |
| Overall | 0.01% | -0.06% | 0.01% | 100% | 103% |
| Class D | 0.00% | 0.08% | -0.04% | 102% | 108% |
| Class F | 0.00% | -0.04% | 0.02% | 101% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.16% | 0.07% | 103% | 103% |
| Class A2 | 0.04% | 0.10% | -0.01% | 101% | 102% |
| Class B | 0.02% | -0.02% | -0.03% | 101% | 107% |
| Class C | -0.04% | -0.10% | -0.19% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.05% | -0.05% | 102% | 104% |
| Class D | 0.03% | -0.02% | 0.05% | 101% | 102% |
| Class F | 0.02% | 0.11% | 0.11% | 98% | 99% |

Table 2: performances obtained when using the two proposed simplifications for MRL and ISP signaling (method 1 + method 2)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 103% | 104% |
| Class A2 | 0.02% | -0.02% | -0.05% | 102% | 104% |
| Class B | 0.02% | -0.01% | 0.05% | 102% | 109% |
| Class C | 0.02% | 0.00% | 0.08% | 98% | 103% |
| Class E | 0.02% | 0.04% | 0.02% | 101% | 104% |
| Overall | 0.02% | 0.00% | 0.02% | 101% | 105% |
| Class D | 0.01% | 0.32% | -0.07% | 99% | 104% |
| Class F | 0.03% | 0.05% | 0.08% | 102% | 106% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | 0.13% | 101% | 104% |
| Class A2 | 0.05% | 0.19% | 0.15% | 102% | 108% |
| Class B | 0.00% | -0.09% | -0.09% | 100% | 107% |
| Class C | -0.01% | -0.02% | -0.09% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | 0.00% | 101% | 105% |
| Class D | 0.05% | -0.17% | 0.12% | 99% | 98% |
| Class F | 0.01% | -0.17% | 0.24% | 100% | 102% |

# Modifications brought to VVC draft 5

## MRL-related modification to VVC draft 5 (JVET-N1001)

The following table shows the modification brought according to method 1 to the table 9-17 of VVC draft 5 of JVET-N1001, which specifies the CABAC context assigned to each bin of the VVC bit-stream syntax.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| intra_luma_ref_idx[ ][ ] | 0 | 1 bypass | na | na | na | Na |
| … | … | … | … | … | … | … |

## ISP-related modifications brought to VVC draft 5 (JVET-N1001)

Table 5 below shows the coding_unit syntax table modification according to the proposed method 2.

Table 5: modification brought by proposed method 2 to the coding_unit syntax table of JVET-N1001.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( cbWidth <= 32 && cbHeight <= 32 ) |  |
| intra_bdpcm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_bdpcm_flag[ x0 ][ y0 ] ) |  |
| intra_bdpcm_dir_flag[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mpm_idx[ x0 ][ y0 ] | ae(v) |
| Else |  |
| intra_mip_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | u(1) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| ………….. |  |
| ………….. |  |
| } |  |

The partial table below shows the CABAC context assignment table of VVC draft 5, as modified by the method 2.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| intra_subpartitions_mode_flag | 0 | na | na | na | na | Na |
| intra_subpartitions_split_flag | 0 bypass | na | na | na | na | Na |
| … | … | … | … | … | … | … |

# Conclusion

This contribution proposes (method 1) to simplify the coding of the MRL index, by coding only the first bin of syntax element intra_luma_ref_idx in context-based mode and coding the second bin in bypass mode.

It is reported that +0.01% luma BD-rate loss is obtained in Random Access with the proposed simplification and +0.01% loss is obtained in AI.

This contribution also proposes (method 2) to simplify the coding of the ISP mode, by coding the flag intra_subpartitions_split_flag in bypass mode.

It is reported that, when combined with method 1, method 2 brings +0.01% luma BD-rate loss in RA over VTM-5.0 and +0.02% loss in AI.

It is suggested to adopt the proposed simplification into future VTM-6.0.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).