| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0468 |
| --- | --- |

| Title: | Non-CE4 : simplified SMVD mode coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, Y. Chen, T.Poirier, F. Galpin 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné | Tel: Email: | +33256856196 fabrice.leleannec@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the coding of the SMVD mode.

In VTM-5.0, the sym_mvd_flag syntax element indicates the use of symmetrical motion vector difference coding on the CU level. It is coded in context-based mode.

This contribution proposes to code this flag in bypass mode. It reportedly brings +0.01% Luma BD-rate difference over VTM-5.0 in RA configuration.

It is suggested to adopt the proposed simplification into next VTM-6.0.

# Introduction

In VTM-5.0, the syntax element sym_mvd_flag indicates the use of SMVD to code the motion vector difference of a given bi-predicted CU. Some off-line analysis has shown that the average entropy of this binary symbol is very close to 1 bit of information, indicating that arithmetic coding of this bin is not likely to give too much benefit in terms of coding efficiency.

Therefore, this contribution proposes to code this syntax element in bypass mode.

# Experimental Results

The performances of the proposed simplification are shown by the following Table 1. As can be seen, +0.01% BD-rate difference over VTM-5.0 is obtained in RA in Luma. The proposed simplification has no impact on LDB and AI configuration.

Table 1: performances obtained with the first proposed simplification (method 1)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | 0.05% | 102% | 102% |
| Class A2 | 0.00% | 0.00% | 0.03% | 101% | 102% |
| Class B | 0.02% | 0.03% | 0.00% | 101% | 107% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | 0.02% | 101% | 104% |
| Class D | 0.02% | -0.01% | 0.09% | 99% | 98% |
| Class F | 0.00% | 0.03% | 0.01% | 100% | 98% |

# Modifications brought to VVC draft 5 (JVET-N1001)

Table 2: modification brought to the coding_unit syntax table of JVET-N1001

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( cbWidth = = 4 && cbHeight = = 4 && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && sps_ibc_enabled_flag && ( cbWidth != 128 ¦ ¦ cbHeight != 128 ) ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| ………….. // INTRA prediction syntax |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| general_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( general_merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| mvd_coding( x0, y0, 0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_amvr_enabled_flag && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ) ) { |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( sps_smvd_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |
| sym_mvd_flag[ x0 ][ y0 ] | u(1) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| ….. |  |
| ….. |  |
| } |  |

The following table shows the modification brought according to method 1 to the table 9-17 of VVC draft 5 of JVET-N1001, which specifies the CABAC context assigned to each bin of the VVC bit-stream syntax.

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … | … | … | … | … | … | … |
| sym_mvd_flag[ ][ ] | 0 bypass | na | na | na | na | na |
| … | … | … | … | … | … | … |

# Conclusion

This contribution proposes to simplify the coding of the SMVD flag, by coding the sym_mvd_flag syntax element in bypass mode.

It is reported that +0.01% luma BD-rate loss is obtained in Random Access with the proposed simplification.

It is suggested to adopt the proposed simplification into future VTM-6.0.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).