| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O 0469_v4 |
| --- | --- |

| Title: | Non-CE3: Simplification of MIP MPM Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Jason Jung, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 dongcheol.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

In this contribution, it is reported that three simplification methods of MPM derivation are proposed as follows : (1) For MPM derivation of a normal intra block, the Planar mode is used when the neighbor block is a MIP intra block, (2) For MPM derivation of a MIP intra block, the predefined mode per MipSizeId is used when the neighbor block is a normal intra block, (3) the combination of methods (1) and (2). It is reportedly shown that the method (1) provides overall BD-rate of 0.01x%, -0.02%, 0.04% with 100%  encoding and 101% decoding time under AI configuration, and 0.01%, 0.09%, 0.06% with 101% encoding and 99% decoding time under RA configuration. The method (2) provides overall BD-rate of 0.00%, -0.02%, 0.00% with 100% encoding and 100% decoding time under AI configuration, and 0.01%, -0.04%, -0.10% with 101% encoding and 100% decoding time under RA configuration. The method (3) provides overall BD-rate of 0.02%, 0.01%, 0.02% with 100% encoding and 100% decoding time under AI configuration, and 0.01%, 0.13%, 0.06% with 101% encoding and 100% decoding time under RA configuration. Therefore, it is recommended to adopt the proposed method which simplifies MPM derivation procedure.

Introduction

Matrix-based Intra Prediction (MIP) is adopted in VVC WD 5.0 [1].

In the current VVC WD 5.0 spec., if the current coding block is a MIP intra block and one of neighbouring blocks is a normal intra block, the intra prediction mode of neighbouring block is converted to MIP mode based on MipSizeId in Table 8-1–Specification of mapping between intra prediction and MIP modes in [1]. The MipSizeId is defined as follows:

- MipSizeId  0 : if both cbWidht and cbHeight are equal to 4

- MipSizeId  1 : if both cbWidth and cbHeight are less than or equal to 8

- MipSizeId  2: if both cbWidht and cbHeight are greater than 8

If the current coding block is a normal intra block and one of neighbouring blocks is a MIP intra block, the MIP mode of neighbouring block is converted to normal intra mode based on Table 8-4–Specification of mapping between MIP mode and intra prediction modes in [1]. The DM mode of the intra chroma prediction is derived with the same manner using the Table 8-4.

Therefore, each intra prediction block related to MIP prediction has to access one of the look-up tables for MPM derivation.

Proposed Solutions

In this contribution, the simplifications of MPM derivation are proposed when MIP is enabled as follows:

Method#1: Only the Planar mode is used when the current block is a normal intra block and one of neighbouring blocks is a MIP intra block.

Method#2: A predefined mode per MipSizeId is used when the current block is a MIP intra block and one of neighbouring blocks is a normal intra block as follows:

{MipSizeId 0  17,

MipSizeId 1  0,

MipSizeId 2  1}.

Method#3: Method#1 + Method#2

Experimental Results

The simulations are performed according to common test conditions (CTC) as described in [3]. AI and RA results are reported.

Table 1: Results of proposed method#1 over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% | 99% | 100% |
| Class A2 | 0.03% | 0.02% | -0.02% | 99% | 100% |
| Class B | 0.01% | -0.04% | 0.09% | 100% | 101% |
| Class C | 0.00% | -0.12% | 0.05% | 100% | 101% |
| Class E | 0.03% | 0.01% | 0.03% | 100% | 101% |
| Overall | 0.01% | -0.02% | 0.04% | 100% | 101% |
| Class D | -0.03% | 0.19% | 0.00% | 100% | 101% |
| Class F | -0.02% | -0.02% | 0.08% | 102% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.22% | 101% | 99% |
| Class A2 | 0.07% | 0.22% | 0.06% | 101% | 99% |
| Class B | 0.01% | 0.04% | 0.01% | 101% | 99% |
| Class C | -0.02% | 0.15% | 0.01% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 101% | 99% |
| Class D | 0.00% | 0.07% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.13% | 0.17% | 101% | 99% |

Table 2: Results of proposed method#2 over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | -0.03% | 101% | 100% |
| Class A2 | 0.00% | -0.01% | -0.05% | 100% | 100% |
| Class B | 0.01% | -0.06% | 0.03% | 100% | 100% |
| Class C | -0.01% | -0.03% | -0.01% | 100% | 100% |
| Class E | -0.01% | -0.02% | 0.03% | 100% | 100% |
| Overall | 0.00% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.08% | -0.15% | 100% | 100% |
| Class F | 0.01% | -0.03% | 0.06% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | 0.10% | 101% | 99% |
| Class A2 | 0.02% | 0.07% | -0.05% | 101% | 100% |
| Class B | 0.01% | -0.03% | -0.17% | 101% | 100% |
| Class C | -0.01% | -0.11% | -0.21% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.04% | -0.10% | 101% | 100% |
| Class D | 0.03% | 0.22% | -0.07% | 100% | 100% |
| Class F | -0.02% | -0.04% | 0.12% | 100% | 100% |

Table 3: Results of proposed method#3 over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.14% | 0.07% | 100% | 99% |
| Class A2 | 0.03% | -0.03% | -0.01% | 99% | 99% |
| Class B | 0.01% | -0.06% | 0.02% | 99% | 100% |
| Class C | 0.00% | 0.02% | 0.01% | 100% | 99% |
| Class E | 0.03% | 0.04% | 0.00% | 99% | 100% |
| Overall | 0.02% | 0.01% | 0.02% | 100% | 100% |
| Class D | 0.00% | 0.21% | -0.08% | 99% | 100% |
| Class F | -0.02% | 0.04% | 0.05% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.09% | 0.27% | 101% | 100% |
| Class A2 | 0.04% | 0.28% | 0.15% | 101% | 100% |
| Class B | 0.01% | 0.12% | -0.21% | 101% | 100% |
| Class C | -0.03% | 0.05% | 0.19% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.13% | 0.06% | 101% | 100% |
| Class D | 0.02% | 0.15% | 0.40% | 100% | 101% |
| Class F | 0.02% | -0.11% | 0.12% | 100% | 101% |

Conclusion

In this contribution, it is reported that three simplification methods of MPM derivation are proposed as follows : (1) Planar mode is used for MPM derivation of the normal intra block when the neighbouring block is MIP applied block, (2) A predefined mode per MipSizeId is used for MPM derivation of the MIP applied block, (3) the combination of (1) and (2). It is reportedly shown that the method (1) provides overall BD-rate by 0.01%, -0.02%, 0.04% with 100% overall encoding time and 101% overall decoding time under All Intra (AI) configuration and 0.01%, 0.09%, 0.06% with 101% overall encoding time and 99% overall decoding time under Random Access (RA) configuration. The method (2) provides overall BD-rate by 0.00%, -0.02%, 0.00% with 100% encoding and 100% decoding time under AI configuration, and 0.01%, -0.04%, -0.10% with 101% encoding and 100% decoding time under RA configuration. The method (3) provides overall BD-rate by 0.02%, 0.01%, 0.02% with 100% encoding and 100% decoding time under AI configuration, and 0.01%, 0.13%, 0.06% with 101% encoding and 100% decoding time under RA configuration. Therefore, it is recommended to adopt the proposed method to the next version of the VVC draft spec.

References

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).