| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0471 |
| --- | --- |

| Title: | AHG8/AHG12: Layer concepts clarifications and improvements for immersive media use cases |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas, Alexandre Gabriel | Tel: Email: | {emmanuel.thomas¸ alexandre.dasilvapratasgabriel}@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution discusses the concept of layer introduced at JVET 14th based on JVET-N0278 [1] and proposes clarification as well as modifications.

In the first section, this contribution presents an analysis of the layer concept as understood from the VVC Draft v5 [2].

In the second section, clarifications are proposed for the definition of coded picture and the scope of the layer identifier.

In the last section, this contribution suggests modifications to the current VVC draft, based on the motivations explained in JVET-N0278 [1] coming from immersive media use cases, which are:

- Relaxing constraints on coded layer video sequence start picture alignment

- Removing layer ordering by increased value of NuhLayerId in access unit

# Introduction

This contribution discusses the concept of layer introduced at JVET 14th based on JVET-N0278 [1] and proposes clarification as well as modifications.

In the first section, this contribution presents an analysis of the layer concept as understood from the VVC Draft v5 [2].

In the second section, clarifications are proposed for the definition of coded picture and the scope of the layer identifier.

In the last section, this contribution suggests modifications to the current VVC draft to better support immersive media use cases as originally motivated by JVET-N0278 [1] which are:

- Relaxing constraints on coded layer video sequence start picture alignment

- Removing layer ordering by increased value of NuhLayerId in access unit

Revisions

- r1 fixes JVET document template issues: adding title back and 'status'

# Analysis of bitstream concepts in VVC Draft v5

The following definitions and semantics are extracted from the VVC Draft v5 [2].

  - coded video sequence (CVS): A sequence of access units that consists, in decoding order, of a CVSS access unit, followed by zero or more access units that are not CVSS access units, including all subsequent access units up to but not including any subsequent access unit that is a CVSS access unit.

  - access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain exactly one coded picture for each present layer access unit.

  - layer: A set of VCL NAL units that all have a particular value of NuhLayerId and the associated non-VCL NAL units.

nuh_layer_id_plus1 minus 1 specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id_plus1 shall be in the range of 1 to 126, inclusive. The value of 127 may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream, decoders shall ignore all data that follow the value 127 for nuh_layer_id_plus1 in a NAL unit.

NOTE 7 – The value of 127 for nuh_layer_id_plus1 may be used to indicate an extended layer identifier in a future extension of this Specification.

The variable NuhLayerId is derived as follows:

NuhLayerId = nuh_layer_id_plus1 − 1		(7-3)

The value of NuhLayerId shall be the same for all VCL NAL units of a coded picture. The value of NuhLayerId of a coded picture is the value of the NuhLayerId of the VCL NAL units of the coded picture.

  - layer access unit: A set of NAL units for which the VCL NAL units all have a particular value of NuhLayerId, that are associated with each other according to a specified classification rule, that are consecutive in decoding order, and that contain exactly one coded picture.

  - coded layer video sequence (CLVS): A sequence of layer access units that consists, in decoding order, of a CLVSS layer access unit, followed by zero or more layer access units that are not CLVSS layer access units, including all subsequent layer access units up to but not including any subsequent layer access unit that is a CLVSS layer access unit.

  - coded picture: A coded representation of a picture comprising VCL NAL units with a particular value of NuhLayerId within an access unit and containing all CTUs of the picture.

According to the above definitions, Figure 1 represents the relations between the different concepts. The following conventions are used:

- A bitstream concept is represented by a rectangle

- The name of the concept is written in the associated rectangle

- A concept part of another one is represented by a nested rectangle

- The cardinality of a concept is indicated by a symbol at the bottom right corner of the rectangle with the convention of  '1': single occurrence and '+': one ore more occurences

Figure 1 - Conceptual model of VVC bitstream concept

Figure 2 represents the relation between CVS and CLVS in the bitstream.

Figure 2 - Bitstream arrangement of CVS and CLVS

# Proposed clarifications in VVC Draft v5

## On coded picture definition

  - coded picture: A coded representation of a picture comprising VCL NAL units with a particular value of NuhLayerId within an access unit and containing all CTUs of the picture.

Issues

Does "particular value" mean the same value? Is "access unit" correct?

From this definition, it is not clear whether a coded picture is part of access unit or part of a layer access unit as the definition of the layer access unit intends to define.

Proposal

  - coded picture: A coded representation of a picture comprising VCL NAL units with the same value of NuhLayerId within a layer access unit and containing all CTUs of the picture.

## On layer identifier scope

Issues

From the above definitions, the scope of a layer identifier is unclear. It should be explicitly defined when two layers with the same value of their identifier can be inferred to be the same layer.

The scope of the layer has a direct impact on bitstream extraction and merging operations and thus should be clarified. For instance, if the CVS is the scope then these operations are defined per CVS and should stop and the end of a CVS since the values of layer identifier in the next CVS may be equal but would not define the same layer as in the previous CVS.

Proposal

There seems to be two possible scopes for layer identifier, and thus layer existence:

- Option 1: CVS scope

- Option 2: bitstream scope

One of those option 	should be clearly defined in the text.

# Proposed modifications to the layer concept

## On start picture

Normative statement (7.4.2.4.3)

"The first access unit of a CVS is a CVSS access unit, wherein each present layer access unit is a CLVSS layer access unit, which is either an IRAP layer access unit with NoIncorrectPicOutputFlag equal to 1 or a GRA layer access unit with NoIncorrectPicOutputFlag equal to 1."

Consequence

This mandates that all the layers have aligned start pictures anywhere in the bitstream.

Normative statement (7.4.2.4.3)

"It is a requirement of bitstream conformance that, when present, each layer access unit in the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP layer access unit, which may be an IDR layer access unit or a CRA layer access unit, or a GRA layer access unit. "

Consequence

This requires that all the CLVS are stopped and restarted whenever one CLVS has ended.

Discussion

In the context of immersive media, as pointed in JVET-N0278 [1], the MPEG output document N18134 mentions that:

In the context of these requirements, an “Object” is defined as a visual element that is independently decodable and represented by its own fully conformant bitstream. This is a draft definition that implies that there is no “subbitstream” or something similar.

This wording implies a great flexibility in the way the "objects", or better called layer in the context of the VVC draft text, can be encoded and decoded independently from each other. As a result, it appears too restricting to require the alignment of start pictures across layers as well as the alignment of the ends of CLVS.

Proposal

It is proposed to remove the constraint on CLVS start picture alignment anywhere but in the first access unit of the first CVS and to remove the constraint on the alignment of the end of CLVS.

## On layer ordering

Normative statement (7.4.2.4.4)

"An access unit consists of one or more layer access units in increasing order of NuhLayerId."

Discussion

Since this new concept of layer in the VVC draft does not express any hierachry between the layers, it seems odd to require any ordering of the layer access units in the access unit. As a matter of fact, this may actually complexify the merging operation to obey this strict ordering.

Proposal

Since this ordering of layers does not seem to be motivated, it is proposed to remove it.

# Conclusion

In order to better support immersive media use cases as originally motivated by JVET-N0278 [1], we recommend to adopt the proposed text clarifications as well the technical changes described above which are:

- Relaxing constraints on coded layer video sequence start picture alignment

- Removing layer ordering by increased value of NuhLayerId in access unit

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- AHG17/AHG15: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134), Jill Boyce, JVET-N0278, Geneva, CH, 19–27 March 2019

- Versatile Video Coding (Draft 5), Benjamin Bross, Jianle Chen, Shan Liu, JVET-N1001, Geneva, CH, 19–27 Mar. 2019