| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, MO, 9-18 Jan. 2019 | Document: JVET-O0473 |
| --- | --- |

| Title: | AHG8: On generating final output picture from independent VVC layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas, Alexandre Gabriel | Tel: Email: | {emmanuel.thomas, alexandre.dasilvapratasgabriel}@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JVET-N0411 [1]. The motivation behind this proposal is to enable use cases involving bitstreams aggregations while limiting the amount of bitstream rewriting operation and providing an interface with the application.

To this end, the Layer Parameter Set (LPS) is proposed to signal the spatial relationship between a given independent VVC layers and its neighbors.

It is asserted that:

- Signaling spatial relationship of independent VVC layers is beneficial for these uses cases

- The spatial relationship between a given independent VVC layers and its neighbors fulfills the need of signaling in the use cases

- Signaling this relative spatial relationship between independent VVC layers prevents degenerative signaling (e.g. resolution scaling, samples overlapping, etc..) compared to signaling absolute spatial relationship in luma samples

- The relative spatial relationship signaled in each layer (e.g. SPS) limits the rewriting to the neighboring layers when adding/removing independent VVC layers

Compared to JVET-N0411, this contribution provides updates on the envisioned decoding steps for generating the final picture using the relative positioning information of the independent VVC layers.

It also provides a new diagram for a layered VVC HRD buffer model implementing the generation of such an output picture.

# Introduction

This contribution is a follow-up of JVET-N0411 [1] submitted at the previous meeting JVET #14. The motivation behind this proposal is to enable use cases involving bitstreams aggregations with the least amount of bitstream rewriting possible as well as interoperable interface with the application.

To this end, the Layer Parameter Set (LPS) is proposed to signal the spatial relationship between a given independent VVC layers and its neighbors

It is asserted that:

- Signaling spatial relationship of independent VVC layers is beneficial

- The spatial relationship between a given independent VVC layers and its neighbors fulfills the need of signaling in the use cases

- Signaling this relative spatial relationship between independent VVC layers prevents degenerative signaling (e.g. resolution scaling, samples overlapping, etc..) compared to signaling absolute spatial relationship in luma samples

- The relative spatial relationship signaled in each layer (e.g. SPS) limits the rewriting to the neighboring layers when adding/removing independent VVC layers

Compared to JVET-N0411, this contribution provides updates on the envisioned decoding steps for generating the final picture using the relative positioning information of the independent VVC layers. It also provides a new diagram for a layered VVC HRD buffer model implementing the generation of such an output picture.

# Problem Statement

There are many relevant use cases where a fixed tile structure is useful over time, some of these cases include, but are not limited to, video conferencing, security cameras, 360 video and panorama streaming. These use cases require adaptability as user preferences may change (user turns their head in a 360 video resulting in different tiles being requested) or the content itself changes natures (user speaking is brought to a more visible location on screen in a conf call).

Figure 1 – Multiple videos received by user resulting in one composite image or a panorama divided into multiple view where a user may only want 1 or 2 tiles

These examples show that a higher-level partitioning of the frames over time is a benefit worth having, and with this in mind the proposal of Layers. Previous contributions have been made explaining in more detail the situations where this can be useful (as well as an explanation of different layout signaling possibilities):

- JVET-L0202

- JVET-N0528

- JVET-N0411

# Proposal

## Goal: Signaling the relative positioning of multiple independent VVC layers

The following image is an example of multiple independent VVC layers having spatial relationships.

This contribution reminds an already proposed high-level syntax (see JVET-N0411 [1]) where the information regarding the spatial relationships between these layers are signaled in new parameter sets, Layer Parameter Set (JPS), attached to each SPS of each layer. The LPS of a given layer describes the neighboring layers in the direction north, east, south and west. This way, a decoder may derive the position in luma sample of each layer and then assemble the decoded samples into a single output picture as expected by the application.

## High-level Syntax: Layer Parameter Set extending SPS

As reminder of previous contribution JVET-N0411 [1], it is proposed to define a new parameter set to signal the spatial relationship of independent VVC layers. In addition, the spatial relationship is relative to a given layer, that is the LPS describes the neighboring layers of the associated layer. As a result, since the information is relative to the layer, it is proposed to attach it to the given layer. Adding the position information of the layer in the SPS would require updating the SPS every time the layout of the layers changes which is not desirable. It is thus proposed to define the Layer Parameter Set (LPS) as an extension of the SPS. This way, the information of the LPS can change without updating the SPS.

| layer_parameter_set( ) { | Descriptor |
| --- | --- |
| lps_sequence_parameter_set_id | ue(v) |
| boundary_identifier_length_minus1 | ue(v) |
| boundary_identifier_north | u(v) |
| boundary_identifier_east | u(v) |
| boundary_identifier_south | u(v) |
| boundary_identifier_west | u(v) |
| } |  |

sps_video_parameter_set_id specifies the value of sps_decoding_parameter_set_id for the SPS referred to by the LPS.

boundary_ identifier_length_minus1 plus 1 specifies the number of bits used to represent the syntax elements boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ] and boundary_identifier_west[ i ]. The value of boundary_id_length_minus1 shall be in the range of 0 to 15, inclusive.

boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ], boundary_identifier_west[ i ] specifies the identifier respecitvely at the north, east, south and west boundary of the decoded picture of the i-th layer within the output picture. The length of the boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ] and boundary_identifier_west[ i ] syntax elements is boundary_id_length_minus1 + 1 bits.

When the values of the pair boundary_identifier_north[ i ] and boundary_identifier_south[ j ] or the values of the pair boundary_identifier_east[ i ] and boundary_identifier_west[ j ] are equal the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the output picture and they share a common boundary respectively at the boundary north/south and east/west. Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the output picture.

## Decoding steps

Below are the main decoding steps expected to be performed:

- Parse VPS and identift the included layers in the bitstream

- Parse SPS of each layer to determine their sizes in luma samples

- Parse LPS of each layer to determine the spatial layout

- Derive the total size in luma sample to hold the final output picture

- Derive the position in luma sample of each layer in the final picture

- Decode the layer video data

- Output final picture (optional cropping of the decoded picture of each layer)

### Step 1: Parse VPS and identify the included layers in the bitstream

The first step is to parse the VPS as defined in the latest VVC draft:

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The parameters vps_max_layers_minus1 and vps_included_layer_id[ i ] allows to determine how many layers are expected and their id which allows to allocate in memory the adequate number of SPS and LPS to be parsed in the next steps.

### Step 2: Parse SPSs of each layer to determine their sizes in luma samples

Each layer contains a SPS. The SPS indicates the size the decoded picture of the layer via the parameters pic_width_in_luma_samples, pic_height_in_luma_samples and optionally conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, conf_win_bottom_offset.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| profile_tier_level( sps_max_sub_layers_minus1 ) |  |
| gra_enabled_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Step 3: Parse LPS of each layer to determine the spatial layout

The decoder must parse the LPSs to derive the layout of the layers.

| layer_parameter_set( ) { | Descriptor |
| --- | --- |
| lps_sequence_parameter_set_id | ue(v) |
| boundary_identifier_length_minus1 | ue(v) |
| boundary_identifier_north | u(v) |
| boundary_identifier_east | u(v) |
| boundary_identifier_south | u(v) |
| boundary_identifier_west | u(v) |
| } |  |

When the values of north and south identifiers are the equal for two different layers, then these layers are adjacent, the same applies to east and west.

Let's assume the following example:

Figure 2 - Example of layer alignment

How these layers are then spatially aligned is described below:

- Layers vertically adjacent (north/south) – Layer 1 and 2 (bottom identifier of 2 and top identifier of the have same value)

  - The decoder will place layer 1’s top left pixel below Layer 2’s bottom left pixel given the rule that north/south neighbors are aligned along their west boundary (cf. LPS syntax in 3.2).

- Layers horizontally adjacent (east/west) – Layer 3 and 1 (right identifier of 3 and left identifier of 1 have same value)

  - The decoder will place layer 1’s top left pixel to the right of layer 3’s top right pixel given the rule that east/west neighbors are aligned along their north boundary (cf. LPS syntax in 3.2).

### Step 4: Derive the total size in luma sample to hold the final output picture

The decoder has now in memory the size in luma sample of all the layers and the layout obtained at the previous step. Based on this, the decoder can now deduce the width and height in luma samples of the final output picture.

Let's take the following example of four layers in such an arrangement.

In this case, one can derive (assuming without conformance window):

FinalPicWidthInLumaSample = PicWidthLayerAInLumaSample + PicWidthLayerBInLumaSample

FinalPicHeightInLumaSample = PicHeightLayerAInLumaSample + PicHeightLayerCInLumaSample

### Step 5: Derive the position in luma sample of each layer in the final picture

Based on the layout obtained at step 3 and the total width and height calculated at the previous step, the decoder can now determine the position of each layer in the total picture.

PictureLayerATopLeftPixelPosX = 0

PictureLayerATopLeftPixelPosY = 0

PictureLayerBTopLeftPixelPosX = PicWidthLayerAInLumaSample

PictureLayerBTopLeftPixelPosY = 0

PictureLayerCTopLeftPixelPosX = 0

PictureLayerCTopLeftPixelPosY = PicHeightLayerAInLumaSample

PictureLayerDTopLeftPixelPosX = PicWidthLayerCInLumaSample

PictureLayerDTopLeftPixelPosY = PicHeightLayerAInLumaSample

### Step 6: Decode the layer video data

The decoder can either decode each layer sequentially or in parallel. Once the decoder has decoded a layer, it can place it in the decoded picture buffer.

### Step 7: Final Picture output (optional cropping of the decoded picture layer)

Based on the total width and height calculated at step 4 and the positions of each layer at step 5, the decoder can generate the final picture.

Here is an example of the final picture being filled up sequentially.

Figure 3 - Sequential decoding example of layer and the corresponding final output picture

Note that optional cropping would occur before placing the decoded picture of a layer into the final output picture.

# Discussion on HRD operations

Below is the HRD buffer model currently in the latest VVC draft.

Figure 4 - VVC HRD buffer model

The decoding steps described in this contribution imply the following design of a layered VVC HRD model as depicted in Figure 5.

Figure 5 - Layered VVC HRD model

# Conclusion

We recommend the adoption of the signaling of spatial relationship between independent VVC layers using the relative positioning as proposed in the new Layer Parameter Set (LPS). In addition, we recommend the inclusion in the VVC draft of the proposed diagram for a layered VVC HRD model.

# References

- JVET-N04111, AHG12: Layout signalling of independent coded regions, Emmanuel Thomas (TNO), 14th Meeting: Geneva, CH, 19–27 March 2019

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).