| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0474-v3 |
| --- | --- |

| Title: | “Non-CE6: Mismatch between text specification and reference software on MTS for IBC-coded block” |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung, Dongcheol Kim, Geonjung Ko Ju-Hyung (John) Son, Jin Sam Kwak (WILUS) Bumshik Lee (Chosun Univ.) | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com bslee@chosun.ac.kr |
| Source: | WILUS Inc. and Chosun University |  |  |

_____________________________

- Abstract

This contribution reports a mismatch between text specification and reference software on MTS (Multiple Transform Selection) for IBC (Intra Block Copy) coded block. In the current specification, MTS can be applied to IBC-coded block when MTS for inter is enabled in SPS-level, while MTS is not applied to IBC-coded block in VTM-5.0 even though the SPS flag is equal to 1. In this proposal, test results of enabling MTS for IBC-coded block are presented. Experimental results show that aligning VTM-5.0 with the current spec text achieves

- For Class F with AI: -0.01% (Y), 111% (EncT), 100% (DecT)

- For Class F with RA: 0.02% (Y), 102% (EncT), 100% (DecT)

- For Class F with LDB: 0.02% (Y), 101% (EncT), 101% (DecT)

- For TGM 4:2:0 with AI: -0.03% (Y), 106% (EncT), 100% (DecT)

- For TGM 4:2:0 with RA: 0.02% (Y), 101% (EncT), 100% (DecT)

- For TGM 4:2:0 with LBD: 0.15% (Y), 102% (EncT), 100% (DecT)

Considering the test results, it is shown that the current MTS scheme is not quite beneficial to IBC-coded block. Therefore, it is proposed to change the specification text as disabling MTS for IBC-coded block to align with the VTM reference software.

Introduction

In the current text specification [1], MTS can be applied to IBC-coded blocks when MTS is enabled for inter (sps_explicit_mts_inter_enabled_flag is equal to 1) as shown in Table 1. Meanwhile, after VTM-4.0 release, there was the bug report #196 in JVET VVC bug tracker that encoding result mismatch between RA parallel and single RA experiments was reported for Class F sequences when MTS is enabled for inter [2]. After the fix of #196, MTS is disabled for IBC-coded block regardless of sps_explicit_mts_inter_enabled_flag in the reference software.

Table 1. Transform unit syntax structure

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| ⋯ |  |
| } |  |

In this proposal, test results with enabling MTS for IBC-coded block are presented.

Experimental results

Aligning VTM-5.0 [3] with text specification regarding MTS for IBC-coded block is tested under the common test condition (CTC) [4] with inter MTS enabled (--MTS=3). Table 2 shows experimental results of enabling MTS for IBC-coded block compared to VTM-5.0.

Table 2. Coding performance of enabling MTS for IBC-coded block

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0+MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrillText | -0.05% | 0.27% | 0.15% | 112% | 99% |
| ArenaOfValor | 0.00% | 0.05% | -0.06% | 114% | 100% |
| SlideEditing | 0.03% | 0.14% | 0.08% | 107% | 99% |
| SlideShow | -0.03% | 0.05% | 0.31% | 111% | 100% |
| Class F | -0.01% | 0.13% | 0.12% | 111% | 100% |
| FlyingGraphics | -0.05% | 0.00% | -0.02% | 108% | 100% |
| Desktop | -0.05% | 0.05% | 0.02% | 104% | 100% |
| Console | 0.04% | 0.08% | 0.01% | 105% | 98% |
| ChineseEditing | -0.05% | 0.03% | 0.01% | 106% | 100% |
| Class TGM | -0.03% | 0.04% | 0.00% | 106% | 100% |
| Overall (F, TGM) | -0.02% | 0.08% | 0.06% | 108% | 100% |
|  | Random Access Main10 |  |  |  |  |
|  | Over VTM-5.0+MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrillText | 0.04% | -0.03% | 0.05% | 102% | 100% |
| ArenaOfValor | -0.02% | -0.01% | -0.04% | 102% | 100% |
| SlideEditing | -0.02% | -0.06% | 0.18% | 102% | 100% |
| SlideShow | 0.08% | 0.10% | 0.52% | 103% | 100% |
| Class F | 0.02% | 0.00% | 0.18% | 102% | 100% |
| FlyingGraphics | -0.07% | 0.02% | -0.17% | 104% | 100% |
| Desktop | 0.05% | 0.01% | -0.05% | 101% | 101% |
| Console | 0.09% | 0.21% | 0.19% | 101% | 99% |
| ChineseEditing | 0.01% | 0.02% | 0.03% | 101% | 99% |
| Class TGM | 0.02% | 0.07% | 0.00% | 101% | 100% |
| Overall (F, TGM) | 0.02% | 0.03% | 0.09% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0+MTS3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| BasketballDrillText | 0.02% | -0.92% | -0.76% | 102% | 99% |
| ArenaOfValor | 0.13% | 0.02% | -0.36% | 102% | 102% |
| SlideEditing | 0.23% | 0.28% | 0.43% | 100% | 102% |
| SlideShow | -0.30% | 0.90% | 2.46% | 102% | 102% |
| Class F | 0.02% | 0.07% | 0.44% | 101% | 101% |
| FlyingGraphics | -0.02% | -0.04% | -0.10% | 105% | 100% |
| Desktop | 0.29% | 0.48% | 0.24% | 100% | 100% |
| Console | 0.19% | 0.08% | 0.20% | 101% | 101% |
| ChineseEditing | 0.13% | 0.17% | -0.05% | 102% | 102% |
| Class TGM | 0.15% | 0.17% | 0.07% | 102% | 100% |
| Overall (F, TGM) | 0.08% | 0.12% | 0.26% | 102% | 101% |

Proposed WD text

7.3.7.10  Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( transform_skip_enabled_flag && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( (( CuPredMode[ x0 ][ y0 ] != = MODE_INTRA MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag )) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| ⋯ |  |
| } |  |

Conclusion

This contribution reports a mismatch between text specification and reference software on MTS for IBC-coded block. Experimental results show that coding efficiency gain from allowing MTS for IBC-coded block as specified in the specification text is marginal. Therefore, it is recommended to align text specification with reference software by disabling MTS for IBC-coded block in the next version of VVC draft specification.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, 14th meeting, Geneva, CH, 19–27 Mar. 2019.

- Bug report #196 in JVET VVC bug tracker, https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/196

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, CH, 19-27 Mar. 2019

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Chosun Univ. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).