| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0475 |
| --- | --- |

| Title: | Non-CE6: Context Simplification for Subblock Transform (SBT) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeeyoon Park (SKKU) Byeungwoo Jeon (SKKU) 2066 Seoburo, Jangangu Suwon, Korea | Tel: Email: | +82-31-290-7186 jiyoonpark@skku.edu bjeon@skku.edu |
| Source: | Sungkyunkwan University (SKKU) |  |  |

_____________________________

- Abstract

This contribution proposes context simplification for cu_sbt_horizontal_flag in the VVC 5.0 by reducing the number of its contexts from 3 to 2. This simple context reduction shows an average of Y(0.01%), U(-0.02%) and V(-0.02%) BDBR for RA; and an average of Y(0.04%), U(0.04%) and V(-0.13%) BDBR for LD. This simplification shows trivial coding efficiency change, especially in Class A1 and A2.

Introduction

In the current VVC 5.0 [1], the cu_sbt_horizontal_flag which specifies division direction of CU into subblocks related to the subblock transform (SBT) uses 3 context models (0/1/2) as shown in Table 1.

Table 1 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_sbt_horizontal_flag | ( cbWidth == cbHeight ) ? 0 : ( cbWidth < cbHeight ) ? 1 : 2 | na | na | na | na | na |

Proposed Simplification of Contexts

It is proposed to simplify the context modeling on cu_sbt_horizontal_flag by reducing the 3 contexts (0/1/2) in VVC 5.0 [1] to 2 contexts (0/1) as shown in Table 2.

Table 2 – Proposed Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_sbt_horizontal_flag | ( cbWidth < cbHeight ) ? 0 : 1 | na | na | na | na | na |

Experiment Result

Experiment is conducted to evaluate the proposed method under the common test condition [2] as below.

- Anchor: VTM-5.0

- Test : VTM-5.0 + SKKU_Scheme (SBT_Simplification)

|  |  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |  |
| Class A1 | 0.00% | -0.20% | 0.09% | 94% | 95% |  |
| Class A2 | 0.00% | 0.07% | -0.07% | 93% | 95% |  |
| Class B | 0.02% | 0.06% | 0.02% | 94% | 93% |  |
| Class C | 0.00% | -0.06% | -0.10% | 94% | 90% |  |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.02% | 94% | 93% |  |
| Class D | 0.06% | 0.00% | 0.09% | 92% | 88% |  |
| Class F | 0.01% | 0.01% | -0.02% | 94% | 90% |  |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.03% | 0.00% | -0.06% | 96% |  | 96% |
| Class C | 0.08% | -0.03% | -0.17% | 96% |  | 84% |
| Class E | 0.01% | 0.18% | -0.19% | 95% |  | 84% |
| Overall | 0.04% | 0.04% | -0.13% | 96% |  | 89% |
| Class D | -0.01% | 0.04% | 0.63% | 96% |  | 90% |
| Class F | 0.03% | -0.06% | -0.63% | 95% |  | 88% |

(test results are in the attached excel file JVET-O0475.xlsm.

Conclusion

In this contribution, we proposed reducing the contexts for cu_sbt_horizontal_flag from 3 to 2. This simplification shows trivial coding efficiency change, especially in Class A1 and A2.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- Frank Bossen, Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

Patent rights declaration(s)

Sungkyunkwan University (SKKU) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).