| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0476-v1 |
| --- | --- |

| Title: | Non-CE6: Syntax change for LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dohyeon Park Yong-Uk Yoon Jae-Gon Kim (KAU) Jinho Lee Jungwon Kang (ETRI) | Tel: Email: | +82-2-300-0414 dhpark@kau.kr yuyoon@kau.kr jgkim@kau.ac.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Korea Aerospace University (KAU), ETRI |  |  |

_____________________________

- Abstract

This contribution proposes an alternative syntax structure that moves the syntax element of the low-frequency non-separable transform (lfnst_idx) from at the end of the coding unit (CU) syntax to the end of the transform unit (TU) syntax. In addition, lfnst_idx is modified to be parsed only at the first TU of the CU. Two different tests are evaluated. Test 1 is applying the LFNST for the first TU only, and Test 2 is sharing the lfnst_idx with all TUs. The experimental results for Test1 reportedly show 0.00%, 0.00%, and 0.00% luma BD-rates performance for AI, RA, and LDB, respectively. The experimental results for Test2 also reportedly show 0.XX%, and 0.XX% luma BD-rates performance for RA, and LDB, respectively.

Introduction

In the current VVC WD 5.0, lfnst_idx is signaled to indicate whether LFNST is used or not and specify the selected transform kernel in the intra mode dependent transform set [1]. If the value of lfnst_idx is equal to 0, LFNST is not used. lfnst_idx is parsed at the end of the syntax structure of the coding unit because it requires checking the quantized coefficient levels of the CU to determine whether to parse the syntax or not. That is, lfnst_idx is able to be parsed after the completion of residual decoding for a given CU. However, the position of LFNST in the current syntax structure may cause a delay problem for a CU that is larger than the maximum transform size. In this case, the CU is divided into multiple TUs, and residual decoding is carried out in the unit of TU. Decoding of each TU is delayed until to know the value of lfnst_idx which can be obtained after completing the residual decoding of all TUs in the given CU. Besides, an additional buffer (e.g., four-times buffer size) is needed to keep all TU’s information during the delay. To resolve these problems, we propose an alternative syntax structure that moves the LFNST syntax into the transform unit syntax structure.

Proposed method

To reduce the bit-overhead which can be occurred from the moved LFNST syntax and to maintain similarity with the existing method, lfnst_idx parsing is only allowed for the first transform unit. There are two different tests as follow:

- Test 1: lfnst_idx for the first TU (LFNST is applied to the first TU only)

- Test 2: lfnst_idx shared by all TUs in the CU (LFNST is applied to all TUs)

The modified syntaxes for CU and TU with the moved position of LFNST are shown below:

Table 1. Proposed coding unit syntax

| …… |  |
| --- | --- |
| } |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = UAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Table 2. Proposed transform unit syntax (Test 1)

| …… |  |
| --- | --- |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? tbWidth / SubWidthC : tbWidth |  |
| lfnstHeight = ( treeType = = UAL_TREE_CHROMA ) ? tbHeight / SubHeightC : tbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] && !subTuIndex ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |

Table 3. Proposed transform unit (Test 2)

| …… |  |
| --- | --- |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? tbWidth / SubWidthC : tbWidth |  |
| lfnstHeight = ( treeType = = UAL_TREE_CHROMA ) ? tbHeight / SubHeightC : tbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| if(!subTuIndex) { |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| FirstTuLfnstIndex = lfnst_idx[ x0 ][ y0 ] |  |
| } |  |
| else |  |
| lfnst_idx[ x0 ][ y0 ] = FirstTuLfnstIndex |  |
| } |  |

Simulation results

The proposed tests are implemented on top of VTM5.0 and evaluated with VTM CTC configurations [2]. Table 4 shows the results of Test 1, and Table 5 shows the results of Test 2. Both tests did not reportedly show any changes in BD-rate. It may be caused that 128x128 intra blocks on P/B slices are extremely rare or this case is limited in the encoder. The encoding runtimes of AI are not reliable.

Table 4. Experimental results for Test 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 87% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 91% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 109% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 121% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 84% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 104% |
| Class F | 0.00% | 0.00% | 0.00% | 76% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 5. Experimental results for Test 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | 10% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Conclusions

This contribution presents a clear draft syntax of LFNST by moving it from the coding unit syntax to the transform unit syntax. Two tests are evaluated. First, lfnst_idx is parsed at the first TU and applied to the first TU only. Secondly, lfnst_idx is parsed at the first TU but shared with all TUs. These solve the delay problem for the largest CU without any performance change and complexity increase.

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, May 2019.

- F. Bossen, J. Boyce, et al., “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

Korea Aerospace University and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).