| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O00480-v1 |
| --- | --- |

| Title: | Non-CE8: On signaling of IBC block vector predictor list size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, an issue in the unified block vector prediction list for IBC mode is identified, where the list size for merge mode can range from 1 to 6 (controlled by MaxNumMergeCand). However, for IBC AMVP mode, the size is always assumed to be 2. When MaxNumMergeCand is signaled to be 1, there will be no second candidate generated from the candidate list for the AMVP mode.

Three solutions are proposed to fix the issue.

In solution 1, when MaxNumMergeCand is equal to 1, the mvp_l0_flag for IBC will not be signaled and inferred to be 0. Simulation results report that, by using the proposed method:

- -0.05%/-0.06%/-0.06% BD rate changes are observed for AI/RA/LDB configurations for class F when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

- -0.07%/-0.05%/-0.30% BD rate changes are observed for AI/RA/LDB configurations for class TGM when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

In solution 2, MaxNumMergeCand for IBC mode is clipped to be at least 2. Simulation results report that, by using the proposed method:

- -0.50%/-0.46%/-0.xx% BD rate changes are observed for AI/RA/LDB configurations for class F when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

- -1.64%/-0.85%/-0.xx% BD rate changes are observed for AI/RA/LDB configurations for class TGM when compared to VTM-5.0 with mvp_l0_flag always signaled as 0.

In solution 3, a separate variable MaxNumIbcCand for IBC mode candidate list size is signaled, which ranges from 2 to 6.

Problem Statement

In current VVC specification, intra block copy mode block vector prediction has only one candidate list for both merge mode and AMVP mode [1]. The size of this list is controlled by MaxNumMergeCand, which is shared with inter merge mode.

The IBC AMVP mode always assumes to have two candidates and therefore a mvp_l0_flag is signaled to choose one predictor candidate from the two in the list. When MaxNumMergeCand is equal to 1, only one candidate in the list will be generated for IBC mode. In this case, there is no defined behaviour that can work properly.

Proposed method

In this proposal, three solutions are suggested to fix the problem.

Solution 1: when MaxNumMergeCand is equal to 1, the mvp_l0_flag for IBC will not be signaled and inferred to be 0.

Related text changes are as follows:

7.3.7.5	Coding unit syntax

| … |  |
| --- | --- |
| if( general_merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if ( MaxNumMergeCand > 1 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_amvr_enabled_flag && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ) ) { |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| … |  |

Solution 2: MaxNumMergeCand for IBC mode is clipped to be at least 2.

Related text changes are as follows:

- 7.3.7.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |

7.4.6.1	General slice header semantics

…

six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slice subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − six_minus_max_num_merge_cand	(7-57)

The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive.

The variable MaxNumIbcCand is set equal to be Max( 2, MaxNumMergeCand ).

8.6.2.2	Derivation process for IBC luma motion vector prediction

…

- When numCurrCand is less than MaxNumIbcCand and smrNumHmvpIbcCand is greater than 0, the derivation process of IBC history-based motion vector candidates as specified in 8.6.2.4 is invoked with mvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrCand as inputs, and modified mvCandList and numCurrCand as outputs.

…

8.6.2.4	Derivation process for IBC history-based motion vector candidates

…

For each candidate in smrHmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpIbcCand, the following ordered steps are repeated until numCurrCand is equal to MaxNumIbcCand:

…

Solution 3: a separate variable MaxNumIbcCand for IBC mode candidate list is signaled, which ranges from 2 to 6.

Related text changes are as follows:

| … | ue(v) |
| --- | --- |
| } else if ( sps_ibc_enabled_flag ) |  |
| six_minus_max_num_ibc_cand | ue(v) |
| slice_qp_delta | se(v) |
| … |  |

- 7.3.7.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |

7.4.6.1	General slice header semantics

…

six_minus_max_num_ibc_cand specifies the maximum number of ibc motion vector prediction (MVP) candidates supported in the slice subtracted from 6. The maximum number of ibc MVP candidates, MaxNumIbcCand is derived as follows:

MaxNumIbcCand = 6 − six_minus_max_num_ibc_cand	(7-57)

The value of MaxNumIbcCand shall be in the range of 2 to 6, inclusive.

…

8.6.2.2	Derivation process for IBC luma motion vector prediction

…

- When numCurrCand is less than MaxNumIbcCand and smrNumHmvpIbcCand is greater than 0, the derivation process of IBC history-based motion vector candidates as specified in 8.6.2.4 is invoked with mvCandList, isInSmr set equal to IsInSmr[ xCb ][ yCb ], and numCurrCand as inputs, and modified mvCandList and numCurrCand as outputs.

…

8.6.2.4	Derivation process for IBC history-based motion vector candidates

…

For each candidate in smrHmvpIbcCandList[ hMvpIdx ] with index hMvpIdx = 1..smrNumHmvpIbcCand, the following ordered steps are repeated until numCurrCand is equal to MaxNumIbcCand:

…

Simulation results

The proposed methods in solution 1 and 2 are implemented on top of VTM-5.0. Simulation is performed using the common test condition as defined in JVET-N1010 plus MaxNumMergeCand=1 and MaxNumTriangleCand=0 are set. Only Class F and Class TGM are tested as IBC is turned off for all other sequences in the CTC. The reference method in these tests is to always signal the mvp_l0_flag as 0. Results are shown in the following table:

Table 1: Solution 1 (infer mvp_l0_flag to be 0):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM50-IBC idx always signaled as 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.05% | -0.15% | -0.16% | 100% | 100% |
| Class SCC 1080p | -0.07% | -0.05% | -0.04% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM50-IBC idx always signaled as 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.06% | -0.21% | -0.04% | 100% | 100% |
| Class SCC 1080p | -0.05% | -0.10% | -0.13% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM50-IBC idx always signaled as 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.06% | -0.37% | -0.33% | 100% | 100% |
| Class SCC 1080p | -0.30% | -0.38% | -0.32% | 99% | 101% |

Solution 2:

Table 2: Solution 2 (clip MaxNumIbcCand to be 2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM50-IBC-idx always signaled as 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.50% | -0.44% | -0.40% | 100% | 97% |
| Class SCC 1080p | -1.64% | -1.55% | -1.57% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM50-IBC-idx always signaled as 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | -0.46% | -0.58% | -0.65% | 99% | 96% |
| Class SCC 1080p | -0.85% | -0.99% | -1.04% | 99% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM50-IBC-idx always signaled as 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class SCC 1080p | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

In this proposal, several solutions are suggested to resolve an identified issue in the signaling of the block vector predictor candidate list. It is recommended to take one of the suggested fixes into VVC specification.

References

[1]	X. Xu, et al, “CE8-related: Combination test of JVET-N0176/JVET-N0317/JVET-N0382 on simplification of IBC vector prediction”, JVET-N0843, Geneva, CH, March 2019

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).