| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0481-v3 |
| --- | --- |

| Title: | Variations of the 8-bit implementation of MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philipp Helle, Jonathan Pfaff, Tobias Hinz, Philipp Merkle, Björn Stallenberger, Michael Schäfer, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Email: | philipp.helle@hhi.fraunhofer.de, jonathan.pfaff@hhi.fraunhofer.de, philipp.merkle@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Note for -v2:

# This version corrects experimental results for Method 2 in the RA configuration (Class A2 results are still unfinished).

# Note for –v3:

This version completes the experimental results for Method 2 in the RA configuration.

# Abstract

At the 14th JVET meeting, held in Geneva from March 19th to March 27th 2019, matrix-based intra prediction (MIP) was adopted. Here, the coefficients of the involved matrices and offset vectors were stored in 10-bit precision. At the Geneva Meeting, it was requested to present a version of MIP where the coefficients are stored in 8-bit precision. In JVET-O0084, such an 8-bit version is proposed. Responding to feedback that was given to the proponents of JVET-O0084, in the present document, two modifications of the 8-bit implementation of JVET-O0084 are proposed. The first proposed modification is the 7-bit version of JVET-O0084. If one implements this proposed method as an 8-bit method, the matrix offsets fW, introduced in JVET-O0084, can be removed. The second proposed modification is proposed on top of the first proposed modification, namely replacing the mean value of the reduced boundary in the algorithm of JVET-O0084 by the first entry of the reduced boundary. Therefore, in comparison to JVET-O0084, the computation of the mean value of the reduced boundary vector can be avoided. Note, that both proposed methods as well as the method of JVET-O0084 are different 8-bit (resp. 7-bit) integer realizations of the same floating point MIP-models. Moreover, both proposed methods are combined with the MIP design simplifications proposed in section 2 of JVET-O0084. The reported results of the first proposed method over VTM-5.0 are -0.04% in the AI configuration and -0.03% in the RA configuration for the luma-component. The tool-off test for MIP of the first proposed method is 0.4% in the AI and 0.3% in the RA configuration. The reported results of the second proposed method over VTM-5.0 are -0.04% in the AI configuration and -0.03% in the RA configuration. The tool-off test for MIP of the second proposed method is 0.4% in the AI and 0.3% in the RA configuration.

# Method 1: 8-bit implementation of MIP with the option to remove the matrix offset fW from JVET-O0084

## Algorithmic description

In this section, a variant of the 8-bit implementation of JVET-O0084 is proposed where the matrix offset fW in the computation of the matrix-vector-product can be removed. Here, the proposed implementation is exactly as in JVET-O0084, Section 1.1. The only difference is that it is proposed that all entries of the matrices belonging to one of the sets  S0, S1, S2 are stored in 7-bit precision instead of 8 bit-precision. Here, each matrix is stored as a matrix with unsigned 7-bit entries. Moreover, it is proposed that the factors fW, introduced in JVET-O0084, are also stored in 7-bit precision, as an unsigned 7-bit number. 	If  Wred and Hred denote the width and the height of the reduced prediction signal and if sW is the shift corresponding to the prediction mode, it is proposed to compute the reduced prediction signal predred  as

predredi=j=0inSize-1(Aij-fW)⋅inputred[j]+1≪sW-1≫sW+dcVal

where i∈0,…, Wred∙ Hred-1. Here, in contrast to JVET-O0084, the differences (Aij-fW) can be stored in 8 bit precision.

If one keeps storing the factors Aij and fW separately (and thus computes the matrix-vector product as in JVET-O0084), then, in the proposed method, they can both be stored in 7-bit and thus, the overall memory requirement of the proposed method is 4.5395 Kilobyte (5120 matrix entries +34 matrix offsets + 34 shifts values, all in 7-bit). If one prefers to only store the differences (Aij-fW), these differences can be stored in 8 bit and thus the total memory requirement of the method is 5.154 Kilobyte (5120 matrix entries + 34 shifts values, all in 8-bit), i.e., it is the same as in JVET-O0084 without the factors fW. It is pointed out that the aforementioned two methods of storage and computation of the matrix-vector product give bit-identical results.

## Experimental results

The following simulations were conducted. First, the effect of the MIP changes proposed in Section 1.1, compared to the VTM-5.0 anchor, both in the AI and the RA configuration, are presented. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler. Moreover, the common test conditions [3] were inferred. Second, a tool-off test for the MIP-version proposed in Section 1.1 is reported. During the conduction of the simulations, there was a heavy overload on the cluster and thus, the reported runtimes are not reliable.

Table 1. Result of the incorporated MIP changes: Reference is VTM-5.0 anchor, test is VTM-5.0 with Method 1 described in Section 1.1, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,08% | -0,17% | -0,13% | 101% | 99% |
| Class A2 | -0,05% | -0,15% | -0,08% | 100% | 101% |
| Class B | -0,05% | -0,10% | -0,05% | 101% | 101% |
| Class C | 0,00% | -0,08% | -0,02% | 101% | 101% |
| Class E | -0,05% | -0,09% | -0,07% | 100% | 102% |
| Overall | -0,04% | -0,11% | -0,07% | 101% | 101% |
| Class D | 0,00% | 0,08% | -0,22% | 101% | 101% |
| Class F | 0,00% | 0,10% | 0,10% | 101% | 102% |

Table 2. Result of the incorporated MIP changes: Reference is VTM-5.0 anchor, test is VTM-5.0 with Method 1 described in Section 1.1, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,06% | -0,16% | -0,16% | 100% | 101% |
| Class A2 | 0,00% | -0,13% | -0,10% | 100% | 99% |
| Class B | -0,02% | -0,06% | -0,29% | 100% | 102% |
| Class C | -0,03% | -0,10% | -0,09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,10% | -0,17% | 100% | 101% |
| Class D | -0,03% | -0,09% | 0,21% | 101% | 101% |
| Class F | -0,05% | -0,14% | -0,10% | 100% | 102% |

Table 3. Result of the tool-off-test for MIP with the incorporated changes: Reference is VTM-5.0 with Method 1 described in Section 1.1, test is VTM-5.0 with MIP off, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,67% | 0,49% | 0,41% | 87% | 102% |
| Class A2 | 0,38% | 0,18% | 0,08% | 88% | 101% |
| Class B | 0,34% | 0,10% | 0,16% | 87% | 99% |
| Class C | 0,30% | 0,17% | 0,23% | 88% | 98% |
| Class E | 0,38% | 0,19% | 0,18% | 87% | 101% |
| Overall | 0,40% | 0,21% | 0,21% | 88% | 100% |
| Class D | 0,37% | 0,28% | 0,31% | 89% | 99% |
| Class F | 0,18% | 0,09% | 0,11% | 93% | 99% |

Table 4. Result of the tool-off-test for MIP with the incorporated changes: Reference is VTM-5.0 with Method 1 described in Section 1.1, test is VTM-5.0 with MIP off, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,43% | 0,83% | 1,13% | 96% | 101% |
| Class A2 | 0,21% | 0,08% | 0,10% | 96% | 103% |
| Class B | 0,29% | 0,36% | 0,51% | 95% | 102% |
| Class C | 0,28% | 0,23% | 0,29% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0,30% | 0,36% | 0,49% | 96% | 101% |
| Class D | 0,31% | 0,06% | 0,01% | 95% | 101% |
| Class F | 0,23% | 0,24% | 0,27% | 94% | 99% |

# Method 2: 8-bit implementation of MIP without mean-value computation of the reduced input

## Algorithmic description

In JVET-O0084, it was proposed to subtract the mean-value of the reduced input from the reduced input before the matrix-vector multiplication and to add it after the matrix vector multiplication. In this section, it is proposed to replace the mean value by the first entry of the reduced input in this process. Thus, the computation of the mean value can be avoided. This method is proposed on top of the method of the previous Section 1. 	
Exactly as in [1] resp. Section 1 of JVET-O0084, for predicting the samples of a rectangular block of width W and height H, MIP takes one line of H reconstructed neighbouring boundary samples left of the block and one line of W reconstructed neighbouring boundary samples above the block as input. Out of these boundary samples, the reduced boundary vector bdryred is obtained by averaging exactly as described in Section 1.2 of [1]. Thus, bdryred is of size size(bdryred)=4, if W=H=4 and of size  sizebdryred=8, else. 	
Next, putting

idxW,H=0for W=H=41for maxW,H=82formaxW,H>8,

one defines the reduced input vector inputred as

inputred0=bdryred0-(1≪bitDepth-1),

inputredj= bdryredj-bdryred0,     j=1,…,sizebdryred-1,

if idxW,H=0 or idxW,H=1 and as,

inputredj= bdryredj+1-bdryred0,     j=0,…,sizebdryred-2,

if idxW,H=2. Here, bitDepth denotes the luma bit-depth. Thus, inputred is of the same size inSize as bdryred for idxW,H=0 or idxW,H=1 and of size inSize=sizebdryred-1,  if idxW,H=2. Exactly as in Section 1.3 of [1], determined by the MIP-mode, a matrix A is selected. Thus, the matrix A belongs to one of three sets S0, S1, S2 of matrices or is a matrix that belongs to the set S2 with some rows left out. The set S0 consists of 18 matrices A0i, i∈0, …, 17 each of which has 16 rows and 4 columns. Matrices of that set are used if idxW,H=0,  i.e., for blocks of size 4×4.  The set S1 consists of 10 matrices A1i, i∈0, …, 9, each of which has 16 rows and 8 columns. Matrices of that set are used if idxW,H=1, i.e., for blocks of sizes 4×8, 8×4 and 8×8. Finally, the set S2 consists of 6 matrices A2i, i∈0, …, 5, each of which has 64 rows and 7 columns. Matrices of that set or parts of these matrices are used idxW,H=2, i.e., for all other block-shapes. As in the previous Section 1, all entries of the matrices belonging to the sets S0, S1 and  S2 are stored in 7 bit as unsigned 7 bit numbers. Also, as in the previous section, the factors fW are stored as 7 bit numbers.	
If  Wred and Hred denote the width and the height of the reduced prediction signal and if sW is the shift corresponding to the prediction mode, it is proposed to compute the reduced prediction signal predred  as

predredi=j=0inSize(Aij-fW)⋅inputred[j]+1≪sW-1≫sW+bdryred0

where i∈0,…, Wred∙ Hred-1. As in the previous Section 1, the differences (Aij-fW) can  be stored in 8 bit precision.

If one keeps storing the factors Aij and fW separately (and thus computes the matrix-vector product as in JVET-O0084), then, in the proposed method, they can both be stored in 7-bit and thus, the overall memory requirement of the proposed method is 4.5395 Kilobyte (5120 matrix entries +34 matrix offsets + 34 shifts values, all in 7-bit). If one prefers to only store the differences (Aij-fW), these differences can be stored in 8 bit and thus the total memory requirement of the method is 5.154 Kilobyte (5120 matrix entries + 34 shifts values, all in 8-bit), i.e., it is the same as in JVET-O0084 without the factors fW. It is pointed out that the aforementioned two methods of storage and computation of the matrix-vector product give bit-identical results.

## Experimental results

The following simulations were conducted. First, the effect of the MIP changes proposed in Section 2.1, compared to the VTM-5.0 anchor, both in the AI and the RA configuration, are presented. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler. Moreover, the common test conditions [3] were inferred. Second, a tool-off test for the MIP-version proposed in Section 2.1 is reported. During the conduction of the simulations, there was a heavy overload on the cluster and thus, the reported runtimes are not reliable.

Table 1. Result of the incorporated MIP changes: Reference is VTM-5.0 anchor, test is VTM-5.0 with -Method 2 described in Section 2.1, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,07% | -0,07% | -0,22% | 102% | 100% |
| Class A2 | -0,05% | -0,08% | -0,09% | 101% | 101% |
| Class B | -0,04% | -0,13% | -0,06% | 102% | 104% |
| Class C | 0,00% | -0,11% | 0,05% | 102% | 107% |
| Class E | -0,06% | -0,25% | -0,10% | 102% | 104% |
| Overall | -0,04% | -0,12% | -0,07% | 102% | 104% |
| Class D | -0,02% | 0,21% | -0,05% | 102% | 109% |
| Class F | 0,00% | 0,00% | 0,12% | 103% | 104% |

Table 2. Result of the incorporated MIP changes: Reference is VTM-5.0 anchor, test is VTM-5.0 with Method 2 described in Section 2.1, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,06% | -0,24% | -0,12% | 100% | 100% |
| Class A2 | -0,02% | -0,04% | -0,01% | 100% | 98% |
| Class B | 0,00% | 0,13% | -0,20% | 100% | 101% |
| Class C | -0,04% | -0,03% | -0,17% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,02% | -0,14% | 100% | 100% |
| Class D | 0,01% | -0,19% | 0,38% | 99% | 103% |
| Class F | -0,03% | -0,15% | 0,22% | 99% | 101% |

Table 3. Result of the tool-off-test for MIP with the incorporated changes: Reference is VTM-5.0 with Method 2 described in Section 2.1, test is VTM-5.0 with MIP off, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,65% | 0,39% | 0,49% | 86% | 100% |
| Class A2 | 0,38% | 0,11% | 0,09% | 88% | 101% |
| Class B | 0,33% | 0,13% | 0,17% | 86% | 97% |
| Class C | 0,31% | 0,20% | 0,15% | 87% | 93% |
| Class E | 0,39% | 0,35% | 0,21% | 86% | 98% |
| Overall | 0,40% | 0,22% | 0,21% | 87% | 97% |
| Class D | 0,39% | 0,15% | 0,13% | 88% | 91% |
| Class F | 0,19% | 0,19% | 0,09% | 91% | 96% |

Table 4. Result of the tool-off-test for MIP with the incorporated changes: Reference is VTM-5.0 with Method 2 described in Section 2.1, test is VTM-5.0 with MIP off, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,43% | 0,91% | 1,09% | 96% | 103% |
| Class A2 | 0,22% | -0,01% | 0,01% | 97% | 103% |
| Class B | 0,27% | 0,17% | 0,43% | 96% | 103% |
| Class C | 0,29% | 0,16% | 0,37% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0,30% | 0,28% | 0,46% | 96% | 102% |
| Class D | 0,27% | 0,16% | -0,16% | 96% | 99% |
| Class F | 0,21% | 0,26% | -0,05% | 95% | 99% |

# Conclusion

This document proposes two Methods which are variations of the 8-bit implementation of JVET-O0084. Method 1 allows storing both the matrix weights Aij and the matrix offsets fW in 7bit precision, thus allowing storing the differences (Aij-fW) in 8bit precision. Method 2 includes Method 1 and additionally avoids calculating the mean of the reduced input vector before the matrix-vector product. Instead of subtracting the mean, it subtracts the first element of the reduced input vector. It is proposed to adopt Method 2 as the requested 8-bit implementation of MIP.

# References

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019.

- B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, Geneva, Switzerland, March 2019

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).