| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SWE, 03–12 July 2019 | Document: JVET-O0484-r1 |
| --- | --- |

| Title: | CE3-related: Improved intra mode coding based on unified MPM list generation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Tobias Hinz, Philipp Merkle, Philipp Helle, Björn Stallenberger, Michael Schäfer, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany Biao Wang, Semih Esenlik, Anand Meher Kotra, Han Gao, Elena Alshina Huawei Technologies Duesseldorf GmbH Riesstr. 25 80992 Munich, Germany | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de,philipp.merkle@hhi.fraunhofer.de, philipp.helle@hhi.franunhofer.de +49 89 264833500 biao.wang@huawei.com semih.esenlik@huawei.com |
| Source: | Fraunhofer HHI Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, an alignment of the newly adopted Unified-MPM (UMPM) generation of VTM 5.0 is proposed. The proposal has two aspects. First, it is proposed to keep all the design features of the adopted UMPM and to only align the generation of the list of the five non-planar MPMs. Second, the planar flag in VTM 5.0 uses two contexts based on whether a current block is applied with Intra Sub-Partition (ISP) coding tool. In this proposal, four contexts are used by also considering the prediction mode of left and above blocks. It is reported that a combined method of these two aspects yields 0.12% coding gain in AI over VTM-5.0, with 99% encoding and 100% decoding time and that it yields … coding gain in RA over VTM-5.0, with 100% encoding and 100% decoding time. Both modifications are jointly proposed by the proponents of UMPM (JVET N0185) and MIP (JVET N0217).

# Alignment of the list generation

## Introduction

At the 14th JVET meeting in Geneva, a unified method of signaling and generating the most probable modes (MPMs) for intra prediction was adopted, [1], [2]. This method has the following two design features:

- In the coding of the MPMs, the planar mode is context coded with a separate flag.

- ISP, MRL and the normal intra prediction mode use the same method to generate the five non-planar MPMs.

Meanwhile, Matrix based Intra Prediction (MIP) [3] was also adopted in the same Geneva meeting. It performs intra prediction not using conventional angular or non-angular prediction modes, but using matrix-vector multiplication with trained matrix coefficients. When integrating these two tools, the intra modes probably falling into the MPM list have been changed, and it is asserted that a harmonization between MIP and UMPM tools is required.

In the present Section 1, a slight alignment of UMPM is proposed, which is asserted to harmonize UMPM and MIP, but keeps the two aforementioned design features of UMPM. It only changes the method by which the five non-planar MPMs are generated for conventional- (non-ISP, non-MRL, non-MIP), ISP- and MRL-intra prediction. More precisely, it is proposed not to have the DC mode in the list of the five non-planar MPMs, if one of the neighboring available PUs (left/above) uses a directional mode. It is asserted that this modification can be seen as a harmonization of UMPM with MIP for the following reason. Since the MIP modes typically predict into a downsampled domain, they are non-directional like the DC mode. Therefore, the DC mode becomes less attractive with the presence of MIP.

## Modified list generation

Based on the UMPM list generation as adopted in VTM-5.0, the following changes to the mode list generation of the five non-planar MPMs for conventional- (non-ISP, non-MRL, non-MIP), ISP- and MRL-intra prediction are proposed. Note that, in comparison to VTM-5.0, the coding of the planar flag as well as the coding of the MPM index for the five non-planar MPMs remain unchanged by the method proposed in the present section.Specification text changes in section 8.4.3 of [1]:

…

- The candModeList[ x ] with x = 0..4 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-24)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-25)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-26)

candModeList[ 3 ] =2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-27)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-28)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-29)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-30)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-31)

candModeList[ 1 ] = candIntraPredModeB	(8-32)

-

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-34)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-35)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-xx)

  - Otherwise, if maxAB − minAB is greater than or equal to 62, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-xx)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-xx)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-xx)

  - Otherwise, if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-xx)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-xx)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-xx)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-xx)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-36)

candModeList[ 4 ] = 2 + ( ( ( maxAB + 61 ) ) % 64 )	(8-37)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..4 is derived as follows:

candModeList[ 0 ] = maxAB		(8-38)

-

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-40)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-42)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-xx)

- Otherwise, the following applies:

candModeList[ 0 ] = INTRA_DC			(8-43)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-44)

candModeList[ 2 ] = INTRA_ANGULAR18	(8-45)

candModeList[ 3 ] = INTRA_ANGULAR46	(8-46)

candModeList[ 4 ] = INTRA_ANGULAR54	(8-47)

# Modification of the context modelling of the planar flag

## Introduction

Due to the high occurrence of planar mode in intra prediction, a new planar flag is introduced in VTM5.0 (intra_luma_not_planar_flag) to identify whether the intra mode of current block is planar mode or not [2]. The (not) planar flag uses regular CABAC coding with two contexts:

- the current block is applied with ISP mode

- the current block is not applied with ISP mode

## Proposed CABAC context modelling

It is found that the (not) planar flag is correlated with the modes of left and above blocks, and with the introduction of the (not) planar flag, it is possible to know whether a left or above block uses planar mode or not. Therefore, more CABAC contexts are proposed for the (not) planar flag based on left and above blocks, specifically, with the following four contexts:

- current block is applied with ISP mode

- current block is not applied with ISP mode and neither left nor above block use planar mode.

- current block is not applied with ISP mode and either left or above block use planar mode.

- current block is not applied with ISP mode and both left or above blocks use planar mode.

The neighboring left (or above) block is considered as if it uses planar mode if one of the following conditions is true:

- it is applied with Matrix-based intra prediction;

- it is applied with inter-prediction;

- it is applied with Intra block copy (IBC) mode;

- it is applied with normal intra prediction, and the prediction mode is planar mode.

It is worth noting that the new context derivation would NOT introduce parsing dependency, as no MPM list construction is required to determine the proposed contexts.

Specification text changes in section 9.5.4.2

Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_luma_not_planar_flag[ ][ ] | 0,1,2,3 (clause 9.5.4.2.10) | na | na | na | na | Na |

##### 9.5.4.2.10 Derivation process of ctxInc for the syntax intra_luma_not_planar_flag

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the width and the height of the current coding block in luma samples cbWidth and cbHeight.

Output of this process is ctxInc.

The location ( xNbL, yNbL ) is set equal to ( x0 − 1, y0 + cbHeight -1) and the variable availableL, specifying the availability of the block located directly to the left of the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

The location ( xNbA, yNbA ) is set equal to ( x0 + cbWidth − 1, y0 − 1 ) and the variable availableA specifying the availability of the coding block located directly above the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause 6.4 with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbA, yNbA ) as inputs, and the output is assigned to availableA.

If y0 − 1 is less than ( ( y0  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), availableA is set to false.

The assignment of ctxInc is specified as follows with condL and condA specified in Table 9-19:

ctxInc = intra_subpartitions_mode_flag[ x0 ][ y0 ]? 0: (1+ ( condL  &&  availableL ) + ( condA  &&  availableA ))  			(9-28)

- Table 9-19 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA |
| --- | --- | --- |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | pred_mode_ibc_flag [ xNbL ][ yNbL ] ¦¦ intra_mip_flag [ xNbL ][ yNbL ] ¦¦ ( CuPredMode[ xNbL ][ yNbL ] = = MODE_INTER ) ¦¦ ( CuPredMode[ xNbL ][ yNbL ] = = MODE_INTRA && intra_luma_not_planar_flag[ xNbL ][ yNbL ] == 0) | pred_mode_ibc_flag [ xNbA ][ yNbA ] ¦¦ intra_mip_flag [ xNbA ][ yNbA ] ¦¦ ( CuPredMode[ xNbA ][ yNbA ] = = MODE_INTER ) ¦¦ ( CuPredMode[ xNbA ][ yNbA ] = = MODE_INTRA && intra_luma_not_planar_flag[ xNbA ][ yNbA ] == 0) |

# Experimental Results

Simulations are performed according to CTC described in [4]. AI and RA results are reported.

The combination of the methods proposed in Section 1 and Section 2 is reported to give the following results:

Table 1. Results of the combination of the methods proposed in Section 1 and Section 2 over VTM 5.0, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,12% | -0,08% | -0,15% | 98% | 99% |
| Class A2 | -0,10% | -0,12% | -0,13% | 99% | 101% |
| Class B | -0,10% | -0,17% | -0,13% | 99% | 102% |
| Class C | -0,13% | -0,17% | -0,04% | 101% | 100% |
| Class E | -0,20% | -0,26% | -0,22% | 99% | 99% |
| Overall | -0,12% | -0,16% | -0,13% | 99% | 100% |
| Class D | -0,06% | 0,12% | -0,09% | 101% | 101% |
| Class F | -0,09% | 0,02% | -0,03% | 101% | 100% |

Table 2. Results of the combination of the methods proposed in Section 1 and Section 2 over VTM 5.0, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | 0,03% | -0,14% | 100% | 103% |
| Class C | -0,08% | -0,01% | 0,03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,00% | 0,04% | 0,26% | 102% | 99% |
| Class F | -0,07% | -0,10% | -0,04% | 100% | 100% |

## Additional results with only MPM modification (aspect 1)

Table 3. Results of the method proposed in Section 1 over VTM 5.0, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,06% | -0,03% | -0,07% | 100% | 98% |
| Class A2 | -0,06% | -0,14% | -0,15% | 99% | 100% |
| Class B | -0,07% | -0,09% | -0,05% | 100% | 102% |
| Class C | -0,10% | -0,08% | -0,08% | 100% | 100% |
| Class E | -0,14% | -0,26% | -0,14% | 100% | 100% |
| Overall | -0,08% | -0,12% | -0,09% | 100% | 100% |
| Class D | -0,04% | 0,12% | -0,02% | 100% | 103% |
| Class F | -0,11% | 0,07% | -0,05% | 101% | 100% |

Table 4. Results of the method proposed in Section 1 over VTM 5.0, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | 0,01% | -0,22% | 99% | 100% |
| Class C | -0,10% | -0,27% | -0,20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0,03% | -0,34% | -0,18% | 100% | 102% |
| Class F | -0,09% | -0,21% | 0,01% | 100% | 99% |

## Additional results with only context modelling modification (aspect 2)

Table 5. Results of the method proposed in Section 2 over VTM 5.0, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.07% | -0.05% | -0.06% | 100% | 101% |
| Class A2 | -0.02% | -0.03% | -0.08% | 101% | 101% |
| Class B | -0.03% | -0.06% | -0.01% | 101% | 101% |
| Class C | -0.01% | -0.03% | 0.06% | 101% | 100% |
| Class E | -0.07% | -0.15% | -0.07% | 101% | 100% |
| Overall | -0.04% | -0.06% | -0.03% | 101% | 101% |
| Class D | -0.03% | 0.16% | -0.07% | 101% | 101% |
| Class F | -0.01% | -0.02% | 0.01% | 101% | 101% |

Table 6. Results of the method proposed in Section 2 over VTM 5.0, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.01% | -0.23% | 0.07% | 101% | 100% |
| Class A2 | 0.01% | 0.08% | 0.04% | 101% | 100% |
| Class B | 0.01% | -0.02% | -0.25% | 100% | 100% |
| Class C | -0.05% | -0.08% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.06% | -0.04% | 101% | 100% |
| Class D | 0.03% | 0.02% | 0.08% | 101% | 101% |
| Class F | 0.04% | 0.02% | 0.09% | 100% | 100% |

# Conclusion

In this document, an alignment of the unified MPM-list construction for the intra mode, adopted at the last meeting, is proposed. The first proposed method keeps the design features of the adopted unified MPM-list construction and only aligns the generation of the list of the five non-planar MPMS by removing the DC-mode from this list if a neighboring mode is angular. The second proposed method modifies the context modelling of the planar flag, by also considering prediction modes of left and above blocks. Compared to VTM 5.0 using AI configuration, a combined approach of the proposed two methods yields an additive coding gain of 0.12%, with no encoding/decoding runtime increase observed. It is proposed to adopt the proposed methods into the future VVC.

# References

- B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, Geneva, Switzerland, March 2019

- B. Wang et al., “CE3-related: A unified MPM list for intra mode coding”, JVET-N0185, Geneva, Switzerland, March 2019

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019.

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).