| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SWE, 03–12 July 2019 | Document: JVET-O0485-r2 |
| --- | --- |

| Title: | Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Tobias Hinz, Philipp Helle, Philipp Merkle, Björn Stallenberger, Michael Schäfer, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de,philipp.merkle@hhi.fraunhofer.de, philipp.helle@hhi.franunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

At the 14th JVET meeting in Geneva, among other tools, the following three intra-tools were adopted:  Low-Frequency Non-Separable Secondary Transform (LFNST), Unified-MPM (UMPM) and Matrix Based Intra Prediction (MIP). The present document proposes an alignment of these three tools. This alignment is built on an 8-bit implementation of MIP, JVET-O0481, Method 2, the latter being requested at the last meeting. The present proposal has two aspects. First, for large blocks, it is proposed to allow the adopted LFNST transforms also for the MIP-modes. Second, for the UMPM, it is proposed not to have the DC-mode in the list of the five non-planar MPMs whenever at least one neighboring mode is angular. It is asserted that the latter change keeps the design features of UMPM and only aligns UMPM with MIP for the reason that MIP has made the DC mode less likely. It is reported that the proposed method yields -0.15% BD-rate savings over the VTM-5.0 in the AI configuration. The encoder runtime is reported to be 94%, the decoder runtime is reported to be 100%. For the RA configuration, it is reported that the proposed method yields -0.03% BD-rate savings over the VTM-5.0. The encoder runtime is reported to be 96%, the decoder runtime is reported to be 99% . Moreover, it is reported that MIP contributes to these numbers as follows.  In the present proposal, the tool-off test for MIP is reported to be 0.45% BD-rate change with an encoder runtime of 93% and a decoder runtime of 102% in the AI configuration and 0.26% BD-rate change for the RA configuration with an encoder runtime of 99% and a decoder runtime of 103%.

Version 2 of this documents includes full results for the RA configuration that were missing as well as a reference in the abstract to the 8-bit implementation of MIP used in the present document.

# 8-bit implementation of MIP

The present proposal uses the 8-bit implementation of MIP as in Section 2 of [6] as its starting point. Here, the MIP modes proposed in [6] are based on the same floating point models as the MIP modes of the initially proposed 8-bit version [5] of MIP. However, in comparison to [5], in [6], the computation of the mean value of the reduced boundary can be avoided. Moreover, in comparison to [5], in [6], the differences Aij-fW of the matrix entries and the matrix offsets can be stored in 8 bit, since all matrix entries Aij and all matrix offsets fW are stored as unsigned 7 bit integers. Also, the MIP-simplifications proposed in [5] are incorporated into the present proposal.

# Alignment with LFNST

It is proposed to allow LFNST also for MIP-modes on blocks of width W and height H if both W and H are greater than or equal to 16. Here, if MIP and LFNST are to be used on such a block, the MIP mode is treated as a conventional intra prediction mode by using the mapping table with MipSizeId=2 of [1], Table 8-4, which maps MIP modes to conventional intra prediction modes.

# Alignment with UMPM

It is proposed to use the modified list construction of the five non-planar MPMS for the conventional- (non-ISP, non-MRL, non-MIP), ISP and MRL-intra prediction modes which is proposed in Section 1 of [8].  Thus, it is proposed not to have the DC-mode in the list of the five non-planar-MPMS if one of the neighboring available PUs (left/above) uses a directional mode. It is asserted that this proposed method keeps all design features of UMPM, which are asserted to be the context coding of the planar mode and a single list of five non-planar MPMS for the conventional, the ISP- and the MRL-intra prediction modes. It is asserted that this modification can be seen as a harmonization of UMPM with MIP for the following reason. Since the MIP modes typically predict into a downsampled domain, they are non-directional, as the DC mode. Therefore, the DC mode becomes less attractive with the presence of MIP.

# Experimental Results

Table 1 and Table 2 below report on the coding performance of the proposed method over the VTM-5.0 anchor.

Table 1. Results of the proposed method over VTM 5.0, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,15% | -0,21% | -0,20% | 94% | 99% |
| Class A2 | -0,13% | -0,19% | -0,16% | 94% | 100% |
| Class B | -0,14% | -0,20% | -0,15% | 94% | 101% |
| Class C | -0,12% | -0,20% | -0,10% | 96% | 100% |
| Class E | -0,21% | -0,30% | -0,16% | 95% | 101% |
| Overall | -0,15% | -0,22% | -0,15% | 94% | 100% |
| Class D | -0,08% | 0,13% | -0,06% | 95% | 102% |
| Class F | -0,10% | 0,06% | 0,01% | 98% | 99% |

Table 2. Results of the proposed method over VTM 5.0, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,07% | -0,49% | -0,25% | 96% | 100% |
| Class A2 | 0,00% | 0,10% | -0,06% | 96% | 98% |
| Class B | -0,02% | 0,09% | -0,12% | 96% | 100% |
| Class C | -0,03% | -0,04% | -0,11% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,06% | -0,13% | 96% | 99% |
| Class D | 0,08% | -0,04% | -0,02% | 97% | 101% |
| Class F | -0,05% | -0,18% | 0,01% | 96% | 99% |

Table 3 and Table 4 below report on the MIP-tool-off test, where both test and reference use the UMPM-scheme proposed in the present contribution.

Table 3. Reference: Proposed method. Test: Proposed method with MIP off. AI configuration.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,70% | 0,50% | 0,39% | 93% | 103% |
| Class A2 | 0,42% | 0,21% | 0,16% | 95% | 102% |
| Class B | 0,39% | 0,15% | 0,21% | 93% | 101% |
| Class C | 0,37% | 0,29% | 0,24% | 93% | 101% |
| Class E | 0,44% | 0,29% | 0,16% | 93% | 102% |
| Overall | 0,45% | 0,27% | 0,23% | 93% | 102% |
| Class D | 0,44% | 0,30% | 0,06% | 94% | 100% |
| Class F | 0,21% | 0,19% | 0,06% | 95% | 103% |

Table 4. Reference: Proposed method. Test: Proposed method with MIP off. RA configuration.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,40% | 1,20% | 1,04% | 99% | 104% |
| Class A2 | 0,18% | 0,14% | 0,26% | 99% | 103% |
| Class B | 0,24% | 0,27% | 0,43% | 99% | 104% |
| Class C | 0,22% | 0,15% | 0,31% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0,26% | 0,40% | 0,48% | 99% | 103% |
| Class D | 0,20% | 0,14% | 0,23% | 99% | 101% |
| Class F | 0,20% | 0,34% | 0,20% | 97% | 104% |

# Conclusion

This contribution proposes an alignment of the three tools LFNST, Unified-MPM and MIP that were all adopted at the last Geneva meeting. Here, the 8-bit variant of MIP as proposed in [6] is used. First, for large blocks, it is proposed to allow LFNST also for MIP. Second, it is proposed to keep the design features of the adopted unified MPM-list construction and to only align the generation of the list of the five non-planar MPMS by removing the DC-mode from this list if a neighboring mode is angular.  A coding gain of -0.15% BD-rate savings with 94% encoder and 100% decoder runtime over the VTM-5.0 in the AI configuration is reported.  For the RA configuration, a coding gain of -0.03% BD-rate savings over the VTM-5.0 with an encoder runtime of 96% and a decoder runtime of 99% is reported. In the proposed method, the MIP-tool-off test is reported to be 0.45% BD-rate change with an encoder runtime of 93% and a decoder runtime of 102% in the AI configuration and 0.26% with an encoder runtime of 99% and a decoder runtime of 103% for the RA configuration. It is recommended to adopt the proposed changes into the next working draft of VVC.

# References

- B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, Geneva, Switzerland, March 2019

- B. Wang et al., “CE3-related: A unified MPM list for intra mode coding”, JVET-N0185, Geneva, Switzerland, March 2019

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019.

- M. Koo et al., “CE6: Reduced Secondary Transform (RST) (CE6-3.1)”, JVT-N0193, Geneva, Switzerland, March 2019.

- J. Pfaff et al., “8-bit implementation and simplification of MIP”, JVET-O0084, Gothenburg, Sweden, July 2019.

- P. Helle et al., “Variation of the 8-bit implementation of MIP”, JVET-O0481, Gothenburg, Sweden, July 2019.

- B. Wang et al., “Non-CE3: Modified CABAC Coding for Intra Prediction Mode”, JVET-O0111, Gothenburg, Sweden, July 2019

- J. Pfaff et al., “Improved intra mode coding based on unified MPM-list generation”, JVET-O0484, Gothenburg, Sweden, July 2019.

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).