| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SWE, 03–12 July 2019 | Document: JVET-O0486-r1 |
| --- | --- |

| Title: | Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST using a single type of MIP prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Tobias Hinz, Philipp Helle, Philipp Merkle, Björn Stallenberger, Michael Schäfer, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de,philipp.merkle@hhi.fraunhofer.de, philipp.helle@hhi.franunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The present document reports a variant of the proposal [9] where the MIP-prediction modes can be shared across all block-shapes excluding 4x4-blocks. Here, the MIP modes belonging to the MipSizeId=2 are used also for blocks of shape 4x8, 8x4 and 8x8. For 4x4-blocks MIP is not available. In the variant of the present document, the total memory requirement of MIP is reported to be 2.3625 kB. It is reported that the proposed method yields -0.14% BD-rate savings over the VTM-5.0 in the AI configuration. The encoder runtime is reported to be 94%, the decoder runtime is reported to be 100%. For the RA configuration, it is reported that the proposed method yields … . Moreover, it is reported that MIP contributes to these numbers as follows.  In the present proposal, the tool-off test for MIP is reported to be 0.45% BD-rate change with an encoder runtime of 94% and a decoder runtime of 102% in the AI configuration and … for the RA configuration.

# Algorithmic description

For 4x4-blocks, MIP is not available in the present contribution.

For blocks of shape 4x8, 8x4 and 8x8, MIP is applied with the matrices for MipSizeId=2. Thus, for such blocks, the reduced boundary bdryred and the reduced input inputred are exactly generated as in [9]. However, to generate a MIP-prediction signal, in the variant described in the present document, a 64×7 matrix A, for which some rows are left out for the case of 4×8- or 8×4-blocks,  is used. To generate the prediction signal, the matrix-vector product A∙inputred is computed. In comparison to the previous MIP-version, the number of multiplications per sample increases from four to eight for the cases of 4×8 or 8×4  and 8×8 blocks.

For the remaining block shapes, MIP is applied exactly as in [9], Also, the changes on the UMPM-design as well as on the combination of MIP with LFNST that were proposed in [9] are enabled in the present document.

For the method described in the present document, only six 64×7 matrices along with six offsets and six shifts are used for all shapes and need to be stored in 7-bit precision. These six  64×7 matrices as well as the offsets and shifts coincide with the matrices belonging to the set S2 in [9] resp. [6].  Thus, a total number of 2688 numbers, each in 7-Bit precision, need to be stored which corresponds to 2.3625 kB of memory.

# Experimental Results

Table 1 and Table 2 below report on the coding performance of the method of the present document over the VTM-5.0 anchor.

Table 1. Results of the method of the present document over VTM 5.0, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,14% | -0,23% | -0,23% | 93% | 101% |
| Class A2 | -0,15% | -0,18% | -0,14% | 94% | 100% |
| Class B | -0,16% | -0,20% | -0,11% | 93% | 101% |
| Class C | -0,07% | -0,08% | -0,08% | 95% | 98% |
| Class E | -0,20% | -0,17% | -0,22% | 94% | 99% |
| Overall | -0,14% | -0,17% | -0,15% | 94% | 100% |
| Class D | 0,05% | 0,18% | -0,07% | 94% | 99% |
| Class F | -0,06% | 0,04% | -0,06% | 96% | 98% |

Table 2. Results of the method of the present document over VTM 5.0, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | 0,09% | -0,13% | 97% | 103% |
| Class C | 0,00% | -0,25% | -0,06% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,10% | -0,02% | 0,12% | 96% | 102% |
| Class F | 0,00% | -0,07% | 0,03% | 95% | 100% |

Table 3 and Table 4 below report on the MIP-tool-off test, where both test and reference use the UMPM-scheme proposed in [9] and also used in the present contribution.

Table 3. Reference: Method of the present document. Test: Method of the present document with MIP off. AI configuration.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,69% | 0,52% | 0,41% | 93% | 101% |
| Class A2 | 0,44% | 0,21% | 0,14% | 94% | 102% |
| Class B | 0,42% | 0,15% | 0,16% | 94% | 102% |
| Class C | 0,32% | 0,17% | 0,23% | 94% | 103% |
| Class E | 0,43% | 0,16% | 0,22% | 94% | 105% |
| Overall | 0,45% | 0,23% | 0,22% | 94% | 102% |
| Class D | 0,31% | 0,26% | 0,08% | 95% | 102% |
| Class F | 0,16% | 0,21% | 0,14% | 97% | 105% |

Table 4. Reference: Method of the present document. Test: Method of the present document with MIP off. RA configuration.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,24% | 0,27% | 0,44% | 98% | 100% |
| Class C | 0,19% | 0,36% | 0,25% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,18% | 0,13% | 0,09% | 99% | 100% |
| Class F | 0,15% | 0,23% | 0,18% | 99% | 104% |

# Conclusion

This contribution reports on a variant of MIP where only a single set of matrices is needed to generate the prediction signal and where thus the MIP-prediction modes are shared across all block shapes excluding 4x4-blocks. For 4x4 blocks, MIP is not available in the setting of the present contribution. The total memory requirement to store all MIP-parameters is reported to be 2.3625 kB in the method of the present document. A coding gain of -0.14% BD-rate savings with 94% encoder and 100% decoder runtime over the VTM-5.0 in the AI configuration is reported.  For the RA configuration, a coding gain of … is reported over the VTM-5.0. In the proposed method, the MIP-tool-off test is reported to be 0.45% BD-rate change with an encoder runtime of 94% and a decoder runtime of 102% in the AI configuration and … for the RA configuration.  It is recommended to adopt the MIP-variant of the present contribution if a further reduction of the number of parameters needed for MIP is desired.

# References

- B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, Geneva, Switzerland, March 2019

- B. Wang et al., “CE3-related: A unified MPM list for intra mode coding”, JVET-N0185, Geneva, Switzerland, March 2019

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019.

- M. Koo et al., “CE6: Reduced Secondary Transform (RST) (CE6-3.1)”, JVT-N0193, Geneva, Switzerland, March 2019.

- J. Pfaff et al., “8-bit implementation and simplification of MIP”, JVET-O0084, Gothenburg, Sweden, July 2019.

- P. Helle et al., “Variation of the 8-bit implementation of MIP”, JVET-O0481, Gothenburg, Sweden, July 2019.

- B. Wang et al., “Non-CE3: Modified CABAC Coding for Intra Prediction Mode”, JVET-O0111, Gothenburg, Sweden, July 2019

- J. Pfaff et al., “Improved intra mode coding based on unified MPM-list generation”, JVET-O0484, Gothenburg, Sweden, July 2019.

- J. Pfaff et al., “Non-CE3: Harmonization of 8-Bit MIP with Unified-MPM and LFNST”, JVET-O0485, Gothenburg, Sweden, July 2019.

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).