| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0490 |
| --- | --- |

| Title: | AHG17: Independent parsing of parameter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to keep the design principle of independent parsing of parameter sets and also proposes solutions for the parsing dependencies introduced the VVC draft 5.

# Introduction

The concept of parameter set was introduced in H.264/AVC and the same principles were followed in HEVC. A parameter set includes a number of parameters within a certain scope, e.g. a picture parameter set has the scope of one or multiple picture, while a sequence parameter set has the scope of a coded video sequence.

One of the design principles was to make each parameter set an own independent unit, which a decoder can parse and understand without the knowledge of any other part of the bitstream. An association between the parameters was only achieved in the activation process. Thus, parameter sets could be provided to a decoder at any point of time and in any order before they are used. There was no need to store them in binary format.

The authors believe that this property is valuable and this design principle should be used in VVC.

If dependencies were allowed, this would create an additional set of constraints for the bitstream: Whenever a parameter set is found, all parameter sets, it refers to (along the activation chain) have to be available. For instance for parsing a PPS, the SPS it refers to has to be available. If the SPS refers to a DPS, that DPS has to be available, as well as the VPS.

This also means that if a parameter set higher up in the chain is changed, all parameter sets depending on it need to be removed from storage because after parsing the parameter set in the first place, the decoder can’t ensure, that parsing would have worked with the parameter set, that the parameter set is activated for. For instance, if a VPS changes, the decoder needs to identify all SPSs referring to that VPS and also all PPSs referring to any of these SPSs and remove them from storage. An alternative solution would be to store parameter sets in binary for and only parse them when activated. But this is not entirely possible, because the decoder would have to at least parse the parameter set ID to properly store and activate the parameter set.

With APS the whole situation gets more complicated. One of the main arguments for introducing a dependency is that all tool related syntax elements could be removed in case the tool is disabled in the DPS. For the parsing of the APS, this means there would be a requirement to have a pointer to the DPS (directly or indirectly through another parameter set). This would lead to a dependency graph, when multiple parameter sets are activated in the slice header.

Another concept that was introduced in H.264/AVC was out-of-bitstream delivery of parameter sets. If parameter sets are pre-stored or shared by other means, it may not be possible to specify an exact timing relationship between parameter sets and their usage. So this would create additional constraints on the delivery of these parameter sets, which need to be taken into consideration by the application specification.

Also in case of redundant coding for error prone environments, additional storage of parameter sets in binary form may be required, if the parameter set is lost, which the following parameter sets depend on. Those packages shouldn’t be discarded because another loss could happen to one of the depending parameter sets that is sent later.

An example would be the following:

If the bitstream contains

SPS0, PPS0, PPS1, PPS2, SPS0, PPS0, PPS1, PPS2

duplicating the parameter sets for error safety

<miss>, PPS0, PPS1, PPS2, SPS0, <miss>, PPS1, PPS2

If SPS0 is lost in the fist transmission, all following PPSs need to be stored in binary format, unless the second copy of SPS0 arrives and they can be parsed.

Another argument for independent parsing is that there are only very few cases in which it is hard to avoid the dependency. So far the group always found a way to avoid the dependency.

Taking into account that the main argument for allowing parsing dependencies among parameter sets is bit savings, it is considered that they are not so big that there is enough justification to abandoning the design goal of independent parameter sets and with it the described benefits of it.

# Parsing dependencies in VVC Draft 5

## Uniform tile partitioning and bricks

Problem:

The uniform tile syntax adopted in JVET-N0498 specifies only the width and height of each tile. The number of tiles can only be calculated, when the number of CTUs in the picture is known. The picture size (and thus number of CTUs) is coded in the SPS, while the number of tiles is required to parse the brick definition.

Possible solutions:

1) Revert to HEVC style uniform tile syntax

Note: Uniform brick spacing uses the same style of syntax. This is not a problem, because the number of bricks is not required before parsing the slice header. But it may be desirable to use the same syntax style in both locations.

2) Signal the number of tiles in the PPS

3) Move coding of tile locations in the SPS (see REF).

## Location of virtual filter boundaries

Problem:

| pps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| --- | --- |
| if( pps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| pps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_ver_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_x[ i ] | u(v) |
| pps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < pps_num_hor_virtual_boundaries; i++ ) |  |
| pps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |

The syntax elements pps_virtual_boundaries_pos_x and pps_virtual_boundaries_pos_y are code u(v), the length of the syntax element is given by:

“The number of bits used to represent pps_virtual_boundaries_pos_x[ i ] is Ceil( Log2( pic_width_in_luma_samples ) − 3 ).”

The syntax element pic_width_in_luma_samples is coded in the SPS.

Possible solution:

Code the syntax elements as ue(v)

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).