| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0491 |
| --- | --- |

| Title: | AHG17: High-level syntax cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Recent adoptions related to ALF introduced new syntax element types into the VVC specification: tb(v) and tu(v), for truncated binary and truncated unary codes. This contribution proposes to replace these codes with u(v) or ue(v) without significant change in coding efficiency.

# Introduction

Recent adoptions related to ALF introduced the syntax element types into the VVC specification: tb(v), tu(v) and uek(v), for truncated binary, truncated unary and higher order Exp-Golob codes. tu(v) is curently only used in one location in the slice header, tb(v) in multiple locations in the slice header and the APS.

It is observed that these additional codes do not provide significant coding efficiency benefit and mostly add complexity to the specification and decoder without providing significant benefit.

# Proposal

We propose to change the slice header as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| […] |  |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| if ( slice_type == I ) |  |
| slice_alf_aps_id_present_flag | tb(v)u(1) |
| else |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < ( slice_type = = I ? slice_ alf_aps_id_present_flag : slice_num_alf_aps_ids_luma ); i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(5) |
| slice_alf_chroma_idc | tu(v)u(2) |
| if( slice_alf_chroma_idc && ( slice_type != I ¦ ¦ slice_num_alf_aps_ids_luma != 1) ) |  |
| slice_alf_aps_id_chroma | u(5) |
| } |  |
| } |  |
| […] |  |
| } |  |

slice_alf_aps_id_present_flag specifies the number of ALF APSs that the slice refers to for I slices. If the syntax element is not present in the bitstream, it is inferred to 0.

slice_num_alf_aps_ids_luma specifies the number of ALF APSs that the slice refers to. The value of slice_num_alf_aps_ids_luma shall be in the range of 0 to 76, inclusive. If the syntax element is not present in the bitstream, it is inferred to the value of slice_alf_aps_id_present_flag.[Ed. (YK): The value range of this tb(v)-coded syntax element was not discussed/agreed at the 15th JVET meeting. The value 6 is just a placeholder put here during the editing of VVC WD5. It should be discussed such that a value that makes the most sense should be put here.]

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to 1 for intra slices and slices in an IRAP picture, and set equal to 6 otherwise.

We propose to change the ALF APS as follows:

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | tb(v)ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v)u(v) |
| } |  |
| alf_luma_use_fixed_filter_flag | u(1) |
| if( alf_luma_use_fixed_filter_flag ) { |  |
| alf_luma_fixed_filter_set_idx | tb(v)ue(v) |
| alf_luma_fixed_filter_pred_present_flag | u(1) |
| if( alf_luma_fixed_filter_pred_present_flag ) { |  |
| for( i = 0; i < NumAlfFilters; i++ ) |  |
| alf_luma_fixed_filter_pred_flag[ i ] | u(1) |
| } |  |
| } |  |
| […] |  |
| } |  |
| […] |  |
| } |  |

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive.

alf_luma_coeff_delta_idx[ filtIdx ] specifies the indices of the signalled adaptive loop filter luma coefficient deltas for the filter class indicated by filtIdx ranging from 0 to NumAlfFilters − 1. When alf_luma_coeff_delta_idx[ filtIdx ] is not present, it is inferred to be equal to 0.

The maximum value maxVal of the truncated binary binarization tb(v) is set equal to alf_luma_num_filters_signalled_minus1. The length of alf_luma_coeff_delta_idx[ filtIdx ] is Ceil( Log2( alf_luma_num_filters_signalled_minus1 + 1 ) ) bits.

alf_luma_fixed_filter_set_idx specifies a fixed filter set index. The value of alf_luma_fixed_filter_set_idx shall be in a range of 0 to 15, inclusive. The maximum value maxVal of the truncated binary binarization tb(v) is set equal to 15.

Removed specification text:

## 9.3 Parsing process for truncated unary codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to tu(v).

Inputs to this process are bits from the RBSP and the maximum value maxVal.

Outputs of this process are syntax element values.

Syntax elements coded as tu(v) are truncated unary coded. The range of possible values for the syntax element is determined first. The range of this syntax element is 0 to maxVal inclusive, with maxVal being greater than or equal to 1. codeNum which is equal to the value of the syntax element is given by a process specified as follows:

codeNum = 0
keepGoing = 1
for(i = 0; i < maxVal && keepGoing; i++){
	keepGoing = read_bits( 1 ) 	(9-3)
	if( keepGoing )
		codeNum ++
}

## 9.4 Parsing process for truncated binary codes

This process is invoked when the descriptor of a syntax element in the syntax tables in subclause 7.3 is equal to tb(v).

Inputs to this process are bits from the RBSP and the maximum value maxVal.

Outputs of this process are syntax element values.

Syntax elements coded as tb(v) are truncated binary coded. The range of possible values for the syntax element is determined first. The range of this syntax element is 0 to maxVal, inclusive, with maxVal being greater than or equal to 1. synVal which is equal to the value of the syntax element is given by a process specified as follows:

thVal = 1
th = −1
while( thVal <= maxVal ) {
	th++
	thVal <<= 1
}
val = 1 << th 	(9-4)
b = maxVal − val
synVal = read_bits( th )
if( synVal >= val − b ) {
	synVal <<= 1
	synVal += read_bits( 1 )
	synVal −= val − b
}

where the value returned from read_bits( th ) is interpreted as a binary representation of an unsigned integer with most significant bit written first.

Notes:

- Parsing of slice_num_alf_aps_ids_luma dependent on I_SLICE was defined in the semantics of the syntax element by specifying different parameters for tb(v). It may be desirable to avoid such a dependency. The performance tests for this proposal were run with the proposed syntax.

- The parameter of tb(v) for alf_luma_num_filters_signalled_minus1 was missing in the specification text.

# Test results

The following tables show the coding results for the proposed syntax changes without adaptation of the rate-estimation for the transmitted parameters:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.01% | 0.01% | 0.01% | 101% | 104% |
| Class E | 0.01% | 0.01% | 0.01% | 101% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 107% |
| Class F | 0.01% | 0.01% | 0.01% | 101% | 103% |

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 102% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.01% | 0.01% | 0.01% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | -0.01% | -0.01% | -0.01% | 100% | 98% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 102% |

The following tables shows the coding results for the proposed syntax changes with adaptation of the rate-estimation for the transmitted parameters:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class E | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | -0.01% | 100% | 102% |
| Class F | 0.01% | 0.01% | 0.01% | 100% | 101% |

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.02% | 99% | 99% |
| Class B | 0.00% | 0.00% | -0.01% | 99% | 102% |
| Class C | 0.00% | 0.02% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | 0.00% | 100% | 100% |
| Class D | -0.01% | 0.02% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.02% | 0.02% | 99% | 99% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.39% | 0.11% | 100% | 100% |
| Class C | 0.05% | -0.09% | 0.00% | 100% | 102% |
| Class E | 0.12% | -0.07% | -0.06% | 99% | 103% |
| Overall | 0.05% | 0.12% | 0.03% | 100% | 101% |
| Class D | -0.06% | -0.52% | 0.67% | 100% | 101% |
| Class F | -0.06% | -0.20% | -0.24% | 99% | 102% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).