| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0492 |
| --- | --- |

| Title: | AHG17: On merging of bitstreams and conformance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add bitstream signalling for merging of several independently coded regions (layer, tiles or bricks) into a joint single-layer bitstream. It is argued in the contribution that such system design is a relevant implementation choice for addressing devices with a single decoder instance only and for the inherent synchronization of decoded output in such a design.

The first aspect of the contribution adds an indication towards applicable merging operations and identification of mergeable bitstreams. An SEI message is proposed that carries a mergeability_id to represent a set of constraints defined by external means such as an application specification or a service operator. The mergeability_id is associated with a mergeability_type describing which NAL unit types and syntax structures are required to be rewritten in the respective merging operation.

The second aspect of the contribution adds an indication towards HRD conformance of merged bitstreams to the PTL syntax. A target_level_idc is given in combination with a target_level_percentage that a merged bitstream would occupy in a merged bitstream with a level equal to target_level_idc.

# Introduction

For use cases such as tiled streaming of 360-degree video, independently coded regions are an important tool. The VVC draft currently offers a layering concept as well as picture sub-divisioning schemes trough tiles and bricks that would allow independently coded regions via:

- HEVC-style encoder-side constrained tiles/bricks

- boundary padded tiles/bricks (pending final adoption)

- independently coded layers within a bitstream

In either case, joint decoding of several independently coded regions such as enabled through OMAF’s viewport-dependent profile for HEVC MCTS is a relevant implementation choice for

- addressing devices with a single decoder instance only

- inherent synchronization of decoded output

Applications involving merging of independently coded regions are dependent on bitstream constraint indication that are usually not used traditional applications that target the feasibility of applying various merging methods and HRD conformance of merged bitstreams.

# Feasibility of merging methods

## Description

Merging of independently coded regions into a single-layer bitstream can be achieved through various methods of varying complexity depending on the coding chosen coding options as listed in the following table.

Table 1: Comparison of merging methods

| Method | Complexity | Proposed mergeability_type |
| --- | --- | --- |
| (Full or Partial) Transcoding Adjust Parameter Sets, Slice Headers and Slice Payloads: - Decode - Pixel-level processing - Re-encode | High Entropy coding and pixel processing | – (not relevant) |
| HEVC-style rewriting Adjust Parameter Sets and Slice Headers: - VPS/SPS: PTL, pic size, block size, format - PPS: tiling - APS: filter, reshaper - SH: slice address, slice QP, trailing bits | Medium Rewriting every NAL unit in a session | 3 |
| Parameter set rewriting: - VPS/SPS: PTL, pic size, block size, format - PPS: tiling - APS: filter, reshaper | Low Rewriting a few NAL units | 2 |
| Parameter set rewriting: - VPS/SPS: PTL, pic size, block size, format - PPS: tiling |  | 1 |
| Parameter set rewriting: - VPS/SPS: PTL, pic size, block size, format, (tiling)1 |  | 0 (subject to footnote 1) |

1For this lowest level of merging complexity, it is required to enable tiling structure signaling on sequence level.

While a full transcode is always a viable option, it is often desirable to use a merging method with lower processing complexity. Any device carrying out or preparing the merging operation and receiving bitstreams to be merged (e.g. a MANE, an FF authoring tool or an end client) cannot easily recognize which of the above merging methods is suitable, i.e. the required complexity level. With the current VVC design, this decision requires in-depth analysis of different parameter sets and slice headers to verify content characteristics and coding decisions, e.g. chroma format, block sizes, quantization parameters and so on.

The proposal is to add, for a given merging method identified through mergeability_type, a mergeability identifier mergeability_id (SEI or VUI) that allows identification of mergeable bitstreams by matching value of such mergeability identifier for a given value of mergeability type.

The exact constraints represented by a value of the mergeability_id associated with a value of mergability_type are defined by external means. For instance, an application specification or a service operator could define a set of constraints regarding the relevant parameters given in Table 1. The identifier space is valid for all layers of a given bitstream or for a set of bitstream defined by external means.

## Proposal

Mergeability SEI message

| mergeability( payloadSize ) { | Descriptor |
| --- | --- |
| num_mergeability_ids_minus1 | u(8) |
| for( i = 0; i <= num_mergeability_ids_minus1[ i ]; i++) { |  |
| mergeability_id[ i ] | u(6) |
| mergeability_type[ i ] | u(2) |
| } |  |
| } |  |

num_mergeability_ids_minus1 plus 1 specifies the number of merge idenfiers in the SEI message. The value of num_mergeability_ids_minus1 shall be in the range of 0 to 255, inclusive.

mergeability_id[ i ] contains the i-th identifying number that may be used to identify bitstreams with matching value of mergeability_id[ i ]which can be merged with the current bitstream into an conformant joint bitstream according to the method associated with mergeability_type[ i ]. The value of mergeability_id[ i ] shall be in the range of 0 to 63, inclusive. Values of mergeability_id[ i ] from 0 to 31, inclusive, may be used as determined by the application. Values of mergeability_id[ i ] from 32 to 63, inclusive, are reserved for future use by ITU-T | ISO/IEC and bitstreams shall not contain such values. Decoders encountering a value of mcts_id[ i ] in the range of 32 to 63, inclusive, shall ignore it.

mergeability_type[ i ] equal to 0 specifies that merging of the bitstream with other bitstreams with matching values of the associated mergeability_id[ i ] into a single joint bitstream does not require changes of any active PPS, active APS, slice headers or slice payloads in the bitstream. mergeability_type[ i ] equal to 1 specifies that merging of the bitstream with other bitstreams with matching values of the associated mergeability_id[ i ] into a single joint bitstream does not require changes of any active APS or NAL unit slice headers or payloads in the bitstream. mergeability_type[ i ] equal to 2 specifies that merging of the bitstream with other bitstreams with matching values of the associated mergeability_id[ i ] into a single joint bitstream does not require changes of any slice header or slice payloads in the bitstream. mergeability_type[ i ] equal to 3 specifies specifies that merging of the bitstream with other bitstreams with matching values of the associated mergeability_id[ i ] into a single joint bitstream does not require changes of any slice payloads in the bitstream. The value of mergeability_type[ i ] shall be in the range of 0 to 3, inclusive.

# HRD conformance

## HRD conformance of merged bitstreams

As listed in detail in Samsung contribution M48084, it is desirable to be able to check the conformance of merged bitstreams beforehand through

- […] luma sample rate of the generated elementary stream

- […] bit rate of the generated elementary stream

- […] fullness of hypothetical reference decoder model

and enable checking these measures against target PTL operation points.

However, signaling of PTL related aspects, specifically the level, is relatively coarse in context of applications such as tile-based streaming (24-96 sub-bitstreams). More fine-grained information on some of the aspects (e.g. luma samples per picture, luma sample rate, block sizes, tiling structure) is available through various parameter set syntax elements that need to be parsed. However, for other aspects, such as the HRD model buffer fullness or fine-grained bitrate information, some decoding of the bitstream (e.g. RPL/DPB status) is required.

It is argued that an additional, more fine-grained information in sub-bitstreams, e.g. in HRD/VUI, would allow a better estimation of the HRD conformance of a merged bitstream without the need for partial decoding. This contribution proposes to add an indication in the form of an additional level (e.g. the target level envisioned by the content creator) and the percentage of the additional level that the sub-bitstream would fulfill in a merged bitstream fully using the limits of the indicated additional level.

## Proposal

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| […] |  |
| level_percentage_present_flag | u(1) |
| if( level_percentage_present_flag ) { |  |
| target_level_idc | u(8) |
| target_level_percentage | u(7) |
| } |  |
| […] |  |
| } |  |

level_percentage_present_flag equal to 1 specifies that the syntax elements target_level_idc and target_level_percentage are present. level_percentage_present_flag equal to 0 specifies that the syntax elements target_level_idc and target_level_percentage are not present.

target_level_idc indicates a level to which the CVS conforms as specified in Annex A. Bitstreams shall not contain values of target_level_idc other than those specified in Annex A. Other values of target_level_idc are reserved for future use by ITU-T | ISO/IEC.

target_level_percentage specifies the percentage of the level limits associated with target_level_idc that the CVS conforms to as specified in Annex A.4.1. The value of target_level_percentage shall be in the range of 0 to 127, inclusive.

Relevance for HRD level conformance

The following is based on text of HEVC level definitions from Annex A.4.1 with changes highlighted in yellow:

When level_percentage_present_flag is equal to 1, an additional conformance point test is enabled by setting level equal to target_level_idc and LevelPercentage equal to target_level_percentage ÷ 128. Otherwise, for all other conformance tests, LevelPercentage is set equal to 1.

The bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- PicSizeInSamplesY shall be less than or equal to Floor( MaxLumaPs * LevelPercentage ), where MaxLumaPs is specified in Table A.6.

- The value of pic_width_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * LevelPercentage * 8 ).

- The value of pic_height_in_luma_samples shall be less than or equal to Sqrt( MaxLumaPs * LevelPercentage * 8 ).

- For level 5 and higher levels, the value of CtbSizeY shall be equal to 32 or 64.

- The value of NumPicTotalCurr shall be less than or equal to 8.

- The value of num_tile_columns_minus1 shall be less than Floor( MaxTileCols * LevelPercentage ) and num_tile_rows_minus1 shall be less than Floor( MaxTileRows * LevelPercentage ), where MaxTileCols and MaxTileRows are specified in Table A.6.

- The value of NumBricksInPic shall be less than or equal to MaxBricksPerPicture, where MaxBricksPerPicture is specified in Table A.6.

[Ed. (RS): It is expected that MaxBricksPerPicture will be limited by level definitions as well in the future.]

- For the VCL HRD parameters, CpbSize[ i ] shall be less than or equal to Floor( CpbVclFactor * MaxCPB* LevelPercentage ) for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause E.3.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[ i ] shall be less than or equal to Floor( CpbNalFactor * MaxCPB* LevelPercentage ) for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in clause E.3.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbNalFactor bits.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).